+------------------------------------------------------------------------------+

|                                                                              |

|Information for IBM Token-Ring CardBus Adapter for OS/2 NDIS 2.0 Point Enabler|

|                                                                              |

+------------------------------------------------------------------------------+

------------------------ Information About This Release ------------------------

Driver Name   Version      Device Driver Type

------------  -------   ------------------------------------------

CB_PEOS2.SYS   1.00     NDIS 2.0 OS/2 CardBus Adapter Point Enabler

This device driver support the IBM 16/4 Token-Ring CardBus Adapter.

------------------------------ General Information -----------------------------

The OS/2 Point Enabler (CB_PEOS2.SYS) operation is only supported on

the IBM ThinkPad Model 600 series of laptop computers. Additionally,

the IBMTRP.OS2 NDIS2 MAC driver version used in conjunction with the

Point Enabler must be Version 2.04 or later.

The OS/2 Point Enabler is invoked from within the CONFIG.SYS file and will

require the addition of a DEVICE= statement for each time it is

invoked to configure an adapter. That is to say, a single invocation

of the Point Enabler is only capable of configuring a single 16/4

Token-Ring CardBus Adapter. Examples which follow should help clarify

this matter. Furthermore, the DEVICE= statements invoking the Point

Enabler must precede the DEVICE= statement which loads the IBMTRP.OS2

device driver.

The OS/2 Point Enabler permits parameters to be specified with its

DEVICE= statement. These parameters can specify the expansion socket

containing the adapter to be configured (SLOT=1 means the top expansion socket

and SLOT=2 means the bottom), the Physical Memory Address block mapping (MEM=),

and the PORT I/O Address assignment (IO=). There are defaults associated with

each of the aforementioned parameters and they are:

SLOT=1

MEM=D0000

IO=5000

It may be necessary to specify parameters to resolve system address assign-

ment conflicts.

CONFIG.SYS Examples:

Example 1 -

Specifing that a single 16/4 Token-Ring CardBus Adapter is to be

configured using the default parameters and the adapter physically

installed in either system expansion socket:

DEVICE=CB_PEOS2.SYS

Example 2 -

Specifing that two 16/4 Token-Ring CardBus Adapters are to be

configured using the default parameters for the adapter in the top

system expansion socket and explicitly specifing the parameters to

be used to configure the adapter installed in the bottom system

expansion socket:

DEVICE=CB_PEOS2.SYS

DEVICE=CB_PEOS2.SYS /SLOT=2 /MEM=C8000 /IO=F200

Example 3 -

Specifing that two 16/4 Token-Ring CardBus Adapters are to be

configured assigning the parameters to be used for the configuration

of both adapters:

DEVICE=CB_PEOS2.SYS /SLOT=1 /MEM=D4000 /IO=F200

DEVICE=CB_PEOS2.SYS /SLOT=2 /MEM=C8000 /IO=F100

(**) Trademarks

---------------

IBM is a trademark of International Business Machines, Inc.