FINDIRQ.COM (VERSION 1.0)                    Copyright (c) Rick Knoblaugh

-------------------------------------------------------------------------

First Published in PC Magazine September 25, 1993 (Utilities)

-------------------------------------------------------------------------

FINDIRQ BY RICK KNOBLAUGH

FINDIRQ reports which interrupt request lines (IRQs) in a system

are assigned to specific devices and which are free to use for installing a

new peripheral device.

When you purchase a scanner or other peripheral device, the installation

instructions usually tell you to set the jumpers on the board for an

``available IRQ.''  Even if you know what an IRQ is, however, chances are you

don't know which ones are in use and which are still free.  If you install

your new device to use an already-assigned IRQ, it may well cause a system

crash.

What you need is FINDIRQ, a utility that tells you which devices in your

system are using which IRQs and which IRQs are unassigned.

An IRQ is simply a hardware interrupt request line that allows a

peripheral to signal the computer's central processing unit that an event

requiring the CPU's attention has occurred.  AT-style machines have 16 IRQ

lines; older PCs have but 8.  A few IRQ assignments are more or less

standard on AT-type machines:  The timer uses IRQ0 and the keyboard uses

IRQ1, for example.  But if you've added optional peripherals, such as a

scanner, mouse, or fax modem, you need a program like FINDIRQ to learn

which IRQs the peripherals have appropriated.

Users of Windows 3.1 and/or DOS 6.0 may wonder whether the new

Microsoft Diagnostics (MSD) program makes FINDIRQ unnecessary.  MSD does

a good job of showing IRQ assignments for standard devices, such as COM

and LPT ports.  However, MSD will not report the IRQ assigned to any

device that is not on its internal list.  MSD will  tell you the name of

the program controlling a given IRQ only if you have not disabled STACKS

in your CONFIG.SYS file (more on this later).

Further, since it lacks FINDIRQ's TSR option, MSD overlooks IRQs

that are enabled only while a device--such as a sound board--is active.

FINDIRQ's TSR option also allows it to detect IRQ usage for devices that

are only activated under Windows.  If you have a Microsoft InPort mouse,

for example, and you use it only under Windows (and no DOS driver for it

is installed), most DOS-based diagnostic programs will never detect that

the device is present.  And while there are a number of commercial

IRQ-detecting programs, each with its strengths and limitations, FINDIRQ

can hold its own against any of them.

To work from the source files, you will need the Microsoft MASM

assembler (or compatible), and the LINK and EXE2BIN utilities.

USING FINDIRQ

If you keep FIND IRQ.COM in a subdirectory on your path, simply

entering  FINDIRQ  from a DOS prompt will bring up an opening display.

The IRQ usage of all standard devices (those of which the BIOS has

knowledge: COMn and LPTn) will be shown, along with that of any optional

devices (like sound boards or SCSI adapters) that are currently enabled.

IRQ assignments for devices controlled by TSR programs will also be

displayed if the TSR is loaded in conventional memory.

To report IRQ information for devices controlled by TSRs that are

loaded in high memory, FINDIRQ must be invoked with LOADHIGH, thus

LH FINDIRQ. Note that when FINDIRQ is loaded high, it can still detect

TSRs loaded into low memory, so if you use high memory on your system,

it's always a good idea to load FINDIRQ high.

In listing the IRQ usage, FINDIRQ assumes certain standard device

IRQ assignments.  For example, if the BIOS reports that two COM ports

are present, it assumes that COM1 uses IRQ4 and COM2 uses IRQ3; these

are the standard AT assignments.  If your system supports steerable

IRQs and you have routed them to nonstandard assignments (a rare

situation), FINDIRQ will not properly report the IRQs used by those

devices.

For standard devices, FINDIRQ displays the device name.  For

optional character devices, FINDIRQ displays the name of the device as

specified by the device driver.  For optional block devices,

``Block Device'' is displayed. If a device is being handled by a TSR,

FINDIRQ displays the name of the TSR program.

FINDIRQ SYNTAX OPTIONS

To document even unusual and elusive interrupts, FINDIRQ provides

a variety of optional parameters.  In summary form, the syntax for the

command is

[LH] FINDIRQ [/t] [/i] [/u] [/d] [/h]

As indicated initially, some interrupts are enabled only when the

devices that use them are active.  To detect such devices--sound boards,

for instance--FINDIRQ must be memory-resident at the time the devices

are activated.  FINDIRQ can be installed as a TSR, therefore, by entering

FINDIRQ /t .

When the TSR component of FINDIRQ is loaded, your system is

constantly being monitored in the background for IRQ activity.  When a

previously unassigned IRQ is enabled, its IRQ information is stored in a

tiny, 2-byte data file, FINDIRQ.DAT, which FINDIRQ creates in its

subdirectory.  FINDIRQ uses the information in this file to supplement

what it knows about IRQ usage.  This scheme can't tell you whether a

temporarily activated IRQ is currently in use, but it can tell you that

the IRQ has been used at some point and is probably not available for

other devices.

If you change your system configuration--by removing or rejumpering

a board, for example--you must reinitialize the FINDIRQ.DAT file.  This

is done by executing the /i option  FINDIRQ /i. Regardless of whether

the TSR component is resident, you display IRQ usage in the same way--by

typing FINDIRQ with no switches.  If the resident portion is currently

loaded, a notation will appear on the left side of the display.  Although

the resident portion of FINDIRQ is very small (less than 3K), you can

unload it if you wish by typing FINDIRQ /u .

In certain cases, programs can erroneously enable IRQs; that is,

the IRQs can appear to be in use when in fact they are not.  To guard

against false reporting, FINDIRQ normally verifies that enabled IRQs

have valid handlers before reporting that they are unavailable.  This is

done by checking that the handler is not simply a ROM default interrupt

handler, whose job is to field spuriously generated interrupts.

When an IRQ is enabled under Windows and no DOS device driver for

the device is installed, however, the handler cannot be verified.  There

are also other instances in which interrupt handlers cannot be verified,

such as when IRQ enabling is intermittent or when a memory manager has

remapped ROM. In these circumstances, FINDIRQ will incorrectly report that

the IRQ is available.  To suppress FINDIRQ's verification requirement and

so avoid this problem, you can execute FINDIRQ with its ``don't verify''

switch: FINDIRQ /d . Finally, to remind yourself of the range of FINDIRQ

options, simply execute the program with its help switch: FINDIRQ /h.

The help screen lists and describes all program options.

IRQ REVIEW

Before delving into FIND IRQ's inner workings, a brief review of

interrupt processing may be helpful.  Interrupts provide a time-saving

alternative to periodically polling all the peripherals attached to the

system to determine whether they require the CPU's attention.  Under an

interrupt system, peripheral devices receive recognition only when they

ask for it.  Thus, the system processor does not waste processing time

repeatedly checking for characters being typed at the keyboard or data

bytes being received by an attached modem.

Devices such as the keyboard controller and the communications UART

chip utilize the interrupt lines labeled IRQ0 through IRQ15.  These lines

go to the 8259 Programmable Interrupt Controllers (PICs), which arbitrate

the interrupt requests and then activate the processor's single interrupt

request line.  A detailed discussion of this process can be found in

The IBM PC from the Inside Out, by Sargent and Shoemaker (Addison-Wesley,

1984, 1986).

Figure 2 (See below) depicts this arrangement and shows the standard

IRQ assignments used by an AT-class machine.  Each 8259 PIC handles eight

interrupt lines; to accommodate additional interrupt lines, multiple 8259s

are tied together.  One serves as the master and has a cascade line that

connects it to slave 8259s.  On the AT, IRQ2 is used for this purpose.

Note, however, that you can still attach devices that utilize IRQ2.

The adapter-card edge connector used for IRQ2 in PC-class machines is

wired to IRQ9 in AT machines. It is then routed via a software interrupt

to the interrupt for IRQ2. This round-about arrangement was implemented

to provide compatibility with original PC adapter boards.

When each 8259 is initialized, the interrupt base value is written

to the PIC.  This value determines which interrupt number is generated.

The master PIC starts at 8, so IRQs 0 through 7 generate interrupts 8

through 15.  The slave PIC starts at 70h, so IRQs 8 thru 15 generate

interrupts 70h through 77h.  The PIC registers are accessible in I/O

space; the master PIC is based at 20h and the slave starts at A0h.

FINDIRQ checks the PIC's Interrupt Mask Register to detect active

interrupts.  In this 8-bit register, each bit position corresponds to

one of the interrupt lines.  The BIOS and other device handlers use

this register to enable or disable the generation of interrupts.

Interrupts are disabled or ``masked'' when a 1 is written to the

corresponding bit position, while a 0 means that they are enabled.

HOW FINDIRQ WORKS

FINDIRQ starts by checking the system model ID byte at 0F000:FFFE

to ascertain whether the system is an AT-class machine (16 IRQ lines)

or an older, PC-type machine (8 IRQ lines).  The program next attempts

to read FINDIRQ.DAT, which exists if FINDIRQ has previously been executed

as a TSR.  The file FINDIRQ.DAT contains 2 bytes (16 bits) that show the

IRQs that have at some point been enabled.

After checking the data file, the PIC mask registers are read to

determine which IRQs are currently enabled.  FINDIRQ maintains its own

copies of the PIC mask registers and sets the bits appropriately as it

learns about the devices present on the system.

IRQs assigned to COM and LPT devices may not be enabled if the

devices are not being used.  The INT 11h equipment check call is used

to determine the number of COM and LPT ports available, and the IRQs

corresponding to these are recorded in the FINDIRQ PIC mask variables.

If a mouse is present and its driver is loaded, determining its IRQ is

easy:  Software interrupt 33h, function 24h returns information about

the type of mouse and its IRQ assignment. For PS/2 mice, the IRQ is not

returned, but the IRQ assignment is always IRQ12.

If an IRQ is enabled and has a standard system assignment, FINDIRQ

can easily display the description for the device.  For example, if IRQ6

is enabled, the utility knows that the device description is ``Floppy.''

For optional devices, FIND IRQ displays the name of the device as it is

specified by the device driver (for character devices) or the name of

the controlling program (in the case of a TSR). In order to do this,

FINDIRQ must locate these items in memory.

FINDING THE DRIVER OR TSR

The first step in finding a device driver or TSR program is to look

at the interrupt vector corresponding to the IRQ. For device drivers,

if stacks have been disabled (that is, if STACKS  0,0 has been entered

in the CONFIG.SYS file), the vector will provide the address directly.

Since most systems have stacks enabled (the default is STACKS 9,128 on

AT-class machines), however, the search must usually continue.

When stacks are enabled, DOS intercepts hardware interrupts, and

when interrupts occur, DOS sets up a stack from its pool of stacks

before dispatching them to the original interrupt handler. This ensures

that stack space will not be exhausted during  hardware interrupt

servicing.  Figure 3 (below) shows the techniques  used to identify the

code segment of the DOS STACKS interrupt  handlers.  Also shown in

Figure 3 is the way FINDIRQ inspects the  code in those handlers as it

looks for the original interrupt  service routine (ISR) address.

For later versions of DOS, FINDIRQ takes advantage of the  memory

control block's subsegment control blocks (these only exist in DOS 4.0

and later).  FINDIRQ uses the undocumented ``list of lists,'' DOS

function 52h, to  establish a pointer to the memory control block (MCB)

chain. Using  the subsegment control blocks within the MCBs, the STACKS

entry can be located, yielding the segment of the STACKS area. (For more

information, see the June 12, 1990, Utilities column.)

FINDING THE NAME

Once the ISR address is determined, FINDIRQ searches the DOS device

driver chain for an entry whose code segment matches that of the

interrupt handler.  Again, the ``list of lists'' DOS function is

utilized, this time to retrieve the pointer to the DOS driver chain.

Andrew Schulman's text, Undocumented DOS (Addison Wesley, 1990), contains

an excellent discussion of the requisite techniques.  If a match is found

in the driver chain and if the driver is a character device driver,

FINDIRQ copies the name of the device contained in the device driver

header into its description area.  For block devices, which are referenced

by drive letters instead of device names, ``Block Device'' is used as

the description.  If the ISR address does not belong to a device driver,

it presumably belongs to a TSR.  Thus the next step is to search the MCB

chain for entries that point to executable code.

The procedures used for searching the driver and MCB chains are

listed in Figure 4 (below).  All programs loaded by DOS begin with a

program segment prefix (PSP), the information that includes a pointer

to the program's copy of the environment.  The job at hand is to locate

the PSP for the TSR.

The 16-byte MCB immediately precedes the block of memory it

describes.  Within each MCB is one field that contains the PSP segment

of the program that owns the memory block and another field that tells

how large the block is. If the block of memory described by the MCB is

also the owner, then the block contains a program; it's not an

environment block or another allocated chunk of memory.  If the ISR

address falls in the block of memory (as defined by its size), then the

TSR is found. Once the TSR is located, the PSP's pointer to the

environment is used to retrieve the program name.  With DOS 3.0 and

later, the program name is stored in the last portion of the environment

block.

One complication that can arise here is that the environment may

have been freed by the TSR.  In such a case, the address space may

actually contain the environment for another program.  To ensure that

it has the right environment, FIND IRQ compares the owner field of the

MCB for the environment with the PSP segment.  If these are different,

the environment has been freed.  Even in such a case, however, the

program name can sometimes still be retrieved; DOS versions 4.0 and

later place the program name in the MCB itself.  If all else fails,

FINDIRQ uses ``Other'' for the description of the program that handles

the interrupt.  IRQ usage that can only be detected with FINDIRQ's TSR

component will usually list ``Other'' as its description.

TSR APPROACH

When FINDIRQ is invoked with the TSR option, it becomes resident in

the system and monitors the enabling of IRQs in the 8259 PIC interrupt

mask registers.  The TSR component of FINDIRQ chains into the user timer

interrupt (INT 1Ch), and the PIC mask registers are read approximately

once a second.  When a new IRQ is enabled, FINDIRQ sets a flag so that

it can update its .DAT file when it is safe to do so.  Interrupt 28h,

the DOS idle handler, provides the means of detecting the period in which

it is safe to use DOS functions from within a TSR (see ``The Inner Life

of a TSR,'' PC Magazine, August 1990).  As mentioned earlier, this data

file is read by FINDIRQ when it is invoked to display IRQ status

information.

SUMMARY

Adding new devices to your system can be a challenge.  The difficulties

are sufficient to have prompted Microsoft to spearhead development of the

Plug and Play Industry Standard Architecture Specification which may

someday make manual configuration obsolete. In fact, IBM's MCA architecture

already supports ``plug and play,'' though it is not in wide use.  But

until automatic configuration becomes a standard, the next time you need

to add a peripheral that uses an IRQ, fire up FINDIRQ and make your IRQ

selection with ease.

-----------------------------------------------------------------------------

RICK KNOBLAUGH IS A FREQUENT CONTRIBUTOR TO PC MAGAZINE.

-----------------------------------------------------------------------------

QUICK REFERENCE GUIDE

FINDIRQ

Rick Knoblaugh                            September 28, 1993 (Utilities)

Purpose:

Reports which interrupt request lines (IRQs) in a system are

assigned to specific devices and which are free to use for installing

a new peripheral device.

Format:

[LH] FINDIRQ [/t] [/i] [/u] [/d] [/h]

Remarks:

Executed from a DOS prompt and without any of its optional

parameters, FINDIRQ identifies installed standard (AT) IRQ devices and

optional character devices by device name.  Optional block devices are

designated by ``Block Device.''  TSR-controlled devices are identified

by their program names.

The command FINDIRQ will detect devices controlled by TSR programs

that are loaded in conventional memory.  Devices controlled by TSRs

loaded in high memory require entering the command as LH FINDIRQ.

Since this latter command form also detects conventionally sited TSRs,

LH FINDIRQ should be used on systems that use high memory.

Peripherals such as sound boards, whose IRQ numbers are enabled only

when the devices are active, can be detected only by activating them

while FINDIRQ is memory-resident.  The command FINDIRQ /t installs the

utility as a TSR; FINDIRQ /u uninstalls it.  While memory-resident,

FINDIRQ records IRQs in the file  FINDIRQ.DAT, which is read before

displaying its on-screen report.  Since any configuration changes render

the previous contents of FINDIRQ.DAT obsolete, the data file can be

reinitialized by entering FINDIRQ /i.

FINDIRQ verifies its interrupt information before presenting it.

In some circumstances, however, such as when an IRQ is enabled under

Windows but no DOS interrupt handler is located, no verification is

possible, so the interrupt will appear to be free when it is not.

To check for such a possibility, the verification requirement can be

turned off with the command FINDIRQ /d.

Finally, a help screen can be displayed by entering FINDIRQ /h.

A Microsoft MASM assembler (or compatible), LINK, and EXE2BIN are required

if the program is to be rebuilt.

---------------------------------------------------------------------------

Standard IRQ Assignments

_________________       _________________

Timer                     IRQ0   |               |       |               |

Keyboard                  IRQ1   |               |       |               |

IRQ2   |   Master      | INTR  |               |

Serial port2             -IRQ3---|    8259       |-------|     CPU       |

Serial port1            | IRQ4   | programmable  |       |               |

Parallel port 2         | IRQ5   |  Interrupt    |       |               |

Floppy disk controller  | IRQ6   |  controller   |       |               |

Parallel                | IRQ7   |_______________|       |_______________|

|

|_________________________________________

|

_________________               |

Real-time clock           IRQ8   |               |               |

IRQ9   |               |               |

IRQ10  |    Slave      |---------------

IRQ11  |    8259       |

PS/2 mouse                IRQ12  | programmable  |

Math coprocessor          IRQ13  |  Interrupt    |

Hard disk controller      IRQ14  |  controller   |

IRQ15  |_______________|

-----------------------------------------------------------------------------

Figure 2: Shown here are the master and slave 8259 interrupt controllers

and the standard IRQ assignments utilized in AT-class platforms.

-----------------------------------------------------------------------------

STACKS Routines

;-------------------------------------------------------------|

;get_stacks_seg - Determine the segment where the DOS STACKS  |

;                 code resides.  This will be used later      |

;                 for tracing back through the STACKS         |

;                 interrupt handlers to find the original     |

;                 ISRs.                                       |

;                                                             |

;             Enter:                                          |

;                      ds = local data segment                |

;                atclassf = TRUE if AT class machine          |

;                 dos_ver = DOS minor ver/major ver           |

;                                                             |

;              Exit:                                          |

;                                                             |

;         dos_handler_seg = segment for DOS STACKS.           |

;                                                             |

;  ds preserved.                                              |

;-------------------------------------------------------------|

get_stacks_seg  proc    near

push    ds

xor     bx, bx

mov     ds, bx                  ;vectors

assume  ds:nothing

;

;Use the segment found in the floppy handler as our default guess of

;where the STACKS code is.

;

;

mov     bx, ((pcflop + BEG_INTS) shl 2) ;vector for IRQ6

mov     ax, [bx].d_segment

cmp     byte ptr cs:dos_ver, 4  ;DOS 4 or better?

jb      get_stacks800

;

;If DOS 4 or better, memory control block, subsegment control blocks

;can be used to find STACKS.

;

mov     ah, DOS_LIST_LISTS

int     21h

mov     dx, es:[bx - 2]         ;segment of first MCB

mov     ds, dx

mov     cx, dx                  ;save it

xor     bx, bx

cmp     [bx].mcb_sig, 'M'       ;valid MCB?

jne     get_stacks800           ;MCBs corrupted

mov     di, [bx].mcb_size       ;get first size

inc     di                      ;add para for the MCB

add     dx, di                  ;after DOS data seg

inc     cx                      ;1st subsegment block

get_stacks300:

cmp     cx, dx                  ;at end of info?

jae     get_stacks800           ;if so, search done

mov     ds, cx                  ;load the segment

cmp     [bx].mcb_sig, 'S'       ;STACKS?

je      get_stacks500

mov     cx, [bx].mcb_owner

add     cx, [bx].mcb_size       ;advance to next block

jmp     short get_stacks300

get_stacks500:

mov     ax, [bx].mcb_owner

get_stacks800:

mov     cs:dos_handler_seg,  ax    ;save it

get_stacks900:

pop     ds

assume  ds:code

ret

get_stacks_seg  endp

;-------------------------------------------------------------|

;ck_for_dos - Given a ptr to an interrupt vector, determine if|

;             the handler is one of those inserted by DOS     |

;             to first check stack before dispatching to      |

;             original ISR.  If it is, attempt to get the     |

;             address of the original ISR.                    |

;                                                             |

;             Enter:                                          |

;                      ds = local data segment                |

;                   es:bx = ptr to vector                     |

;         dos_handler_seg = segment of the DOS handlers       |

;                                                             |

;              Exit:                                          |

;                                                             |

;                Carry set if no DOS handler present or       |

;                          can't find original ISR.           |

;                                                             |

;             else  es:bx = ptr to original ISR               |

;                                                             |

;  All other registers preserved.                             |

;--------------------------------------------------------------|

ck_for_dos      proc    near

push    ax

push    cx

push    di

mov     ax, dos_handler_seg

cmp     es:[bx].d_segment, ax   ;same seg STACKS?

jne     ck_for_900

les     di, es:[bx]             ;es:di = &handler

mov     al, es:[di]             ;get byte of code

;

;Several different flavors of these stack handlers are used in the

;various iterations of DOS. Look for the following cases:

;

;case 1:             jmp  label

;                    dd   &old_isr

;

;            label:  call handler   ;put offset of &ISR on stack

;                    dw  offset var ;and go get it and call it

;

;

;case 2:             call handler   ;put offset of &ISR on stack

;                    dd   &old_isr

;

;

;case 3:             jmp  label

;                    dd   &old_isr

;

;             label:  ...

;

;                    pushf

;                    cs:

;                    call far [xxxx] ;also this it may not follow the

;                                    ;jmp

;case 4:             push ax

;                    ...

;                    ...

;

;                    pushf

;                    cs:

;                    call far [xxxx]   ;xxxx is the offset of the

;                                      ;variable that holds the &ISR

cmp     al, OP_JMP      ;is it a jmp

jne     ck_for_200

inc     di

sub     ah, ah

mov     al, es:[di]     ;get next type of code

inc     di              ;past 2nd byte of rel jmp

add     di, ax          ;get to location of jmp

mov     al, es:[di]     ;get byte of code there

ck_for_200:

cmp     al, OP_CALL     ;is it a call

jne     ck_for_300

mov     di, es:[di + 3] ;offset to dd that holds &ISR

les     bx, es:[di]     ;&ISR

jmp     short ck_for_999 ;exit with carry clear

ck_for_300:

mov     cx, BYTES_TO_SEARCH

ck_for_400:

mov     al, OP_PUSHF

repne   scasb           ;pushf instruction?

jcxz    ck_for_900

mov     al, OP_CS       ;CS override?

scasb

jnz     ck_for_400

mov     ax, OP_CALL_MEM16

scasw                   ;call?

jnz     ck_for_400

mov     di, es:[di]     ;offset of var that holds &ISR

les     bx, es:[di]     ;get &ISR

ck_for_800:

jmp     short ck_for_999 ;exit with carry clear

ck_for_900:

stc

ck_for_999:

pop     di

pop     cx

pop     ax

ret

ck_for_dos      endp

-----------------------------------------------------------------------

Figure 3: These routines identify the segment where the DOS STACKS

interrupt handlers reside and retrieve the original interrupt service

routine address.

-------------------------------------------------------------------------

Search Procedures

;-------------------------------------------------------------|

;find_drivers - For all IRQs that are active, look to see if  |

;               we have a description for the device.  If we  |

;               don't, the device using this IRQ must be      |

;               something we don't know about, such as a      |

;               sound board, SCSI adapter, etc.  In this case,|

;               go get the driver name or program name and    |

;               put it in the description for the IRQ.        |

;                                                             |

;               In the event that a driver or TSR (with       |

;               PSP) is not found, assign "Other" for the     |

;               device description.                           |

;                                                             |

;                                                             |

;          Enter:                                             |

;                  ds=local data segment                      |

;                atclassf = TRUE if AT class machine          |

;                                                             |

;                pic_mask = PIC mask values.                  |

;                                                             |

;          device_desc = offset to array of description       |

;                        offsets.                             |

;                                                             |

;           Exit:                                             |

;                                                             |

;  ds, bp preserved, es trashed.                              |

;--------------------------------------------------------------|

find_drivers    proc    near

push    bp

mov     ah, DOS_LIST_LISTS

int     21h             ;get es:bx ptr to lists

cmp     dos_ver, 0003h  ;DOS 3.0?

jne     find_drv_050

lea     bx, es:[bx].nul_dev_head30 ;ptr to NUL device

jmp     short find_Drv_080

find_drv_050:

lea     bx, es:[bx].nul_dev_head   ;ptr to NUL device

find_drv_080:

mov     ax, es

mov     drv_ptr.d_segment, ax   ;save ptr to start

mov     drv_ptr.d_offset, bx    ;of driver chain

xor     al, al                  ;IRQ counter

mov     bp, pic_mask

mov     cx, 16                  ;16 IRQs on AT

cmp     atclassf, TRUE          ;AT machine?

je      find_drv_100

shr     cx, 1                   ;only 8 IRQs on PC

find_drv_100:

shr     bp, 1                   ;check IRQ

jc      find_drv_600            ;skip if not active

find_drv_110:

call    get_desc_entry          ;description for IRQ

xor     bx, bx

cmp     [di], bx                ;is there one?

je      find_drv_125

;

;If there is already a description for the device, don't need to look

;for driver.  However, there is one exception.  If we are on an AT,

;we always assign an initial description of "cascade" to IRQ2.  If

;that is the only description we have (e.g. we haven't got a mouse on

;IRQ2, etc.), go ahead and look for a driver using IRQ2.

;

cmp     atclassf, TRUE          ;AT machine?

jne     find_drv_600            ;if not, no cascade

cmp     al, 2                   ;doing IRQ2?

jne     find_drv_600            ;if not, no test

cmp     [di + 2], bx            ;check 2nd desc (bx=0)

jne     find_drv_600

add     di, 2                   ;next desc entry

find_drv_125:

mov     es, bx                  ;point to vector

mov     bl, al                  ;irq number

cmp     al, 8

jb      find_drv_150

add     bl, (BEG_SLAVE_INTS  - 8)

jmp     short find_drv_175

find_drv_150:

add     bl, BEG_INTS

find_drv_175:

shl     bx, 1

shl     bx, 1                   ;offset into vectors

;See if pointing to DOS stack handlers.  If so, carry is not set and

;es:bx points to address of original ISR.

;

call    ck_for_dos

jnc     find_drv_200            ;es:bx point to ISR

les     bx, es:[bx]             ;point to ISR

find_drv_200:

call    load_drv_desc

find_drv_600:

inc     al                      ;advance IRQ counter

loop    find_drv_100

find_drv_900:

pop     bp

ret

find_drivers    endp

;-------------------------------------------------------------|

;load_drv_desc - Retrieve and load driver description.        |

;                                                             |

;          Enter:                                             |

;                  ds=local data segment                      |

;                  di=offset of entry for description         |

;               es:bx=ptr to ISR for which description is     |

;                     needed.                                 |

;        device_desc = offset to array of description         |

;                      offsets.                               |

;       drv_desc_ptr = offset of next work description area.  |

;                                                             |

;           Exit:                                             |

;                 [di]=offset of description.                 |

;         drv_desc_ptr advanced to next work description area.|

;                                                             |

;  all registers saved.                                       |

;--------------------------------------------------------------|

load_drv_desc   proc    near

push    ax              ;save picmask/IRQ counter

push    cx              ;save loop count

push    di              ;save offset of desc entry

push    bx                      ;save offset

mov     dx, es                  ;address in dx:bx

call    is_it_driver            ;see if driver or TSR

pop     dx                      ;get back offset (bx)

jnc     load_drv_300            ;jmp if not a driver

mov     dx, offset block_desc   ;"Block Device"

cmp     al, 2                   ;IRQ2? if so, may have

jne     load_drv_290            ;"Cascade" if AT - use

mov     dx, offset block_abbrv  ;abbrev desc to fit

load_drv_290:

test    es:[bx].dev_attrib, 8000h ;char driver?

jz      load_drv_550     ;if not, use "Block Device"

call    mov_char_name    ;if so, move char device name

jmp     short load_drv_450

load_drv_300:

;

;Wasn't a driver.  Must be a TSR.  Using the pointer to environment

;from the TSR's PSP, find the name of executable.

;

;

mov     bx, dx                  ;offset of ISR

call    determine_psp           ;return nc if PSP found

;

;If a PSP is found, carry will NOT be set.  Also, if PSP is found, and

;ax is nonzero, ax holds the segment of the environment.  If ax is

;zero then the environment was freed.  If such cases, es:bx will be

;returned as a pointer to the filename description in the MCB

; (if DOS is 4.x or better - otherwise bx=0)

;

jnc     load_drv_350

load_drv_330:

mov     dx, offset other_desc   ;can't get filename

jmp     short load_drv_550      ;so, go assign "Other"

load_drv_350:

or      ax, ax                  ;environment found?

jnz     load_drv_370            ;if so, get filename

or      bx, bx                  ;name in MCB?

jz      load_drv_330            ;if not, print "Other"

jmp     short load_drv_400      ;go get name

load_drv_370:

mov     es, ax

call    retrieve_path           ;get ptr to filename

jc      load_drv_330            ;if data not right

load_drv_400:

mov     di, drv_desc_ptr

mov     dx, di                  ;save desc offset

call    mov_file_name

load_drv_450:

mov     al, STR_TERM

stosb                           ;terminate it

add     drv_desc_ptr, size drv_descrip

load_drv_550:

pop     di

pop     cx

pop     ax

mov     [di], dx                ;store offset of name

ret

load_drv_desc   endp

;-------------------------------------------------------------|

;determine_psp - An ISR was found and it is not part of a     |

;                device driver.  Search the MCB chain for     |

;                PSP associated with code that would contain  |

;                the ISR.  If found, check the environment    |

;                entry for the PSP.  If environment has not   |

;                been freed, return ax=environment segment. If|

;                not found and DOS is 4.x or better, return   |

;                es:bx as ptr to filename in MCB.             |

;                                                             |

;                                                             |

;          Enter:                                             |

;                  es:bx = ptr to ISR                         |

;                                                             |

;           Exit:                                             |

;                     cy set if PSP NOT found else nc         |

;                     also if found ax=environment seg (zero  |

;                                      if it was freed)       |

;                                                             |

;                          if found and ax=0, es:bx=ptr       |

;                                       to filename in MCB    |

;                                       (zero if < DOS 4)     |

;                                                             |

;   ds preserved.                                             |

;-------------------------------------------------------------|

determine_psp   proc    near

push    ds

mov     dx, es                  ;segment of ISR

mov     cl, 4

shr     bx, cl                  ;convert to paragraphs

add     dx, bx                  ;para location of code

mov     ah, DOS_LIST_LISTS

int     21h

mov     di, es:[bx - 2]         ;segment of first MCB

mov     es, di

xor     bx, bx                  ;offset zero in MCB

mov     si, es                  ;save MCB segment

determine_p200:

inc     si                      ;ready for next one

cmp     es:[bx].mcb_sig, 'Z'    ;at end of chain?

je      determine_p900          ;if so, not found

cmp     es:[bx].mcb_sig, 'M'    ;valid entry?

jne     determine_p900          ;if not, MCBs wrong

determine_p300:

mov     cx, es:[bx].mcb_owner   ;get MCB owner seg

mov     di, cx                  ;save it

mov     ax, es:[bx].mcb_size    ;and its size

cmp     cx, si                  ;is this a PSP?

jne     determine_p700          ;if not, go next PSP

cmp     dx, cx                  ;compare segment

jb      determine_p700          ;beneath the segment

add     cx, ax                  ;get end of the area

cmp     dx, cx

ja      determine_p700          ;above the segment

assume  ds:nothing

mov     ds, di                  ;ds=seg of owner

mov     ax, [bx].psp_environ_seg

or      ax, ax                  ;freed environment?

jz      determine_p400          ;if so, try plan b

dec     ax                      ;1 paragraph back

mov     ds, ax                  ;is seg of MCB for it

inc     ax                      ;back to "environment"

cmp     [bx].mcb_owner, di      ;belongs to PSP?

je      determine_p999          ;yes, exit nc, ax=seg

determine_p400:

xor     ax, ax                  ;indicate none

cmp     byte ptr cs:dos_ver, 4  ;DOS 4 or better?

;

;if yes, we can return filename

;if not, bx=0 indicating must print "Other"

;

cmc                             ;carry clear if < DOS 4

jnb     determine_p999          ;nc and bx=0 if < DOS 4

determine_p500:

clc

lea     bx, es:[bx].mcb_fname

jmp     short determine_p999    ;nc, es:bx = &filename

determine_p700:

add     si, ax                  ;get to next MCB

mov     es, si

jmp     short determine_p200

determine_p900:

stc

determine_p999:

assume  ds:code

pop     ds

ret

determine_psp   endp

---------------------------------------------------------------------------

Figure 4:  The procedures listed above are used to search the driver and

MCB chains.

---------------------------------------------------------------------------#############################################################