This file is a more specialised look at hardware disc access. Requests for a

specialised DOC file covering this material have been fairly numerous now, so

I'm going to add this to the DOC file list. As per usual, this file will con-

tain material culled from the Amiga Hardware Reference Manual plus extra add-

itions of my own where deemed necessary (given that the Amiga manual writers

are all devotees of the terse school of writing, the additions are deemed to

be necessary on a regular basis!).

Contents:

Hardware Disc Access:Introduction

Hardware Disc Access:Hardware Organisation

Hardware Disc Access:CIA Timers For Delays

Hardware Disc Access:MFM And GCR Encoding

Hardware Disc Access:Basic I/O

Hardware Disc Access:Track Selection

Hardware Disc Access:Hardware Bugs

Hardware Disc Access:File Updates

Hardware Disc Access:Introduction

There have been a number of requests (particularly among ACC members)

for a DOC file on the subject of hardware disc access. Well, here it is. This

is partly a reworking of the Hardware Reference Manual (considered a strong

contender in some circles for the Plain English Society's gobbledegook award)

and various snippets of information that have come my way. As usual, I'll try

to make this file as simple to understand as possible given the subject mat-

ter.

Why hardware disc access? Well, games writers form one group of pro-

grammers who immediately come to mind when one thinks of people who require

this sort of information, as they are ALWAYS looking for sneaky tricks with

which they can bypass the OS and do their own thing. Whether it's custom boot

code or finding an alternative low-level disc data organisation to cram more

sound samples or graphics data into their creations, games programmers always

'hit the hardware' first and foremost.

Further down the list come system rpogrammers, who need hardware info

in order to provide those operating system routines that allow the remainder

of the programming community to write applications without having to reinvent

the wheel many times over to perform tasks such as loading and saving files.

Then there's the purely curious. ACC members usually fall into this

category, whether or not they fall into the previous two categories. This DOC

file should please this latter group too.

Programmers who have previously been satisfied with the DOS library

should take note of the following caveats:

1) ANY direct access to the hardware is GUARANTEED to

destroy Exec's multitasking integrity. If you want to

access hardware without compromising multitasking,

use the resources (disk.resource and misc.resource

etc, accessed in a like manner to devices);

2) Direct access to the disc hardware is complicated by

the use of two chip sets (the CIAs and the custom

chips);

3) Direct access to the disc hardware also requires a

knowledge of hardware-level disc formats (more on

this later). If you are not sure what MFM and GCR

formats are, then I suggest seeking out suitable

reference material before using the information in

this DOC file.

These warnings given, it is now time to move on. But before I do, note also

that this information applies to floppy disc controllers ONLY. Since I do not

have a hard disc, and certainly do not have a spare hard disc to experiment

with, I cannot give information on hard disc controllers (and in any case, it

is highly unlikely that anyone reading this DOC file is then going to go away

and write system software unless they're already employed by Commodore-Amiga,

and in that case they'll have far more information to hand than here!).

Hardware Disc Access:Hardware Organisation

Having stated that the disc hardware uses two different chip sets, I shall

now outline the disc hardware organisation.

The CIA chips (component number 8520) are used for drive selection &

disc sensing. The custom chips (specifically, Paula) are used for drive con-

trol (read/write mode), and to link areas of memory reserved by the program-

mer for disc access to the disc system. Note before I continue that as is the

case for ALL memory intended to be used in conjunction with the custom chips,

disc read/write buffers MUST be in CHIP RAM. This normally lies at the add-

resses given below:

$00000-$7FFFF		: A500/A1000/A2000 with

original chip set

$00000-$FFFFF		: A500/A1000/A2000/A1500

with Fatter Agnus chip

and the new A500P

$00000-$1FFFFF		: A3000 with Super Fat

Agnus Chip (for rich

programmers only!)

However, this may NOT be true in the future! Commodore has already made four

changes to the chip set (the latest being the Enhanced Chip Set specifica-

tion as found on latest A3000s) and future changes may see future Amigas with

relocatable CHIP RAM pages, so you have been warned! For a method of reser-

ving CHIP RAM that works on ALL machines, use the Exec AllocMem() call via

exec.library (this HAS to work or else Commodore will face lawsuits!).

Having dealt with CHIP RAM, now to the registers of the disc hardware

system. First, the CIA registers. These lie at addresses $BFD000-$BFEFFF, but

not all of these addresses are used. Below is a table outlining the CIA regi-

ster set used by the disc hardware, including addresses, register names, and

functions. NOTE:many signals accepted by the CIA registers are ACTIVE LOW, in

other words, to turn the function ON requires a ZERO to be written into the

approriate register or register bit (as opposed to the normal signal logic,

called ACTIVE HIGH). In the table below, (L) denotes ACTIVE LOW, and (H) de-

notes ACTIVE HIGH.

Register	Bit	Function

--------	---	--------

CIAAPRA		 5	DSKRDY signal (L).

($BFE001)

Drive will pull this signal low (clear

the bit) when the drive motor is known

to be rotating at full speed (see Note 1

below).

4	DSKTRACK0 signal (L).

Drive will pull this signal low (clear

the bit) when the disc drive read/write

heads are positioned over track zero (see

Note 2 below).

When this signal is active, software MUST

NOT ATTEMPT TO STEP THE HEADS OUTWARDS!

Modern add-on drives ignore the signal in

these circumstances, but older drives may

try to step out, resulting in a 35 repair

bill for a dead drive! YOU HAVE BEEN WARNED!

3	DSKPROT signal (L).

Disc write protected signal:

0 = Disc write protected

1 = Disc writeable

2	DSKCHANGE (L).

Disc changed signal. This bit becomes

zero when a disc is removed from the

drive (signalling a disc change). The

signal only becomes 1 again if a disc

is inserted AND a step pulse is re-

ceived (built-in safety feature, which

makes sure that a real disc is being

accessed, and not thin air).

CIABPRB		 7	DSKMOTOR (L).

($BFD000)

Disc motor control signal. Implemented

in a non-standard fashion. Drives LATCH

the signal (i.e., keep their own private

copy of it) when selected. So, to force

a drive motor to be in a given state,

set or clear this bit, THEN send a signal

along one of the drive select lines. The

drive will "remember" the state of its

motor once the select signal is turned

off.

0 = Motor ON

1 = Motor OFF

CAUTION:after turning the motor ON,

software MUST wait for either 1/2 sec

(500 ms) or DSKRDY to go low. Other-

wise, the motor will NOT be at full

speed, and a write operation will

trash the disc!

6	DSKSEL3 (L).

Select drive 3 (DF3: if available on

the user's system).

0 = Select drive for operation

1 = Drive ignores other signals

5	DSKSEL2 (L).

Select drive 2 (DF2: if available on

the user's system).

0 = Select drive for operation

1 = Drive ignores other signals

4	DSKSEL1 (L).

Select drive 1 (DF1: if available on

the user's system).

0 = Select drive for operation

1 = Drive ignores other signals

3	DSKSEL0 (L).

Select drive 0 (DF0:), THE internal drive

on A500s (some A1000/2000 have two inter-

nal drives), and the one used for booting

WorkBench under KickStart 1.2/1.3.

0 = Select drive for operation

1 = Drive ignores other signals

2	DSKSIDE

Specify which disc read/write head to use.

All Amiga drives are double sided. Hence

there are two read-write heads. Use:

0 = Select UPPER surface disc head

1 = Select LOWER surface disc head

This value MUST be stable for at least

100 microseconds before writing, and

at least 1.3 MILLIseconds must elapse

before switching DSKSIDE.

1	DSKDIREC

Specify direction to seek the heads.

0 = Seek TOWARD CENTRE SPINDLE

1 = Seek TOWARD OUTER EDGE

This signal MUST be set up BEFORE

issuing a disc step signal. DO NOT

use a single write to the register

to perform both tasks! Use (for

example):

MOVE.B #$0A,CIABPRB

... short delay, e.g. NOP

MOVE.B #$09,CIABPRB

to step the heads outward on DF0:.

0	DSKSTEP (L).

Step the heads of the disc a distance

of one track. So, to step 5 tracks,

this signal needs to be sent 5 times.

The signal MUST be sent as:

Write 1 into bit

Write 0 into bit

Very short delay, e.g. a NOP

Write 1 into bit

As in:

MOVE.B	#$08,CIABPRB

MOVE.B	#$09,CIABPRB

NOP

MOVE.B	#$08,CIABPRB

for EACH INDIVIDUAL step operation.

See notes 3,4.

CIABICR		 4	DSKINDEX (L).

($BFDD00)

Disc index pulse. Can be used to gen-

erate a level 6 interrupt.

Notes on Above Table

1) DSKRDY valid ONLY when drive motor is ON. At other times,

configuration information may obscure the meaning of this

input.

2) Track zero is the OUTERMOST track of a disc. The usual

largest track is track 79, located nearest the centre of

the disc. Some drives (BUT NOT ALL!) are capable of

reading and writing up to track 83. DO NOT ASSUME that

software attempting such access will work on all machines!

3) Amiga drives are guaranteed to step to the next track within

3 milliseconds (3 ms). Some drives will perform much faster

than this, but NOT ALL. Using software delay loops (decre-

menting a counter) is NOT ACCEPTABLE (and not portable to

machines using 68020/68030 processors with higher clock

speeds). A better method is to use the CIA timers to pro-

vide the delay. See later.

4) When reversing directions, a MINIMUM delay of 18 ms is

required from the last step pulse. Settle time for Amiga

drives is specified at 15 ms.

That covers the CIA portion of the disc hardware. Now I present a table of

registers, bits and functions, this time for the various custom chip regis-

ters used by the disc hardware system (located at $DFF000 onwards). Note:

many registers exist as pairs, one for reading, one for writing. Write-only

register are denoted by (W), read-only registers by (R).

Register	Bits	Function

--------	----	--------

$DFF020		 3-0	DSKPTH (W)

$DFF022		15-0	DSKPTL (W)

Disc data pointer registers

Contains a pointer to the area of CHIP RAM

used for data transfer. Store the address

in these two registers. A single MOVE.L

can be used for this.

$DFF024			DSKLEN (W)

Disc data length & access register

15	DMAEN

Secondary disc DMA enable. Disc access

using a DMA channel CANNOT take place if

this bit is zero!

14	WRITE

0 = Read access

1 = Write access

13-0	LENGTH

These bits contain the number of WORDS to

transfer.

$DFF01A			DSKBYTR (R)

Disc data byte & status read

15	DSKBYT

Set to 1 by disc system when bits 0-7

of this register contain a valid byte

of data. When this register is read by

the 68000 or DMA, this bit is cleared

for the next data byte status.

14	DMAON

Indicates when DMA is ACTUALLY ON. Is

only set to 1 when ALL DMA enable bits

pertinent to the disc system are set!

13	DISKWRITE

Reflects the state of the WRITE bit in

DSKLEN above.

12	WORDEQUAL

Indicates that the DSKSYNC register

equals the disc input stream. This

bit is only true while the input

stream matches the sync register,

which can be for as little as 2

microseconds! See below for DSKSYNC.

11-8	Currently unused. DO NOT RELY ON READ

VALUE.

7-0	DATA

Value of disc data byte that has been

read.

$DFF07E			DSKSYNC (W)

Disc synchronisation value register

15-0	Disc synchronisation value. Normally

set to the magic MFM sync mark value

of $4489 by AmigaDOS. Hardware pro-

grammers can experiment with other

values (CAUTION!).

$DFF010			ADKCONR (R)

$DFF09E			ADKCON 	(W)

Audio and Disc Control Register

Use in the following manner:

MOVE.W	ADKCONR,D0 for READING

MOVE.W	D0,ADKCON  for WRITING

15	SET/CLR

Best illustrated by an example. If

the word $8001 is written to a regi-

ster with a SET/CLR bit, then bit 0

will be SET, and all other bits left

untouched. If the word $0001 is writ-

ten, then bit 0 will be CLEARED, and

all other bits left untouched. This

allows the programmer to affect ONLY

those bits required for the given

operation. Also, see typed_hardware.doc

for more info.

14	PRECOMP1

13	PRECOMP0

Precompensation value. Values are:

00 : No precompensation

01 : 140 ns precompensation

10 : 280 ns precompensation

11 : 560 ns precompensation

12	MFMPREC

0 = GCR precompensation (allows

Amiga drives to read Apple

Macintosh discs with certain

programming tricks)

1 = MFM (Normal AmigaDOS setting,

also allows Atari ST/IBM PC

discs to be read)

10	WORDSYNC

Set to 1 to allow synchronising and

starting of DMA on disc read of a word.

DSKSYCN contains the synchronisation

word. VERY USEFUL.

9	MSBSYNC

Set to 1 to enable synchronisation on

most significant bit of input (used for

GCR setting)

8	FAST

Value of 1 selects 2 microseconds per

bit cell (MFM). Data must be valid raw

MFM data. Value of 0 selects 4 micro-

seconds per bit cell (GCR).

This covers the register organisation of the disc hardware. Before covering

how to use the registers for disc access, it is time to cover MFM encoding &

related topics.

Hardware Disc Access:CIA Timers For Delays

The CIA chips contain on-board timer hardware, that allows the programmer to

implement precisely timed delay loops that can migrate happily across differ-

ent versions of the Amiga. This facility will be vital in managing the disc

hardware, so I shall now describe the usage of the CIA timers to generate a

fixed-period delay loop.

Now, the CIA timers are often chosen to implement delay loops that

are relatively independent of the remaining Amiga hardware. However, accord-

ing to the Concise Hardware Reference Manual, the timing signals for NTSC and

PAL Amigas differs slightly. The timing signals are:

NTSC	:	715.909 KHz

PAL	:	715.379 KHz

However, these timing signals remain constant across different Amiga specifi-

cations, from base A500 all the way to A3000 with 68040 accelerator board. As

a means of creating processor-independent delay loops, the CIAs are the prime

choice.

The best timer to use under most circumstances is the CIA-B Timer B,

and this timer can be found at $BFD600/$BFD700. It is a 16-bit timer, and to

use it requires setting control bits in the CIA control registers. But first

let's see the sequence of events in action:

MOVE.W	#TIMEVAL,D0	;Timer value

MOVE.B	D0,CIABTBLO	;set low byte

LSL.W	#8,D0

MOVE.B	D0,CIABTBHI	;set high byte

This sequence sets the CIA-B Timer B write only latches. These are registers

that hold the timing value until the control register forces this value to be

loaded into the actual timer counters and counting to begin. Having set the

timer latches, it is time to start the timer. An instruction that can be used

is:

MOVE.B	#%00011001,CIABCRB

If one refers to the Concise Hardware Reference Manual, and examines the bit

allocations of CIABCRB, the effect of the above instruction is:

1) Alarm bit (bit 7):don't care about it, so set

it to 0.

2) Inmode bits (Bits 6,5):Want timer to count the

incoming clock signal pulses, so set these bits

to 00.

3) Load bit (bit 4):Force timer to load the latch

values into the timer, and then count down to

zero, so set this bit to 1.

4) Runmode bit (bit 3):Set to 1 for one-shot mode

(since we only want a single delay).

5) Outmode bit (bit 2):Set to 0 to receive a pulse

on the line being sensed.

6) PBON bit (bit 1):Set to 0 (leave PB7 line alone

as it's needed elsewhere!).

7) Start bit (bit 0):Set to 1 to start the timer

counting down.

OK, now we've set off our timer. Now we need to be able to sense when it has

reaced the timeout condition (i.e., time is up). Well, the best way is to set

up the interrupt control register (CIABICR) to allow the CIA to generate an

interrupt upon a timeout. To allow Timer B to generate an interrupt within

the CIA itself, use:

MOVE.B	#%10000010,CIABICR

or similar. This writes the control bits into the CIA's interrupt latch, and

allows Timer B to create an interrupt within the CIA.

Now, reading the same register address will read the CIA-B interrupt

input bits, and if Bit 1 is set, then the timer has timed out. Of course, as

CIA-B will generate a level 6 interrupt (IPL6 on the 68000), it is possible

to write a level 6 interrupt handler to check for this, but it is also poss-

ible to read the register directly (especially if the level 6 interrupt vec-

tor is disabled or redirected by the programmer for other reasons).

Timer A on the CIA-B can also be used (as it is not in use for any

operating system purposes) but if the operating system has been killed off by

the programmer, ALL of the timers are free. The above code applies mainly if

at least some facets of the operating system are being kept alive (for true

friendliness within the multitasking environment, use the CIA Resource, and

the Exec OpenResource() call).

Also, the programmer can choose to test the bits of the CIA interrupt

register directly or write an interrupt handler to do it (INT2 if using the

CIA-A timers, INT6 if using CIA-B). Don't forget to enable the required in-

terrupt in the 4703 interrupt controller (custom chip-write to INTENA, at ad-

dress $DFF09A to control the required interrupts), and to change the value of

the 68000 status register if required by your code. This requires access to

supervisor mode, which can be done in two ways. First, there is an Exec call

that will run a piece of code in supervisor mode:

MOVE.L	ExecBase,A6

JSR	Superstate(A6)

MOVE.L	D0,Old_SSTk	;keep this value!

and allow recovery of user mode via:

MOVE.L	Old_SSTk,D0

MOVE.L	ExecBase,A6

JSR	UserState(A6)

or alternatively, if you're killing off the operating system, try this rather

naughty piece of code:

LEA	MyTrap0(PC),A0

MOVE.L	A0,$80		;addr of TRAP #0 vector

TRAP #0

...		;from here on, MOVE SR is

;allowed as you are in

;supervisor mode!

...

MyTrap0		MOVE.W	(SP)+,D0	;get status reg

BSET	#13,D0		;set supervisor mode

MOVE.W	D0,-(SP)	;put back

RTE			;return in supervisor mode!

This is a very naughty piece of code (which destroys multitasking integrity)

of the sort that a games coder might like to use when returning to the oper-

ating system is not called for.

Now that we can implement CIA timer delay loops (needed later on), it

is time to look at MFM and GCR data encoding.

Hardware Disc Access:MFM And GCR Encoding

Those who haven't encountered direct writing to disc before may be surprised

to find that the actual bit patterns written to the disc differ from the bit

patterns of the data that the programmer writes to disc when using AmigaDOS

routines. Well, all is to be revealed.

The fundamental problem of writing data to a disc is detecting the

difference between a zero bit and a one bit. The next problem is detecting

the difference between individual bits, be they ones or zeros. Since the disc

read/write head uses magnetic fields for its operations, there are only two

possibilities:magnetic field on and magnetic field off. Allocating each of

these signals to a zero and one bit respectively would render detection of an

individual bit impossible.

To solve this problem, a scheme called frequency modulation was de-

vised, whereby CHANGES in the magnetic field value written to the disc were

used to signal the start of a bit, and the TYPE of change encountered used

to determine which type of bit was written during the original write opera-

tion. This scheme works for low-volume data storage, but when the density of

the data exceeds a certain value, the frequency of read/write errors builds

up to an unacceptable value. For the technically minded, the problem is that

high-speed flux changes take time to generate the respective electronic sig-

nal change, a time known as the half zero bit length, and the minimum value

of this is the main determinant of the maximum density of data storable on a

disc. Also, there is a delay during initiation of read/write operations, this

value being the precompensation value (although Amiga disc drives are capable

of handling read/write operations with zero precompensation). Generally, the

faster the read/write operation, the more precompensation needed.

Overcoming the problem of large precompensation values for densely

packed data on a disc led to the formation of Modified Frequency Modulated

encoding (MFM). This encoding scheme expands the actual data to twice its

original size, but in doing so generates raw data to write to the disc that

is efficient enough to allow 880K of user data to be stored on a 3.5 inch

disc.

So, what is MFM? The exact relationship is:

User Data Bit		MFM Bits

-------------		--------

1		  01

0		  10 if following a 0 data bit

0		  00 if following a 1 data bit

Now according to the Amiga Hardware Reference Manual, it is possible to use

the blitter to encode and decode the MFM data. Well, for the record, I have

not tried this, so those willing to experiment might like to disassemble the

trackdisk.device and see if Commodore have written blitter MFM routines (and

if they haven't, why mention this?). Really clever programmers reading this

can write their own.

GCR encoding is an entirely different matter. Sad to say, Commodore-

Amiga do NOT give any in-depth information on this subject, other than to say

that a translation table (called a nibbliser table) is used to prevent too

many consecutive 1 or 0 bits occuring in the data stream. Users who want this

information should obtain a comprehensive reference work covering hardware-

level disc formats and floppy disc hardware in general. Anyone familiar with

Apple II (there's nostalgia for you!) disc drives and their hardware-level

format, or the Apple Macintosh disc format, please get in touch so that I can

provide information on GCR encoding for Amiga users!

As I was perusing the subject, a piece of code that performed MFM en-

coding and decoding without the need for the trackdisk device appeared on ACC

disc 18, so I decided to have a further look. The code turned out to be quite

interesting (as it had no comments explaining it-thanks Mark Flemans for an

example of the minimalist approach to documentation!), but apparently works.

Hardware Disc Access:Basic I/O

Now is the time to start using the registers and performing some actual disc

operations. The following code is all my own, to be considered freeware and

rippable/editable to the reader's preferences/sexual leanings etc.

First, disc access via the DMA channels. This is the usual method, &

has the advantage of allowing the 68000 to get on with other processing while

a disc is being read. Now users with one disc drive will not normally need to

consider the drive select signals, but I shall include the drive select code

so that the code can be used by users with more than one drive. The sequence

of operations looks like this in assembler:

MOVE.B	#$78,CIABPRB	;switch ON motor, no drive select

MOVE.B	#$F0,CIABPRB	;select drive 0

bwait1		BTST	#5,CIAAPRA	;drive ready? (Busy wait-sigh)

BNE.S	bwait1		;wait until so

LEA	$DFF000,A5	;point to custom chips

MOVE.L	#BUFFER,DSKPTH(A5)	;point to our buffer

* The two statement sets below are commented out. Remove the comment

* characters for the operation wanted.

*		MOVE.W	#$4000,DSKLEN(A5)	;turn off secondary disc DMA

*		MOVE.W	#$8000,D0		;secondary DMA on, READ DISC

*		ADD.W	#DATASIZE,D0		;no of WORDS to READ

*		MOVE.W	D0,DSKLEN(A5)		;need to write TWICE to

*		MOVE.W	D0,DSKLEN(A5)		;start the operation!

*		MOVE.W	#$4000,DSKLEN(A5)	;turn off secondary disc DMA

*		MOVE.W	#$C000,D0		;secondary DMA on, WRITE DISC

*		ADD.W	#DATASIZE,D0		;no of WORDS to WRITE

*		MOVE.W	D0,DSKLEN(A5)		;need to write TWICE to

*		MOVE.W	D0,DSKLEN(A5)		;start the operation!

... here wait for DMA to complete

MOVE.W	#$4000,DSKLEN(A5)	;turn off disc DMA again

MOVE.B	#$F8,CIABPRB		;turn motor OFF

MOVE.B	#$F0,CIABPRB		;select drive 0

Ok, some notes about the above code. First, the motor is switched on, with

no drive select signals sent. Because the signals are ACTIVE LOW, the bits

to turn OFF a function are SET. The same applies to the drive select signal

immediately following.

Now anyone who has read my previous DOC files will know that I HATE

busy waits, such as the one that I've sadly included here. Well, this code is

intended to be simple to understand, so I'll break the golden rule and use a

busy wait to wait for the drive to be ready. This just waits for the DSKRDY

bit to go low. Of course, an alternative is to use the disk index pulse bit

to trigger a level 6 interrupt, and then read DSKRDY within the interrupt,

while your code does something more useful than twiddle its thumbs.

Now, point A5 at the custom chips, to make life easier (especially if

you use the hardware.i ACC include file), then point the DSKPTH/L registers

at the buffer of your choice.

Here, we decide whether to read or write to disc. Both code sections

are included above. Just remove the comment markers from the one required, &

delete the other.

Note that in order to set off a DMA disc operation, the DSKLEN regi-

ster must be written to TWICE. This is intended to prevent accidental writes

to a disc (which are of course always to be avoided!), and ensure that when

a disc operation is requested, the hardware receives a genuine request. The

above code does this.

Of course, it is possible to read or write the 'hard' way, using the

68000 to keep track of things, but the above is more efficient (barring the

busy wait).

Now, some restrictions. First, the above should be used to read or

write an entire track at a time. It is possible to modify it to read in one

sector at a time, but the above is not intended to be an advanced routine.

Sector-level disc I/O requires using the DSKBYTR and DSKSYNC registers, and

also probably requires handling via interrupts not only for maximum effic-

iency, but for maximum data integrity too. When using the above code for a

track read or write, the size of the buffer should be:

(512 * 11)	   *	2	+ 2	= 11266 bytes

Size of 1 track        MFM

Overhead

The reason for the size given is 1) a single sector is 512 bytes of user data

on a standard AmigaDOS disc; 2) there are 11 sectors per track; 3) all data

in MFM format is expanded to twice the uncoded size; 4) an extra word is to

be reserved to take account of a hardware bug (see below) that Commodore just

happen to have mentioned in the Amiga Hardware Reference Manual.

Now, there remains another task to be performed before reading data

from a disc. That is to make sure that the data coming off the disc is syn-

chronised to the actual data layout. The programmer wants to be sure that a

data stream coming in actually STARTS at the first word of the first sector

of the track being read. How is this done?

Well, for an AmigaDOS disc, load DSKSYNC with the magic value $4489.

This is a value called a 'sync mark'. This value is chosen so that there is

no way possible that any data on the disc encodes to this value. There are a

number of such values, but AmigaDOS uses $4489. Normal user data will never

encode to this number using MFM encoding as given above, and so the disc sys-

tem can use this value to mark the start of the track.

Having loaded DSKSYNC with the sync mark value, enable the WORDSYNC

bit in ADKCON prior to reading or writing. Then, data transfer will NOT be

performed until the sync mark is found, at which point the following word of

data on the disc will be the start point for data transfer. Use:

MOVE.W	#$4489,DSKSYNC(A5)

where A5 points to the start of the custom chip registers ($DFF000). Then set

WORDSYNC in ADKCON:

MOVE.W	#$9500,ADKCON(A5)

which sets the WORDSYNC, MFMPREC and FAST bits. Do this BEFORE the main code

for reading or writing discs above.

Hardware Disc Access:Track Selection

This is the final basic function to be performed. Track selection is a mite

complicated, because the drives do not signal directly (except for track 0)

when the heads are aligned over a given track. So, to perform this function,

the programmer MUST first seek to track 0 (at which point the DSKTRACK0 bit

of CIAAPRA will become zero. This requires first that the drive be selected,

then the motor turned on, then DSKDIREC set to 1 to force a seek to the outer

edge of the disc (where track 0 lies). Then, send a step pulse to DSKSTEP as

outlined above until DSKTRACK0 goes low. Then, SAVE this track number in a

variable set up within the program for this purpose. From this point on, to

find a given track requires a seek in the given direction, incrementing the

variable each time a step inwards is performed, and decrementing the varia-

ble when a step outwards is performed. Locating a specified track then requ-

ires a compare with the desired track number. For example, after selecting a

drive and turning on the motor, code such as the following may be used:

FindTRK0	btst	#4,CIAAPRA	;hit track 0?

beq.s	FoundTRK0	;skip if so

move.b	#%11110010,CIABPRB	;seek to outer, DF0:

...	;here set up a delay using a CIA timer

...	;for 18 milliseconds

move.b	#%11110001,CIABPRB	;set DSKSTEP high

move.b	#%11110000,CIABPRB	;then low

move.b	#%11110001,CIABPRB	;then high again

...	;here set up 3 ms delay using

...	;a CIA timer

bra.s	FindTRK0		;and check if hit track 0

FoundTRK0	clr.w	TrackNumber	;absolute variable (sigh)

...			;from here, can seek anywhere!

Having used this, code to seek in a given direction can use a second varia-

ble (call it TrackWanted). A CMP between the two will establish the seek di-

rection. Alternatively, use:

move.w	TrackNumber,d0

move.w	TrackWanted,d1

sub.w	d0,d1		;check seek direction

beq.s	Done		;already there!

bpl.s	Seek_Inward

neg.w	d1

move.b	#%11110110,CIABPRB	;set DSKDIREC outwards

...		;here CIA timer delay, 18 ms

Seek_Out	move.b	#%11110001,CIABPRB	;set DSKSTEP high

move.b	#%11110000,CIABPRB	;then low

move.b	#%11110001,CIABPRB	;then high again

subq.w	#1,TrackNumber		;update track count

cmp.w	TrackWanted,TrackNumber	;found req'd track?

beq.s	Seek_Done		;got it!

subq.w	#1,d1

bne.s	Seek_out

...

Seek_Inward	move.b	#%11110100,CIABPRB	;set DSKDIREC inwards

...		;here CIA timer delay, 18 ms

Seek_In		move.b	#%11110001,CIABPRB	;set DSKSTEP high

move.b	#%11110000,CIABPRB	;then low

move.b	#%11110001,CIABPRB	;then high again

addq.w	#1,TrackNumber		;update track count

cmp.w	TrackWanted,TrackNumber	;found req'd track?

beq.s	Seek_Done		;got it!

subq.w	#1,d1

bne.s	Seek_In

Seek_Done	...

This should cover most needs for hardware disc programmers. Those requiring a

routine to perform sector-level access, be warned that the facilities provi-

ded for sector-level access are limited. First, the DSKBYTR register must be

polled on a regular basis while the track is being scanned. Second, since the

disc system does NOT maintain sector counters in hardware, the programmer has

to provide variables for this purpose within the program. Unless there is a

shortage of available memory, and sector-level access is the ONLY option open

to the programmer, then reading an entire track in one go is a far superior

option.

Hardware Disc Access:Hardware Bugs

Now the bad news. The Amiga Hardware Reference Manual contains a short para-

graph (tucked away where it can't be seen too easily so as not to alarm the

first-time hardware programmer!) which mentions a disc hardware bug. Well, it

is really two bugs, one occurring during a read operation, and one occurring

during a write operation.

Firstly, during a read operation, the last word requested may not be

read. This doesn't always happen, but just in case it does, the buffer for a

hardware disc read should be one word longer than necessary. Also, for write

operations, the extra last word should match the first word of the buffer. I

forgot to reserve space for sync marks in the buffer information above (sorry

about that!), so make the first word the sync mark, then a track full of data

and finally a copy of the sync mark.

Hardware Disc Access:File Updates

For updated versions of this file, or any of the other files in this series

of DOC files, the contact addresses are:

Dave Edwards		Mark Meany

232 Hale Road		12 Hinkler Road

WIDNES			Thornhill

Cheshire		SOUTHAMPTON

WA8 8QA			Hants

The first of these is mine. Sending a blank formatted disc to me at the above

address, with a stamped, self-addressed jiffy bag for return postage (stand-

ard 1st class stamp sufficient) will get you the files of your choice, writ-

ten to your disc in uncompressed ASCII format. Sending a disc to Mark at the

above address (plus jiffy bag, stamp etc.) will get you one of the ACC club

discs which contain these files spread over a period (they were intended for

the ACC club readership initially, and are now Public Domain DOCs distributed

as part of the ACC club material by Mark). All DOC files appearing on the ACC

Club discs are in compressed format, and you'll need the packer/depacker that

is supplied with the ACC discs to be able to edit them, if this is your wish.

Should anyone out there have information to add to the files, please

contact either Mark or myself at the addresses above, sending your info as an

uncompressed ASCII file. I shall add the info to the relevant file when time

allows, and send off the latest version upon request. All contributors will

be acknowledged gratefully.

**** VERY IMPORTANT ****

Since the DOC files have now grown alarmingly in number (and size-they occupy

nearly 3MB when decompressed!) from 1/10/91 onwards, ALL requests to me for a

copy of the DOC DISC should be accompanied by TWO blank discs for each set of

DOC DISCS required. So, if three sets of DOC DISCS are required, the reader

should now send SIX blank discs, for example. Until I can arrange with Mark

to have the latest updates sent to him, please send ALL requests for DOC DISC

copies directly to me until some time in December 1991 (by which time the ACC

should have the latest updates via Mark). To give the reader some idea of the

monumental size of the DOCS, the files occupy 900K AFTER compression, and at

the time of typing occupy 2,787,490 bytes when unpacked! So, this should tie

your reading time up for the next three years...

UPDATE	: typed_hardware_disc.doc current as of 14/2/92

Contributors : Dave Edwards

Mark Meany (ACC 17)

Vandal of Killers (Snippets, ACC 17)

Mark Flemans (ACC 18, movetrack00.s &

tcode.s