MOTOROLA MC68020 INSTRUCTION SET

================================

This file contains ALL of the extra instructions and addressing modes found

on the Motorola MC68020 processor, as found in Amiga 2000 machines with acc-

elerator cards fitted, and which can be executed (with very few exceptions)

on Amiga 3000 machines fitted with the Motorola MC68030 processor.

The file shall be divided into several sections. First, I shall cover

the additional addressing modes available to MC68020 assembly language prog-

rammers, and the syntaxes used for those new modes. Then, I shall cover fully

the instruction extension words applicable to the MC68020, which follow the

main instruction opcodes and provide the MC68020 with the instruction infor-

mation available to access the new addressing modes. And finally, the comp-

lete set of MC68020 extra instructions PLUS OPCODES follow, documented accor-

ding to the standards laid down in the Official Motorola Documentation.

MC68020 - NEW ADDRESSING MODES

==============================

The principal new modes available to the MC68020 fall into two classes:memory

indirect addressing, and bit field addressing. First, I shall outline some of

the conventions used throughout this file. They are:

<EA>	: Effective Address. Stands for the

address of the operand under con-

sideration, whether computed at

assembly-time or runtime.

{ }	: Curly braces are used to denote a

bit field. Bit fields are covered

below.

off	: Offset for bit fields.

wd	: Width for bit fields.

[ ]	: Denotes "contents of".

bd	: Stands for Base Displacement in

memory indirect addressing.

od	: Stands for Outer Displacement in

memory indirect addressing.

An	: One of the address registers A0-A7.

Xn	: Any 680x0 data or address register

used in the context of an index

register.

scale	: The multiplier applied to the value

of the index register in memory in-

direct addresing.

size	: The size attribute applied to the

index register in memory indirect

addressing.

And now, for the various memory indirect addressing modes.

1) Memory Indirect Preindexed (using Address Register).

Generation	: <EA> = [bd + [An] + [Xn.s]*size] + od

Assembler	: ([bd,An,Xn.s*size],od)

Mode		: 110 in instruction address mode field

Description	: This addressing mode uses two constant displacements,

an address register, and an index register, PLUS a

scale factor (five components in all).

First, the contents of the specified Address Register

are added to the value of the Base Displacement. Then,

the contents of the specified Index Register are mul-

tiplied by the specified scale factor (1,2,4 or 8),

and this value is added to the whole.

This value is then used as a pointer into memory, and

a longword pointer extracted from memory. The value of

the Outer Displacement is then added to the value of

this longword pointer, and this value becomes the

effective address for the specified operand.

The legal ranges for the various components are:

An		: Any 680x0 address register, A0-A7.

bd		: Base displacement. Can be null, or

can be word or longword sized. Word

sized displacements are sign-extended

to longword values prior to addition.

Xn		: Index register. Can be either a

680x0 data register or an address

register.

s		: Either .W for word operands, or

.L for longword operands. NOTE:

word values are sign-extended to

longword values prior to addition

in common with all 680x0 address

arithmetic.

size		: The legal values for the scale

factor are 1,2,4 or 8.

od		: Outer displacement. Can be null,

or can be word or longword sized.

Again, word-sized displacements

are sign-extended to longword dis-

placements prior to addition.

Example		: Let bd = $40000, od = $8000, A0 contain the value

$20000, D2 contain the value $100, and the longword

value stored at location $60400 be $30000.

Then, the effective address of:

([$40000,A0,D2.W*4],$8000.w)

is:

1) $40000 + $20000 + ($100 * 4)

= $60400

2) Contents of $60400 = $30000

3) $30000 + $8000.w = $0003000 + $FFFF8000

= $28000

Therefore the effective address is $28000.

2) Memory Indirect Postindexed (using Address Register).

Generation	: <EA> = [bd + [An]] + [Xn.s]*size + od

Assembler	: ([bd,An],Xn.s*size,od)

Mode		: 110 in instruction address mode field

Description	: This addressing mode uses the same five components as

Memory Indirect Preindexed, but in a different order.

First, the contents of the specified address register

are added to the value of the Base Displacement. This

value is used as a pointer into memory, from which a

longword value is extracted. This value has added to

it the value of the contents of the specified index

register (multiplied by the given scale factor) and

the value of the Outer Displacement, to create the

final effective address.

The range of legal values for the various components

id identical to that for Memory Indirect Preindexed.

Example		: Let bd = $1000, od = $20000, A2 contain the value

$10000, D7 contain the value $40000, and the long-

word value stored at $11000 be $58000.

Then, the effective address of:

([$10000,A2],D7.L*1,$20000)

is:

1) $1000 + $10000 = $11000

2) Contents of $11000 = $58000

3) $58000 + $40000 + $20000 = $B8000

Therefore the effective address is $B8000.

3) Program Counter Memory Indirect With Preindex.

Generation	: <EA> = [bd + [PC] + [Xn.s]*size] + od

Assembler	: ([bd,PC,Xn.s*size],od)

Mode		: 111 in instruction address mode field

Description	: Add to the value of Base Displacement the value of the

Program Counter, and the value contained in the speci-

fied Index Register (multiplied by the specified scale

factor). Use this total value as a pointer into memory

and extract a longword value from this memory location.

Add to this longword value the value of the Outer Dis-

placement. This final result is the effective address

of the operand.

Example		: Let bd = $4000, PC = $2C00, D5 = $FFFF, and od = $1000.

Let location $6BFE contain the longword $5A800.

Then, the effective address of:

([$4000,PC,D5.W*2],$1000)

is:

1) $4000 + $2C00 + ($FFFFFFFF * 2)

= $6BFE

2) Contents of $6BFE = $5A800.

3) $5A800 + $1000 = $5B800.

Therefore the effective address is $5B800.

4) Program Counter Memory Indirect With Postindex.

Generation	: <EA> = [bd + [PC]] + [Xn.s]*size + od

Assembler	: ([bd,PC],Xn.s*size,od)

Mode		: 111 in instruction address mode field

Description	: Add to the value of Base Displacement the value of the

Program Counter, and use this as a pointer into memory.

Extract the longword value stored at this address, and

add to this extracted value the value of the specified

Index Register (multiplied by the specified scale fac-

tor), and the value of the Outer Displacement. This fi-

nal value is the effective address of the operand.

Example		: Let bd = $6000, PC = $1A00, D1 = $0020, and od = $7000.

Let location $7A00 contain the longword $FC0000.

Then, the effective address of:

([$6000,PC],D1.L*4,$7000)

is:

1) $6000 + $1A00 = $7A00

2) Contents of $7A00 = $FC0000.

3) $FC0000 + ($0020 * 4) + $7000 = $FC7080.

Therefore the effective address is $FC7080.

Note that with ANY of the memory indirect modes, the programmer can omit any

desired component for the desired effect. For example:

+

68020 Instruction			68000 Equivalent

MOVE.L	([Base]),D0			MOVE.L Base,A0

MOVE.L (A0),D0

MOVE.L	([Base,A0]),D0			MOVE.L	Base(A0),A0

MOVE.L	(A0),D0

JMP	([0,A0],D0.W*4)			ADD.W	D0,D0

ADD.W	D0,D0

MOVE.L	0(A0,D0.W),A0

JMP	(A0)

MOVE.W	([Base,A0],D2.L*8,Outer),D7	MOVE.L	Base(A0),A0

ADD.L	D2,D2

ADD.L	D2,D2

ADD.L	D2,D2

LEA	Outer(A0),A0

MOVE.L	0(A0,D2.L),D7

Note that the 68020 forms can complete the given operation in one instruction

whereas the 68000 needs two or more instructions to perform the same task, &

requires extra register use. Hence the use of the '+' character to highlight

the fact that the equivalence is NOT complete - in general the 68020 does NOT

change the contents of the registers in the source operand UNLESS the destin-

ation happens to be one of those registers also. Note also how the 68020 can

access a jump table in one instruction - see the 68000 equivalent alongside!

It can be seen from this that the 68020 addressing extensions are so

far ahead of the already powerful 68000 addressing modes (which still kick an

Intel chip into a cocked hat!) that Motorola didn't need to add any more to

the 68030 and 68040 (after all, many programmers still can't cope with single

instructions that are equivalent to half a C program...).

It remains to cover bit fields. Bit fields are so radically different

from normal addressing modes that they do take a little getting used to. The

basic idea is that any collection of bits, of any size (within certain rest-

rictions), aligned anywhere in memory (not even on a byte boundary!) can be

accessed and manipulated.

A bit field is denoted by the legend:

{offset:width}

where either of the values 'offset' or 'width' can be specified within an in-

struction by a constant value or a data register reference. A comparison of

bit addressing (for instructions such as BCHG) and bit field addressing is:

7 6 5 4 3 2 1 0

Byte N-1 |	Byte N		|   Byte N+1	| Byte N+2

Bit Addressing

7 6 5 4 3 2 1 0

Byte N-1 |	Byte N		|   Byte N+1	| Byte N+2

0 1 2 3 4   5 6 7 8 9 ...

<---->

offset

<------------------------>

width

Bit Field Addressing

In the above, a Bit Field is a consecutive string of 1 to 32 bits, specified

by a Base Address (the address of the byte containing Bit 0 of the Bit Field,

byte N in the example above), an Offset (the distance in bits from the most

significant bit of the Base Address) and the Width (number of bits that form

the Bit Field).

If the Offset is specified using an immediate data value, then the

Offset ranges from 0 to 31. Simialrly, the Width ranges from 1 to 32, but in

this case the value is limited to the range 1-32 whether specified as an im-

mediate value or as a data register reference.

When a data register is used, then the Offset specification can range

from:

31		 31

-2	to	2  - 1

which means that the first bit of the Bit Field might reside in a byte which

is many bytes distant from the Base Address defined in any instruction Bit

Field reference! This facility allows scanning of Bit Field Arrays, an exam-

ple of which is:

Byte:		N		N+1		N+2	  ...

Bit : 7 6 5 4 3 2 1 0 | 7 6 5 4 3 2 1 0 | 7 6 5 4 3 2 1 0 ...

BF  : 1 1 1 1 1 2 2 2   2 2 3 3 3 3 3 4   4 4 4 4 5 5 5 5 ...

Here, Bit Field 2 follows Bit Field 1 in the array immediately. If Byte N is

the Base Address for the entire Bit Field Array, then a data register can be

used to contain the Offset, and the next element in the array referenced by

adding the value of the Width (here, 5) to the value of the Offset contained

in the data register, and use one address to reference the entire array with-

out having to recompute the Base Address! In this case, our Bit Field Array,

containing Bit Field elements of 5 bits, can contain up to 4 BILLION elements

(assuming sufficient memory to store them all) all referenced via the same

Base Address!

And by way of example, for the above Bit Field Array, if register A0

of the MC68020 points to Byte N above, then the various elements are referen-

ced as follows using immediate offset and width values:

Element #1	: (A0){0:5}

Element #2	: (A0){5:5}

Element #3	: (A0){10:5}

....

Element #8	: 4(A0){3:5} (NOT (A0){35:5} !!!)

Element #9	: 4(A0){8:5}

etc.

MC68020 EXTRA REGISTERS

=======================

The MC68020 possesses a number of extra registers, available to programs ex-

ecuting in supervisor mode. Also, the status register contains extra bits. A

summary of the registers is as follows (all registers are 32 bits wide unless

otherwise stated):

D0 - D7 : Data Registers as per MC68000.

A0 - A7	: Address Registers as per MC68000.

SR	: Status Register (16 bits). See below.

CCR	: Condition Code Register as per MC68000.

A7'	: Also ISP : Interrupt Stack Pointer.

A7''	: Also MSP : Master Stack Pointer.

VBR	: Vector Base Register.

CACR	: Cache Control Register (4 bits).

CAAR	: Cache Address Register.

SFC	: Source Function Code Register (3 bits).

DFC	: Destination Function Code Register (3 bits).

The status register bits are allocated as follows:

Bit	Meaning

---	-------

0	Carry Flag (CCR).

1	Overflow Flag (CCR).

2	Zero Flag (CCR).

3	Negative Flag (CCR).

4	Extend Flag (CCR).

8	IPL0 Interrupt Bit.

9	IPL1 Interrupt Bit.

10	IPL2 Interrupt Bit.

12	Master Stack Bit.

13	Supervisor Mode Bit.

14	Trace On Change Of Program Flow.

15	Trace (Single-Step Through Program).

The Vector Base Register allows different tasks to have different sets of ex-

ception vectors located in different areas of memory. The standard addresses

for the MC68000 exception vectors are now treated as offsets to be added to

the contents of the VBR, and when a MC68020 exception occurs, the exception

vector will be fetched from this new address. When the VBR contains zero, the

MC68020 operates in an identical fashion to the MC68000.

The Supervisor Stack Pointer (SSP) is now divided into two different

entities. The Interrupt Stack Pointer (ISP) is accessed when the Master Stack

Bit of the Status Register is 0 (in which case, behaviour is identical to the

MC68000). When the M bit is 1, then the Master Stack Pointer is used instead

of the Interrupt Stack Pointer. The MC68020 can also access the User Stack

Pointer (USP) in supervisor mode via the appropriate instructions as per the

MC68000, but also has available several new instructions to access the USP.

The Source Function Code (SFC) and Destination Function Code (DFC)

Registers allow MC68020-based systems equipped with a MC68851 Paged Memory

Management Unit (PMMU) to access areas of memory protected according to the

definitions of the 'address space' within the PMMU. The MOVEC instruction is

used to change the contents of these registers (in supervisor mode only!) to

allow a supervisor-mode program to read to or write from memory protected by

a PMMU. The values that these registers store are:

000	Undefined, Reserved.

001	User Data Space.

010	User Program Space.

011	Undefined, Reserved.

100	Undefined, Reserved.

101	Supervisor Data Space.

110	Supervisor Program Space.

111	CPU Space (see CALLM, BKPT).

These registers are used in conjunction with the MOVES instruction in

supervisor mode to allow supervisor-mode programs to transfer data to either

user program/data space or supervisor program space without causing a PMMU

exception condition (memory access violation).

The Cache Control Register (CACR) allows the on-chip cache memory to

be controlled. The bits of the CACR are:

Bit	Meaning

---	-------

0	E bit : 1 = enable cache

1	F bit : 1 = freeze cache (prevent

updating)

2	CE bit : 1 = invalidate single cache

entry (CPU clears valid

bit for entry)

3	C bit : 1 = invalidate all cache entries

The entries to be invalidated using the C/CE bits are controlled by the Cache

Address Register (CAAR). The address of the invalidated entry is written here

PRIOR to writing to the C/CE bits using MOVEC, and allows logical or virtual

addresses to be invalidated prior to task switching in a multiprogramming en-

vironment.

MC68020 EXCEPTION VECTOR TABLE

==============================

The MC68020 has an extended exception vector table. This vector table is de-

fined by the table below. In the table, SD refers to Supervisor Data Space,

and SP to Supervisor Program Space. FPCP refers to the Floating-Point Copro-

cessor, and PMMU to the Paged Memory Management Unit.

Address	Vector	Assignment

-------	------	----------

$0000	0  SP	Reset:Initial ISP

$0004	1  SP	Reset:Initial PC

$0008	2  SD	Bus Error

$000C	3  SD	Address Error

$0010	4  SD	Illegal Instruction

$0014	5  SD	Zero Divide

$0018	6  SD	CHK/CHK2 Instruction

$001C	7  SD	cpTRAPcc, TRAPcc, TRAPV Instructions

$0020	8  SD	Privilege Violation

$0024	9  SD	Trace Exception

$0028	10 SD	Line-A Emulator (1010)

$002C	11 SD	Line-F Emulator (1111) & Coprocessor Instructions

$0030	12 SD	Unassigned, Reserved

$0034	13 SD	Coprocessor Protocol Violation

$0038	14 SD	Format Error

$003C	15 SD	Uninitialised Interrupt

$0040 to   SD	Unassigned, Reserved (16-23)

$005C

$0060	24 SD	Spurious Interrupt

$0064	25 SD	Level 1 Interrupt Autovector (IPL1)

$0068	26 SD	Level 2 Interrupt Autovector (IPL2)

$006C	27 SD	Level 3 Interrupt Autovector (IPL3)

$0070	28 SD	Level 4 Interrupt Autovector (IPL4)

$0074	29 SD	Level 5 Interrupt Autovector (IPL5)

$0078	30 SD	Level 6 Interrupt Autovector (IPL6)

$007C	31 SD	Level 7 Interrupt Autovector (IPL7)

$0080 to   SD	TRAP 0-15 Instruction Vectors (32-47)

$00BC

$00C0	48 SD	FPCP Branch Or Set on Unordered Condition

$00C4	49 SD	FPCP Inexact Result

$00C8	50 SD	FPCP Divide By Zero

$00CC	51 SD	FPCP Underflow

$00D0	52 SD	FPCP Operand Error

$00D4	53 SD	FPCP Overflow

$00D8	54 SD	FPCP Signalling NAN (Not-A-Number)

$00DC	55 SD	Unassigned, Reserved

$00E0	56 SD	PMMU Configuration Error

$00E4	57 SD	PMMU Illegal Operation

$00E8	58 SD	PMMU Access Level Violation

$00EC to   SD	Unassigned, Reserved (59-63)

$00FC

$0100 to   SD	User Defined Vectors (64 to 255)

$03FC

Special exception vectors new to the MC68020 include the cpTRAPcc and TRAPcc

instruction vector (shared with the MC68000 TRAPV vector) for handling those

instructions, and the range of coprocessor handling exceptions. These include

execptions for the MC68881/MC68882 mathematics coprocessor (FPCP), and the

MC68851 Paged Memory Management Unit (PMMU).

The MC68020 now saves exception-generated data on the ISP or MSP in

the form of Stack Frames. These take a number of formats, and if the MC68020

encounters a Stack Frame that does not conform to these formats, then the CPU

takes the Format Error exception. This can be triggered by an illegal format

encountered during RTE, CALLM/RTM or cpRESTORE. This occurs if the Format

field (see below) does not conform to the expected specifications.

The handling of the F-line exception changes in systems containing a

MC68020 plus one or more coprocessors. In such systems, the exception is ta-

ken only if the coprocessor(s) do not recognise an instruction passed to them

unless one of the coprocessor-specific exceptions occurs.

Also changed is the Address Error handling-now, an address error only

occurs when an attempt is made to execute an instruction that is not word-al-

igned. The MC68020 can access word or longword data that is not aligned, but

with a degradation in performance as a result. The processor creates a Bus

Error stack frame.

The Bus Error exception is now created by nonexistent memory, memory

errors reported to the CPU by external circuitry, or memory access errors of

certain types reported by a MC68851 PMMU. Some coprocessor errors can also be

the cause of a Bus Error-exercise CARE!

Coprocessor exceptions include the Floating-Point Coprocessor (FPCP)

exceptions, which include floating-point underflow/overflow, inexact result,

operand error, and a condition called 'Signalling NAN'. This refers to the

use of certain special values (called NANs or 'Not-A-Number's) to signal to

the CPU certain special conditions not signalled via the normal routes. As

well as the FPCP exeptions, there are Paged Memory Management Unit (PMMU) ex-

ceptions, including illegal operation, access violation (caused by operations

such as an attempt by a user-mode program to write to a memory area designa-

ted as part of the supervisor data space by the PMMU, or an attempt to issue

a CALLM instruction without having been granted the correct access level), &

the Configuration Error exception which is caused by faults in the operating

system and its memory management (the only cure for which is to abort and re-

boot the operating system!).

The final exception does not have a vector assigned to it-because it

is a complete catastrophe for the system. A double bus fault (Bus Error with-

in a Bus Error exception handler, for example) is considered to be so bad for

the system that the processor enters the Halted State. The ONLY way out is an

external RESET!

MC68020 EXCEPTION STACK FRAMES

==============================

The MC68020 stores exception-generated data onto the ISP or MSP in the form

of a Stack Frame. At least four words are stored on the stack, as follows:

SP	Status Register

SP+2	Program Counter High Word

SP+4	Program Counter Low Word

SP+6	Format Word

The Format Word is defined by:

F F F F V V V V V V V V V V V V

where FFFF is the format type field, and VVVV... is the Vector Offset field,

identifying the exception number taken. Legal values for the FFFF field, and

the number of words saved on the stack, are:

0000	Short Format (4 words)

0001	Throwaway Format (4 words)

0010	Instruction Exception (6 words)

0011 to Unassigned, Reserved

0111

1000	MC68010 Bus Fault (29 words)

1001	Coprocessor Mid-Instruction (10 words)

1010	MC68020 Short Bus Fault (16 words)

1011	MC68020 Long Bus Fault (46 words)

1100 to Unassigned, Reserved

1111

For example, the Format $02 Stack Frame is defined by:

SP	Value of SR

SP+2	Value of PC High Word

SP+4	Value of PC Low Word

SP+6	Format Word

SP+8	Instruction Address High Word

SP+10	Instruction Address Low Word

The next stack frames to be defined are the MC68020 Short Bus-Cycle Fault or

Format $0A Frame, and the Long-Frame Bus-Cycle Fault (or Format $0B) Frame.

First, the Format $0A Frame:

SP	Value of SR

SP+2	Program Counter (2 words)

SP+6	Format Word $AXXX

SP+8	Internal Register Value

SP+10	Special Status Word

SP+12	Instruction Pipe Stage C

SP+14	Instruction Pipe Stage B

SP+16	Data Cycle Fault Address (2 words)

SP+20	Internal Register Value

SP+22	Internal Register Value

SP+24	Data Output Buffer (2 words)

SP+28	Internal Register Value

SP+30	Internal Register Value

and finally, the Format $0B Frame:

SP	Value of SR

SP+2	Program Counter (2 words)

SP+6	Format Word $BXXX

SP+8	Internal Register Value

SP+10	Special Status Word

SP+12	Instruction Pipe Stage C

SP+14	Instruction Pipe Stage B

SP+16	Data Cycle Fault Address (2 words)

SP+20	Internal Registers (2 words)

SP+24	Data Output Buffer (2 words)

SP+28	Internal Registers (4 words)

SP+36	Stage B Address (2 words)

SP+40	Internal Registers (2 words)

SP+44	Data Input Buffer (2 words)

SP+48	Internal Registers (22 words!)

Information on the MC68010 Bus-Cycle Fault Format Stack Frame can be found in

the official Motorola MC68010 User's Manual, but is not of great use here.

MC68020 INSTRUCTION EXTENSION WORDS

===================================

The MC68020 extends the range of MC68000 instructions by using a range of ex-

tension words to the basic instructions. The format of a typical MC68020 in-

struction is:

Word 1 : x x x x x x x x x x M M M R R R

Word 2 : Extension word

Word 3 : Any extra required operand words

...

where the extra operand words follow in the order:source operands first, des-

tination operands next.

Here, the MMM and RRR fields determine the addressing mode of the in-

struction in accordance with the following table (applicable to ALL Motorola

MC680x0 processors unless otherwise stated):

MMM	RRR	Mode

---	---	----

000	xxx	Data Register Direct : xxx = register number 0-7

and operand is one of D0 - D7.

001	xxx	Address Register Direct : xxx = register number 0-7

and operand is one of A0 - A7.

010	xxx	Address Register Indirect : xxx = 0 to 7,

operand is one of (A0) to (A7).

011	xxx	Postincrement:xxx = 0 to 7, operand is one

of (A0)+ to (A7)+.

100	xxx	Predecrement:xxx = 0 to 7, operand is on

of -(A0) to -(A7).

101	xxx	Address Register Indirect With Displacement:

xxx = 0 to 7, operand is one of

disp(A0) to disp(A7). The displacement word

follows the instruction word.

110	xxx	Address Register Indirect With Displacement

And Index:xxx = 0 to 7, operand is one of

disp(A0,Xn) to disp(A7,Xn). The index regis-

ter Xn is specified in the extension word(s),

and the displacement follows this extension

word.

111	000	Absolute short:address of operand follows

instruction word as one word.

111	001	Absolute long:address of operand follows

instruction word as two words.

111	010	Program Counter Indirect With Displacement.

The displacement follows the instruction

word as one word.

111	011	Program Counter Indirect With Displacement

And Index (all 680x0 series), and Memory Indirect

(68020 onwards).

111	100	Immediate Data. The immediate data follows the

instruction word, using one or two words as is

required.

The extension words fall into two types:the brief extension word (which is a

slight generalisation of the standard MC68000 extension word), and the full-

format extension word (which allows memory indirect addressing modes etc).

First, the Brief Format extension word has the format defined by the

template below:

D X X X W S S 0 d d d d d d d d

The various fields in this extension word are:

D	: D/A Field. Specifies the index register type for

disp(An,Xn) and disp(PC,Xn).

0 = Data Register Dn

1 = Address Register An

XXX	: Register number field. Specifies which register is

the specified index register, ranging from 0-7.

W	: W/L field. Specifies whether the index register is

word sized or longword sized.

0 = word sized (value is sign-extended during

address computation)

1 = longword sized.

SS	: Scale field. Determines the scale in memory indirect

and the extended form of the MC68000disp(An,Xn)/

disp(PC,Xn) which on the MC68020 allows the use of a

scale multiplier. The values are:

00	: Multiplier = 1

01	: Multiplier = 2

10	: Multiplier = 4

11	: Multiplier = 8

The value SS=00 corresponds to the value used in the

original MC68000 extension word. The MC68020 allows

multipliers to be used as in:

MOVE.W	0(A0,D2.W*2),D3

ddd..	: Displacement field. Contains the byte value of the dis-

placement used in disp(An,Xn) and disp(PC,Xn) addressing

modes. This byte is sign-extended to a LONGWORD value

during computation of the effective address (so that a

value of $80 corresponds to -128).

The Full Format extension word is far more complex, since it must contain the

complete definition of any memory indirect addressing modes used. The Full

Format extension word is defined using:

D X X X W S S 1 B I Z Z 0 J J J

where:

D	: D/A field. Specifies whether index register is

a data register or an address register.

0 = Data Register Dn

1 = Address Register An

XXX	: Index Register Number 0-7.

W	: W/L Field. Specifies:

0 = Index register word-sized. Contents sign-

extended to longword prior to address com-

putation.

1 = Index register longword-sized.

SS	: Scale Factor field. Values are:

00	: Multiplier = 1

01	: Multiplier = 2

10	: Multiplier = 4

11	: Multiplier = 8

B	: Base Suppress (BS) field.

0 = Add contents of base address register to

base displacement.

1 = Suppress base address register-use base

displacement only.

I	: Index Suppress (IS) field.

0 = Evaluate index operand and add to address

1 = Suppress index operand, do not use.

ZZ	: Base Displacement Size (BD SIZE) field.

00 = Reserved.

01 = Null Displacement.

10 = Base Displacement Word Sized.

11 = Base Displacement Longword Sized.

JJJ	: Index/Indirect Selection (I/IS) Field.

Used in conjunction with the IS bit according to

the table below. 'od' refers as always to the

Outer Displacement value.

IS	I/IS		Operation

--	----		---------

0	000		No Memory Indirection

0	001		Indirect Preindex, NULL od

0	010		Indirect Preindex, word od

0	011		Indirect Preindex, long od

0	100		Reserved

0	101		Indirect Postindex, NULL od

0	110		Indirect Postindex, word od

0	111		Indirect Postindex, long od

1	000		No Memory Indirection

1	001		Indirect, No index, NULL od

1	010		Indirect, No index, word od

1	011		Indirect, No index, long od

1	100-111		All Reserved

Once a Full Format extension word follows an instruction, then the Base Dis-

placement words follow that word, and the Outer Displacement words follow on

from those. Of course, if either the Base Displacement or the Outer Displace-

ment do not exist (are NULL) then those words are omitted.

Note that for the MOVE instruction on the MC68020, it is possible to

select both source and destination Memory Indirect operands! In this case,

the order of the instruction words is as follows:

Word 1		: MOVE instruction

Word 2		: Source extension full format

Words 3,4	: Base Displacement if non-NULL,

omitted otherwise

Words 5,6	: Outer Displacement if non-NULL,

omitted otherwise

Word 7		: Destination extension full format

Words 8,9	: Base Displacement if non-NULL,

omitted otherwise

Words 10,11	: Outer Displacement if non-NULL,

omitted otherwise

Thus a 68020 instruction can consist of as many as 11 words (!) provided that

all of the specified components exist. Needless to say, executing such a MOVE

takes rather longer than MOVE.W D0,D1, for example, but gives some indication

of the enhanced power of the new 32-bit members of the family.

MC68020 EXTRA INSTRUCTIONS AND EXTENSIONS

=========================================

TO MC68000 INSTRUCTIONS

=======================

Before covering specific instructions and extensions to the standard MC68000

instructions allowed by the MC68020, the following should be noted:

1) Those instructions containing the MMM RRR fields to determine

the addressing mode of operands retain their original format,

extra values of those fields being used by the MC68020 to ac-

cess new addressing modes (e.g., memory indirect). The major

change is from the MC68000 instruction extension word for the

indexed operands to either the MC68020 Brief Format or MC68020

Full Format extension words (covered above);

2) If there ARE changes to the instruction format, then these

changes will be covered below.

The full list of extended and new instructions now follows.

Bcc	: Branch Conditionally (Includes BRA) (MC68020)

Operation : IF condition is TRUE

THEN

PC + d -> PC

ELSE

continue normal sequential operation

Assembler : Bcc <LABEL>

Attributes: Size=Byte,Word,Long

Description	: The conditional expression specified in the

instruction is evaluated and the result tested.

If the result is TRUE, then the displacement is

sign-extended to 32 bits and added on to the

current value of the program counter, at which

point program execution resumes. If the result

is FALSE, program execution continues sequen-

tially as normal.

Condition Codes : NOT AFFECTED.

Instruction Format :

0 1 1 0 C C C C D D D D D D D D

Instruction Fields :

CCCC	: Condition code field. Specifies the condi-

tional expression to compute prior to the

branch decision. Values are given in the

conditional branch table in the companion

file typed_68000.doc. A field value of 0000

corresponds to a BRA instruction, and a

field value of 0001 corresponds to a BSR in-

struction (see below).

DDDD..	: Displacement field. The 68020 accepts THREE

displacement sizes. For short branches, this

field contains the byte-sized displacement

which is sign-extended to 32 bits prior to

addition to the program counter.

If this field contains zero (00000000) then

the word following the instruction word con-

tains a word-sized displacement, which is

sign-extended to 32 bits prior to adding to

the program counter. This mode and the prev-

ious mode is available to ALL MC680x0 CPUs.

If this field contains $FF (11111111), then

the two words following the instruction word

contain a longword-sized displacement to be

added directly to the program counter. This

mode is available to the MC68020 onwards.

BFCHG	: Test Bit Field And Change

Operation : dst{off:wd} -> CCR

NOT(dst{off:wd}) -> dst{off:wd}

Assembler : BFCHG <ea>{off:wd}

Attributes: Unsized.

Description	: Test the state of the specified bit field and

alter the condition codes accordingly. Once

the condition codes have been duly altered,

invert all of the specified bits of the spec-

ified bit field.

Condition Codes : X N Z V C

- * * 0 0

N : Set if the most significant bit of the

bit field is zero. Cleared otherwise.

Z : Set if all the bits of the bit field are

zero. Cleared otherwise.

V : Always Cleared.

C : Always Cleared.

X : Not Affected.

Instruction Format :

1 1 1 0 1 0 1 0 1 1 M M M R R R

0 0 0 0 A O O O O O B W W W W W

Instruction Fields :

MMM, RRR: See MC68020 Instruction Extension

Words Above.

A	: A = 0 if the offset is an immediate

value, A = 1 if the offset is con-

tained in a data register.

OOO..	: Offset field. If the offset is an

immediate value, then the value 0-31

is stored here. If the offset is

contained in a data register, then

the register number D0-D7 is stored

here.

B	: B = 0 if the width is an immediate

value. B = 1 if the width is contained

in a data register.

WWW..	: Width Field. If the width is an immed-

iate value, then the values 0, 1-31 are

stored here, representing a width of

32, 1-31 respectively. If the width is

contained in a data register, then the

register number D0-D7 is stored here.

Addressing Modes : All modes EXCEPT:

An

(An)+

-(An)

#<data>

and PC relative modes, are allowed for the

destination operand.

BFCLR	: Test Bit Field And Clear

Operation : dst{off:wd} -> CCR

0 -> dst{off:wd}

Assembler : BFCLR <ea>{off:wd}

Attributes: Unsized.

Description	: Test the state of the specified bit field and

alter the condition codes accordingly. Once

the condition codes have been duly altered,

clear all of the bits of the specified bit

field.

Condition Codes : X N Z V C

- * * 0 0

N : Set if the most significant bit of the

bit field is zero. Cleared otherwise.

Z : Set if all the bits of the bit field are

zero. Cleared otherwise.

V : Always Cleared.

C : Always Cleared.

X : Not Affected.

Instruction Format :

1 1 1 0 1 1 0 0 1 1 M M M R R R

0 0 0 0 A O O O O O B W W W W W

Instruction Fields :

MMM, RRR: See MC68020 Instruction Extension

Words Above.

A	: A = 0 if the offset is an immediate

value, A = 1 if the offset is con-

tained in a data register.

OOO..	: Offset field. If the offset is an

immediate value, then the value 0-31

is stored here. If the offset is

contained in a data register, then

the register number D0-D7 is stored

here.

B	: B = 0 if the width is an immediate

value. B = 1 if the width is contained

in a data register.

WWW..	: Width Field. If the width is an immed-

iate value, then the values 0, 1-31 are

stored here, representing a width of

32, 1-31 respectively. If the width is

contained in a data register, then the

register number D0-D7 is stored here.

Addressing Modes : All modes EXCEPT:

An

(An)+

-(An)

#<data>

and PC relative modes, are allowed for the

destination operand.

BFEXTS	: Bit Field Extract Signed

Operation : src{off:wd} -> Dn

Assembler : BFEXTS <ea>{off:wd},Dn

Attributes: Unsized

Description	: Extract a bit field from the specified source

location, sign-extend to 32 bits, and store the

result in the specified data register.

Condition Codes	: X N Z V C

- * * 0 0

N Set if the most significant bit of the bit

field is set. Cleared otherwise.

Z Set if the bit field contains all zeros.

Cleared otherwise.

V Always Cleared.

C Always Cleared.

X Not Affected.

Instruction Format :

1 1 1 0 1 0 1 1 1 1 M M M R R R

0 D D D A O O O O O B W W W W W

Instruction Fields :

MMM, RRR: See MC68020 Instruction Extension Words

above.

DDD	: Destination Data Register field. Specifies

the destination data register D0-D7.

A	: A = 0 if the offset is an immediate

value, A = 1 if the offset is con-

tained in a data register.

OOO..	: Offset field. If the offset is an

immediate value, then the value 0-31

is stored here. If the offset is

contained in a data register, then

the register number D0-D7 is stored

here.

B	: B = 0 if the width is an immediate

value. B = 1 if the width is contained

in a data register.

WWW..	: Width Field. If the width is an immed-

iate value, then the values 0, 1-31 are

stored here, representing a width of

32, 1-31 respectively. If the width is

contained in a data register, then the

register number D0-D7 is stored here.

Addressing Modes : All modes EXCEPT:

An

(An)+

-(An)

#<data>

are allowed for the source operand.

BFEXTU	: Bit Field Extract Unsigned

Operation : src{off:wd} -> Dn

Assembler : BFEXTU <ea>{off:wd},Dn

Attributes: Unsized

Description	: Extract a bit field from the specified source

location, zero-extend to 32 bits, and store the

result in the specified data register.

Condition Codes	: X N Z V C

- * * 0 0

N Set if the most significant bit of the bit

field is set. Cleared otherwise.

Z Set if the bit field contains all zeros.

Cleared otherwise.

V Always Cleared.

C Always Cleared.

X Not Affected.

Instruction Format :

1 1 1 0 1 0 0 1 1 1 M M M R R R

0 D D D A O O O O O B W W W W W

Instruction Fields :

MMM, RRR: See MC68020 Instruction Extension Words

above.

DDD	: Destination Data Register field. Specifies

the destination data register D0-D7.

A	: A = 0 if the offset is an immediate

value, A = 1 if the offset is con-

tained in a data register.

OOO..	: Offset field. If the offset is an

immediate value, then the value 0-31

is stored here. If the offset is

contained in a data register, then

the register number D0-D7 is stored

here.

B	: B = 0 if the width is an immediate

value. B = 1 if the width is contained

in a data register.

WWW..	: Width Field. If the width is an immed-

iate value, then the values 0, 1-31 are

stored here, representing a width of

32, 1-31 respectively. If the width is

contained in a data register, then the

register number D0-D7 is stored here.

Addressing Modes : All modes EXCEPT:

An

(An)+

-(An)

#<data>

are allowed for the source operand.

BFFFO	: Bit Field Find First One

Operation : src{off:wd} bit scan -> Dn

Assembler : BFFFO <ea>{off:wd},Dn

Attributes: Unsized

Description	: Scan the specified bit field for the presence

of a '1' bit, starting at the most significant

bit of the bit field (the leftmost bit).

If a '1' bit is found, then the bit offset of

the first '1' bit located (which is the bit

offset of the source operand plus the relative

offset from the start of the bit field) is

stored in the specified data register.

If the bit field contains all zeros, then the

value stored in the specified data register is

the value of the source operand bit offset PLUS

the value of the source operand field width,

which is the offset to the next element of a

bit field array. The condition codes are set

according to the contents of the original bit

field.

Condition Codes	: X N Z V C

- * * 0 0

N Set if the most significant bit of the bit

field is 1. Cleared otherwise.

Z Set if all bits of the bit field contain

zero. Cleared otherwise.

V Always Cleared.

C Always Cleared.

X Not Affected.

Instruction Format :

1 1 1 0 1 1 0 1 1 1 M M M R R R

0 D D D A O O O O O B W W W W W

Instruction Fields :

MMM, RRR: See MC68020 Instruction Extension Words

above.

DDD	: Destination Data Register field. Specifies

the destination data register D0-D7.

A	: A = 0 if the offset is an immediate

value, A = 1 if the offset is con-

tained in a data register.

OOO..	: Offset field. If the offset is an

immediate value, then the value 0-31

is stored here. If the offset is

contained in a data register, then

the register number D0-D7 is stored

here.

B	: B = 0 if the width is an immediate

value. B = 1 if the width is contained

in a data register.

WWW..	: Width Field. If the width is an immed-

iate value, then the values 0, 1-31 are

stored here, representing a width of

32, 1-31 respectively. If the width is

contained in a data register, then the

register number D0-D7 is stored here.

Addressing Modes : All modes EXCEPT:

An

(An)+

-(An)

#<data>

are allowed for the source operand.

BFINS	: Bit Field Insert

Operation : Dn -> dst{off:wd}

Assembler : BFINS Dn,<ea>{off:wd}

Attributes: Unsized

Description	: Move a bit field from the low-order bits of the

specified data register and store the bits in

the bit field specified by the destination lo-

cation. The condition codes are set according to

the FINAL STATE of the bit field after insertion.

Condition Codes	: X N Z V C

- * * 0 0

N Set if the most significant bit of the resul-

tant bit field is set. Cleared otherwise.

Z Set if the resultant bit field contains all

zeros. Cleared otherwise.

V Always Cleared.

C Always Cleared.

X Not Affected.

Instruction Format :

1 1 1 0 1 1 1 1 1 1 M M M R R R

0 D D D A O O O O O B W W W W W

Instruction Fields :

MMM, RRR: See MC68020 Instruction Extension Words

above.

DDD	: Destination Data Register field. Specifies

the destination data register D0-D7.

A	: A = 0 if the offset is an immediate

value, A = 1 if the offset is con-

tained in a data register.

OOO..	: Offset field. If the offset is an

immediate value, then the value 0-31

is stored here. If the offset is

contained in a data register, then

the register number D0-D7 is stored

here.

B	: B = 0 if the width is an immediate

value. B = 1 if the width is contained

in a data register.

WWW..	: Width Field. If the width is an immed-

iate value, then the values 0, 1-31 are

stored here, representing a width of

32, 1-31 respectively. If the width is

contained in a data register, then the

register number D0-D7 is stored here.

Addressing Modes : All modes EXCEPT:

An

(An)+

-(An)

#<data>

and PC relative modes, are allowed for the

destination operand.

BFSET	: Test Bit Field And Set

Operation : dst{off:wd} -> CCR

1 -> dst{off:wd}

Assembler : BFSET <ea>{off:wd}

Attributes: Unsized.

Description	: Test the state of the specified bit field and

alter the condition codes accordingly. Once

the condition codes have been duly altered,

set all of the bits of the specified bit field

(to 1).

Condition Codes : X N Z V C

- * * 0 0

N : Set if the most significant bit of the

bit field is zero. Cleared otherwise.

Z : Set if all the bits of the bit field are

zero. Cleared otherwise.

V : Always Cleared.

C : Always Cleared.

X : Not Affected.

Instruction Format :

1 1 1 0 1 1 1 0 1 1 M M M R R R

0 0 0 0 A O O O O O B W W W W W

Instruction Fields :

MMM, RRR: See MC68020 Instruction Extension

Words Above.

A	: A = 0 if the offset is an immediate

value, A = 1 if the offset is con-

tained in a data register.

OOO..	: Offset field. If the offset is an

immediate value, then the value 0-31

is stored here. If the offset is

contained in a data register, then

the register number D0-D7 is stored

here.

B	: B = 0 if the width is an immediate

value. B = 1 if the width is contained

in a data register.

WWW..	: Width Field. If the width is an immed-

iate value, then the values 0, 1-31 are

stored here, representing a width of

32, 1-31 respectively. If the width is

contained in a data register, then the

register number D0-D7 is stored here.

Addressing Modes : All modes EXCEPT:

An

(An)+

-(An)

#<data>

and PC relative modes, are allowed for the

destination operand.

BFTST	: Test Bit Field

Operation : dst{off:wd} -> CCR

Assembler : BFTST <ea>{off:wd}

Attributes: Unsized.

Description	: Test the state of the specified bit field and

alter the condition codes accordingly.

Condition Codes : X N Z V C

- * * 0 0

N : Set if the most significant bit of the

bit field is zero. Cleared otherwise.

Z : Set if all the bits of the bit field are

zero. Cleared otherwise.

V : Always Cleared.

C : Always Cleared.

X : Not Affected.

Instruction Format :

1 1 1 0 1 0 0 0 1 1 M M M R R R

0 0 0 0 A O O O O O B W W W W W

Instruction Fields :

MMM, RRR: See MC68020 Instruction Extension

Words Above.

A	: A = 0 if the offset is an immediate

value, A = 1 if the offset is con-

tained in a data register.

OOO..	: Offset field. If the offset is an

immediate value, then the value 0-31

is stored here. If the offset is

contained in a data register, then

the register number D0-D7 is stored

here.

B	: B = 0 if the width is an immediate

value. B = 1 if the width is contained

in a data register.

WWW..	: Width Field. If the width is an immed-

iate value, then the values 0, 1-31 are

stored here, representing a width of

32, 1-31 respectively. If the width is

contained in a data register, then the

register number D0-D7 is stored here.

Addressing Modes : All modes EXCEPT:

An

(An)+

-(An)

#<data>

are allowed for the destination operand.

BKPT	: Breakpoint (MC68020)

Operation : IF Breakpoint Vector Acknowledged

THEN

execute returned operation word

ELSE

trap via illegal instruction vector

Assembler : BKPT #<data>

Attributes: Unsized

Description	: This instruction is used in conjunction

with external hardware support to provide

a breakpoint function for debuggers and

real-time emulators. As such, the opera-

tion will be implementation dependent.

Typically, this instruction will be sup-

ported by the MC68851 Paged Memory Manage-

Ment Unit, which contains the hardware re-

quired to replace the overwritten instruc-

tion word.

The MC68020 generates a Breakpoint Acknow-

edge bus cycle, with the immediate data on

address lines A2-A4, address lines A0 and A1

on the address bus being pulled low.

The normal response to a BKPT by supporting

hardware is to send an operation word (typi-

cally the original replaced instruction) on

the data lines with the DSACKx signal asser-

ted (this is an ACTIVE LOW signal). The op-

eration word is executed in place of the

breakpoint instruction.

For the exception response, a bus error sig-

nal will cause the MC68020 to take the ille-

gal instruction exception.

If external support for this instruction is

NOT provided in hardware, then the illegal

instruction exception is taken.

Condition Codes : NOT AFFECTED.

Instruction Format :

0 1 0 0 1 0 0 0 0 1 0 0 1 V V V

Instruction Fields :

VVV	: Vector field. Specified the breakpoint for which

the processor is to request the corresponding

operation word. Eight breakpoints from BKPT #0

to BKPT #7 are supported by the MC68851, and

the MC68851 can save overwritten instructions

and replace them upon execution of the BKPT

instruction. Other hardware support possibil-

ities exist, but the MC68851 PMMU is THE support

system recommended by Motorola.

BSR	: Branch To Subroutine (MC68020 onwards)

Operation : SP - 4 -> SP

PC -> (SP)

PC + d -> PC

Assembler : BSR <label>

Attributes: Size=Byte, Word, Long

Description	: The long word address of the instruction

immediately following the BSR instruction

is pushed onto the stack. The value of the

displacement is then added to the value of

the program counter, and program execution

continues at this new location.

For the 68020, the BSR instruction is ex-

tended to allow longword displacements to

be specified, allowing relative subroutine

calls to be performed across the entire

address space of the machine.

Condition Codes : NOT AFFECTED.

Instruction Format :

0 1 1 0 0 0 0 1 D D D D D D D D

Instruction Fields :

DDDD..	: Displacement field. The 68020 accepts THREE

displacement sizes. For short branches, this

field contains the byte-sized displacement

which is sign-extended to 32 bits prior to

addition to the program counter.

If this field contains zero (00000000) then

the word following the instruction word con-

tains a word-sized displacement, which is

sign-extended to 32 bits prior to adding to

the program counter. This and the previous

mode is available to all MC680x0 CPUs.

If this field contains $FF (11111111), then

the two words following the instruction word

contain a longword-sized displacement to be

added directly to the program counter. This

mode is available to the MC68020 onwards.

CALLM	: Call Module (MC68020 ONLY!)

Operation : Save current module state on stack.

Load new module state from destination.

Assembler : CALLM #<data>,<ea>

Attributes: Unsized.

Description	: The destination effective address is the

address of a module descriptor. A module

frame is created at the top of the stack,

and the current module state is saved in

the stack frame thus reserved.

A new module state is loaded from the de-

scriptor addressed by the destination op-

erand. Up to 255 bytes of arguments are

passed to the new module, the size of the

argument block being specified in the source

immediate operand.

Condition Codes : NOT AFFECTED.

Instruction Format:

0 0 0 0 0 1 1 0 1 1 M M M R R R

0 0 0 0 0 0 0 0 A A A A A A A A

Instruction Fields:

MMM, RRR: See MC68020 Instruction Extension

Words Above.

AAA...	: Argument Count field. Specifies the number

of bytes of arguments to be transferred to

the new module (as specified in the source

operand). Legal values are from 0 to 255

bytes. This same number of bytes is removed

from the stack by the RTM instruction (see

below).

Addressing Modes : All modes EXCEPT:

An

(An)+

-(An)

#<data>

are allowed for the destination operand.

CAS	: Compare And Swap With Operand

Operation : dst - Dc -> CCR

IF Z=1

THEN

Du -> dst

ELSE

dst -> Dc

Assembler : CAS Dc,Du,<ea>

Attributes: Size = Byte, Word, Long

Description	: This instruction performs a Compare And Swap

operation. The destination operand is com-

pared with the contents of the data register

containing the comparison operand (here, Dc)

and the condition codes set accrordingly. If

the Zero flag is set (indicating that the two

operands have equal values), then the contents

of the data register containing the update op-

erand (here, Du) is moved to the destination

location. If the Zero flag is cleared, then

the destination operand is left unchanged, and

its value moved to the data register used to

store the comparison operand.

This operation is performed using an indivis-

ible read-modify-write cycle to allow synchron-

isation of several processors. See notes below.

Condition Codes : X N Z V C

N : Set if the result is negative. Cleared

otherwise.

Z : Set if the result is zero. Cleared other-

wise.

V : Set if an overflow is generated. Cleared

otherwise.

C : Set if a carry is generated. Cleared

otherwise.

X : Not Affected.

Instruction Format :

0 0 0 0 1 Z Z 0 1 1 M M M R R R

0 0 0 0 0 0 0 U U U 0 0 0 C C C

Instruction Fields:

MMM, RRR: See MC68020 Instruction Extension

Words Above.

ZZ	: Size field. Specifies the size of the

operation:

01 = Byte operation.

10 = Word operation.

11 = Long operation.

UUU	: Specifies the data register to be used to

contain the update operand to be moved to

the destination effective address if the

comparison is successful.

CCC	: Specifies the data register to be used to

contain the comparison operand to compare

with the destination operand.

Addressing Modes : All modes EXCEPT:

Dn

An

#<data>

and PC relative modes, are allowed for the

destination operand.

Notes : This instruction may be used to manage a linked list

safely in a multiprocessor environment, where the

linked list is used to manage a resource shared be-

tween several processors and their tasks.

CAS2	: Compare And Swap Two Operands Simultaneously

Operation : dst1 - Dc1 -> CCR

IF Z=1

THEN

dst2 - Dc2 -> CCR

IF Z=1

THEN

Du1 -> dst1

Du2 -> dst2

ELSE

dst1 -> Dc1

dst2 -> Dc2

ELSE

dst1 -> Dc1

dst2 -> Dc2

Assembler : CAS Dc1:Dc2,Du1:Du2,(Rn1):(Rn2)

Attributes: Size = Word, Long

Description	: This operation first compares the contents of

the first compare operand and the first dest-

ination operand, and sets the condition codes.

If the Zero flag is set after this first com-

parison, a second comparison is made, between

the second comparison operand and the second

destination operand, and the condition codes

set accordingly.

If the Zero flag is set after the second com-

parison, then the contents of the two update

operands are moved to the respective destin-

ation operands.

If either comparison fails, and the Zero flag

is cleared, then the destination operands are

left unmodified, and the values of the destin-

ation operands are copied to the respective

comparison operands.

NOTE : Due to the complexity of this instruction,

the programmer is free to specify data registers

in the destination operands, and use them as if

they were address registers. This is the ONLY

instruction where such freedom is allowed! For

example, it is legal to use:

CAS2 D0:D1,D2:D3,(D4):(D5)

and use D4 and D5 as though they were address

registers using the (An) addressing mode.

This operation is performed using an indivis-

ible read-modify-write cycle to allow synchron-

isation of several processors. See notes below.

Condition Codes : X N Z V C

N : Set if the result is negative. Cleared

otherwise.

Z : Set if the result is zero. Cleared other-

wise.

V : Set if an overflow is generated. Cleared

otherwise.

C : Set if a carry is generated. Cleared

otherwise.

X : Not Affected.

Instruction Format :

0 0 0 0 1 Z Z 0 1 1 1 1 1 1 0 0

A R R R 0 0 0 U U U 0 0 0 C C C

B S S S 0 0 0 V V V 0 0 0 D D D

Instruction Fields:

MMM, RRR: See MC68020 Instruction Extension

Words Above.

ZZ	: Size field. Specifies the size of the

operation:

01 = Byte operation.

10 = Word operation.

11 = Long operation.

A	: D/A1 field. A = 0 if the RRR field specifies

a data register for the Rn1 operand, A = 1 if

the RRR field specifies an address register.

RRR	: Register number for the Rn1 operand above.

UUU	: Specifies which data register is to be used

for the first update operand (Du1 above).

CCC	: Specifies which data register is to be used

for the first comparison operand (Dc1 above).

B	: D/A2 field. B = 0 if the SSS field specifies

a data register for the Rn2 operand, B = 1 if

the SSS field specifies an address register.

SSS	: Register number for the Rn2 operand above.

VVV	: Specifies which data register is to be used

for the second update operand (Du2 above).

DDD	: Specifies which data register is to be used

for the second comparison operand (Dc2 above).

Addressing Modes : All modes EXCEPT:

Dn

An

(An)+

-(An)

#<data>

and PC relative modes, are allowed for the

destination operand.

Notes : This instruction may be used to safely update a doubly-

linked list in a multiprocessor environment where that

list is used to manage a resource shared across several

processors and their tasks.

CHK	: Check Register Against Bounds (MC68020)

Operation : IF Dn < 0 OR Dn > <ea>

THEN

TRAP (CHK exception vector)

ELSE

continue normal execution

Assembler : CHK <ea>,Dn

Attributes: Size = Word, Long.

Description	: The content of the data register is examined

and compared to the upper bound stored at the

source effective address. The upper bound is

a twos complement integer.

If the register value is less than zero, or

greater than the upper bound, then the pro-

cessor initiates exception processing via the

CHK exception vector.

The 68000 version of this instruction only

allows word-sized comparisons. The instruction

in this case is identical across both proces-

sors (see ZZZ field below).

Condition Codes : X N Z V C

- * U U U

N : Set if Dn<0, cleared if Dn> <ea>.

Undefined otherwise.

Z : Undefined.

V : Undefined.

C : Undefined.

X : Not Affected.

Instruction Format :

0 1 0 0 D D D Z Z Z R R R M M M

Instruction Fields :

MMM, RRR: See MC68020 Instruction Extension

Words Above.

ZZZ	: Size field. The two legal values

are:

110 : Word operation.

100 : Long operation.

DDD	: Data Register field. Specifies the data

register to use for the destination op-

erand.

Addressing Modes : All modes EXECPT An (address register

direct) are allowed.

CHK2	: Check Register Against Bounds Pair And TRAP

Operation : IF Dn < <ea lower bound>

OR

Dn > <ea upper bound>

THEN

TRAP (CHK exception vector)

ELSE

continue normal execution

Assembler : CHK2 <ea>,Rn

Attributes: Size = Byte, Word, Long.

Description	: The contents of the specified register (here,

either data or address register) are compared

with the PAIR of bounds specifiers stored at

the source effective address in the order:

<ea>	: Lower Bound

<ea+N>	: Upper Bound

where N is the size of the operand.

If the value of the specified register is less

than the value of the lower bound, or greater

than the value of the upper bound, then an ex-

ception is taken via the CHK exception vector.

If the value lies within the specified bounds,

then execution continues as normal.

If the destination register is a data register,

then the comparison only affects that portion of

the data register whose size is specified.

If the destination register is an address regi-

ster, then the bounds values are sign-extended

to 32 bits before the comparison is performed.

Note:either a signed or an unsigned comparison

can be performed. For signed comparisons, the

lower bound should be arithmetically smaller,

and for unsigned comparisons, the lower bound

should be logically smaller.

Condition Codes : X N Z V C

- U * U *

N : Undefined.

Z : Set if Rn is equal to either bound. Cleared

otherwise.

V : Undefined.

C : Set if Rn is out of bounds. Cleared other-

wise.

X : Not Affected.

Instruction Format :

0 0 0 0 0 Z Z 0 1 1 M M M R R R

A D D D 1 0 0 0 0 0 0 0 0 0 0 0

Instruction Fields :

MMM, RRR: See MC68020 Instruction Extension

Words Above.

ZZ	: Size field. The two legal values

are:

01 : Byte operation.

10 : Word operation.

11 : Long operation.

A	: D/A field. A = 0 if the register to be checked

is a data register, A = 1 if the register to be

checked is an address register.

DDD	: Register field. Specifies the data or address

register to use for the destination operand.

Addressing Modes : All modes EXECPT:

Dn

An

(An)+

-(An)

#<data>

are allowed for the source operand.

CMP2	: Check Register Against Bounds Pair

Operation : Compare Rn against upper & lower bounds -> CCR

Assembler : CHK2 <ea>,Rn

Attributes: Size = Byte, Word, Long.

Description	: The contents of the specified register (here,

either data or address register) are compared

with the PAIR of bounds specifiers stored at

the source effective address in the order:

<ea>	: Lower Bound

<ea+N>	: Upper Bound

where N is the size of the operand.

If the destination register is a data register,

then the comparison only affects that portion of

the data register whose size is specified.

If the destination register is an address regi-

ster, then the bounds values are sign-extended

to 32 bits before the comparison is performed.

Note:either a signed or an unsigned comparison

can be performed. For signed comparisons, the

lower bound should be arithmetically smaller,

and for unsigned comparisons, the lower bound

should be logically smaller.

This instruction is analogous to CHK2, but is

provided to allow comparisons without the need

for exception processing.

Condition Codes : X N Z V C

- U * U *

N : Undefined.

Z : Set if Rn is equal to either bound. Cleared

otherwise.

V : Undefined.

C : Set if Rn is out of bounds. Cleared other-

wise.

X : Not Affected.

Instruction Format :

0 0 0 0 0 Z Z 0 1 1 M M M R R R

A D D D 0 0 0 0 0 0 0 0 0 0 0 0

Instruction Fields :

MMM, RRR: See MC68020 Instruction Extension

Words Above.

ZZ	: Size field. The two legal values

are:

01 : Byte operation.

10 : Word operation.

11 : Long operation.

A	: D/A field. A = 0 if the register to be checked

is a data register, A = 1 if the register to be

checked is an address register.

DDD	: Register field. Specifies the data or address

register to use for the destination operand.

Addressing Modes : All modes EXECPT:

Dn

An

(An)+

-(An)

#<data>

are allowed for the source operand.

cpBcc	: Branch On Coprocessor Condition

Operation : If cpcc = TRUE

THEN

PC + d -> PC

ELSE

continue normal execution

Assembler : cpBcc <label>

Attributes: Size = Word, Long

Description	: If the specified coprocessor condition is

true, then the value of the displacement

is added onto the value of the program

counter, and execution continues at this

new location. If the specified coprocessor

condition is false, then execution contin-

ues at the next instruction in sequence.

If the branch is a word branch, then the

displacement is sign-extended to 32 bits

before being added to the PC.

Condition Codes : NOT AFFECTED.

Instruction Format :

1 1 1 1 I I I 0 1 W C C C C C C

X X X X X X X X X X X X X X X X

Instruction Fields :

III	: Coprocessor ID field. Specifies the

coprocessor that is to perform this

operation.

Current values:

000 = MC68851 Paged Memory Management Unit

001 = MC6881/68882 Mathematics Coprocessor

W	: W/L field. W = 0 specifies a word-length

branch, W = 1 specifies a longword-length

branch.

CCC...	: Coprocessor condition field. Contains the

condition specification for the given co-

processor. This field is passed to the

coprocessor, which provides directives to

the CPU for processing this instruction.

XXX...	: Optional Coprocessor Defined Extension Word(s)

Following these words are either one word for a 16-bit

branch displacement, or two words for a 32-bit displace-

ment branch.

cpDBcc	: Test Coprocessor Condition, Decrement And Branch

Operation : IF cpcc = FALSE

THEN

Dn - 1 -> Dn

IF Dn <> -1

THEN

PC + d -> PC

ELSE

continue normal execution

ELSE

continue normal execution

Assembler : cpDBcc Dn,<label>

Attributes: Size = Word.

Description	: If the specified coprocessor condition is true,

then execution continues with the next instruc-

tion. If the specified coprocessor condition is

false, then the low order word of the specified

data register is decremented by 1. If the result

of this decrementation is equal to -1, execution

continues with the next instruction. Otherwise

the 16-bit displacement is sign-extended to 32

bits and added on to the program counter, and ex-

ecution continues at this new location.

Condition Codes : NOT AFFECTED.

Instruction Format :

1 1 1 1 I I I 0 0 1 0 0 1 D D D

0 0 0 0 0 0 0 0 0 0 C C C C C C

X X X X X X X X X X X X X X X X

Instruction Fields :

III	: Coprocessor ID field. Specifies the coprocesor

that is to process this operation. See cpBcc

above for current coprocessor IDs.

DDD	: Data Register field. Specifies the data regis-

ter to be used as the counter.

CCC...	: Coprocessor condition field. Contains the

condition specification for the given co-

processor. This field is passed to the

coprocessor, which provides directives to

the CPU for processing this instruction.

XXX...	: Optional Coprocessor Defined Extension Word(s).

Following these words is a 16-bit word containing the

displacement to be used.

cpGEN	: Coprocessor General Function.

Operation : Pass Command Word To Coprocessor.

Assembler : cpGEN <parameters>

<parameters> as defined by given coprocessor

Attributes: Unsized.

Description	: This instruction is used by coprocessors to

specify the general data movement and data

processing operations for the specified co-

processor. The coprocessor determines the

operation from the Coprocessor Command Word

immediately following the cpGEN command word.

Usually, a coprocessor defines specific ins-

tances of this instruction to provide its in-

struction set.

Condition Codes : Determined By Coprocessor. Unchanged Otherwise.

Instruction Format :

1 1 1 1 I I I 0 0 0 M M M R R R

C C C C C C C C C C C C C C C C

X X X X X X X X X X X X X X X X

Instruction Fields :

III	: Coprocessor ID field. Specifies the coprocesor

that is to process this operation. See cpBcc

above for current coprocessor IDs.

MMM, RRR: See MC68020 Instruction Extension Words Above.

Used to specify an operand in main memory for

the coprocessor operation if specified, or

the CPU registers.

CCC...	: Coprocessor Command Field. Specifies the co-

processor instruction to be performed.

XXX...	: Optional Coprocessor Defined Extension Word(s).

cpRESTORE	: Restore Coprocessor Machine State

(PRIVIEGED INSTRUCTION)

Operation : IF supervisor state

THEN

Restore Coprocessor Machine State

ELSE

TRAP (Privilege Violation).

Assembler : cpRESTORE <ea>

Attributes: Unsized.

Description	: If the CPU is in supervisor mode, then restore

the machine state of the coprocessor as saved

earlier by a cpSAVE instruction (see below).

If the CPU is in user mode, take the Privilege

Violation exception.

Condition Codes : NOT AFFECTED.

Instruction Format :

1 1 1 1 I I I 1 0 1 M M M R R

Instruction Fields :

III	: Coprocessor ID field. Specifies the coprocesor

that is to process this operation. See cpBcc

above for current coprocessor IDs.

MMM, RRR: See MC68020 Instruction Extension Words Above.

Used to specify the effective address of the

saved coprocessor state to restore.

Addressing Modes : All EXECPT:

Dn

An

-(An)

#<data>

are allowed for the effective address operand.

Notes : If the coprocessor passes a communication format word

of the 'come again' form for continued processing,

pending interrupts are not serviced. It is the PRO-

GRAMMER'S responsibility to take account of this!

cpSAVE	: Save Coprocessor Machine State

(PRIVILEGED INSTRUCTION)

Operation : IF supervisor state

THEN

Save Coprocessor Machine State

ELSE

TRAP (Privilege Violation).

Assembler : cpSAVE <ea>

Attributes: Unsized.

Description	: If the CPU is in the supervisor mode, then

save the entire machine state of the speci-

fied coprocessor at the destination effec-

tive address. If the CPU is in user mode,

then take the Privilege Violation exception.

Condition Codes : NOT AFFECTED.

Instruction Format :

1 1 1 1 I I I 1 0 0 M M M R R R

Instruction Fields :

III	: Coprocessor ID field. Specifies the coprocesor

that is to process this operation. See cpBcc

above for current coprocessor IDs.

MMM, RRR: See MC68020 Instruction Extension Words Above.

Used to specify the effective address of the

saved coprocessor state to restore.

Addressing Modes : All EXECPT:

Dn

An

(An)+

#<data>

are allowed for the effective address operand.

cpScc	: Set On Coprocessor Condition

Operation : IF cpcc = TRUE

THEN

1111...111 -> dst

ELSE

0000...000 -> dst

Assembler : cpScc <ea>

Attributes: Size = Byte.

Description	: Test the specified coprocessor condition. If

the condition is TRUE, set all bits of the

byte destination operand. If the condition

is FALSE, clear all bits of the byte desti-

nation operand.

Condition Codes : NOT FFFECTED.

Instruction Format :

1 1 1 1 I I I 0 0 1 M M M R R R

0 0 0 0 0 0 0 0 0 0 C C C C C C

X X X X X X X X X X X X X X X X

Instruction Fields :

III	: Coprocessor ID field. Specifies the coprocesor

that is to process this operation. See cpBcc

above for current coprocessor IDs.

MMM, RRR: See MC68020 Instruction Extension Words Above.

Used to specify an operand in main memory for

the coprocessor operation if specified, or

the CPU registers.

CCC...	: Coprocessor Condition Field. Specifies the co-

processor condition to be tested.

XXX...	: Optional Coprocessor Defined Extension Word(s).

Addressing Modes : All EXECPT:

An

#<data>

are allowed for the effective address operand.

cpTRAPcc	: Trap On Coprocessor Condition.

Operation : IF cpcc = TRUE

THEN

TRAP (cpTRAP exception vector)

ELSE

continue normal execution.

Assembler : cpTRAPcc #<data>

Attributes: Unsized or Size = Word.

Description	: If the selected coprocessor condition is true,

then the CPU initiates exception processing.

The vector number is generated to reference the

TRAPV/TRAPcc exception vector, and the stacked

value of the program counter is the address of

the next instruction. If the selected condition

is false, then execution continues with the next

instruction. Following the condition word and

any coprocessor defined extension words is a

user-defined data operand specified as immediate

data, to be used by the trap handler.

Condition Codes : NOT AFFECTED.

Instuction Format :

1 1 1 1 I I I 0 0 1 1 1 1 M M M

0 0 0 0 0 0 0 0 0 0 C C C C C C

X X X X X X X X X X X X X X X X

Instruction Fields :

III	: Coprocessor ID field. Specifies the coprocesor

that is to process this operation. See cpBcc

above for current coprocessor IDs.

MMM	: Op-Mode field. Specifies the instruction form.

010 : Instruction is followed by one operand

word.

011 : Instruction is followed by two operand

words.

100 : Instruction has no operand words.

CCC...	: Coprocessor Condition field. Specifies the co-

processor condition to be tested.

XXX...	: Optional Coprocessor Defined Extension Word(s).

If the Op-Mode field specifies one or two immediate data

operand words, then the operand words follow the optional

coprocessor defined extension words.

DIVSL	: Divide Signed Long.

Operation : dst/src -> dst

Assembler : DIVS.L <ea>,Dq	;32/32 -> 32r

DIVS.L <ea>,Dr:Dq	;64/32 -> 32r:32q

DIVSL.L <ea>,Dr:Dq	;32/32 -> 32r:32q

Attributes: Size = Word, Long.

Description	: Divide the destination operand by the source

operand, and store the result in the destina-

tion. The operation is performed using signed

arithmetic.

The first long form operates on a 32-bit divi-

dend (the destination) and a 32-bit divisor

(the source). The result is a 32-bit quotient

stored in the destination, and the remainder

is discarded.

The second long form operates on a 64-bit divi-

dend (the destination) and a 32-bit divisor (the

source). The result is a 32-bit quotient and a

32-bit remainder.

The third long form operates on a 32-bit divi-

dend (the destination) and a 32-bit divisor (the

source). The result is a 32-bit quotient and a

32-bit remainder.

Two special conditions may arise:

1) Division by zero : take the Zero Divide

exception.

2) Overflow detected & set before completion

of the instruction. If this occurs, then

the destination operand(s) are not affected.

Condition Codes : X N Z V C

- * * * 0

N : Set if the quotient is negative. Undefined if

overflow or divide by zero.

Z : Set if the quotient is zero. Cleared otherwise.

Undefined if overflow or divide by zero.

V : Set if division overflow is detected. Cleared

otherwise.

C : Always Cleared.

X : Not Affected.

Instruction Format :

0 1 0 0 1 1 0 0 0 1 M M M R R R

0 Q Q Q 1 Z 0 0 0 0 0 0 0 D D D

Instruction Fields :

MMM, RRR: See MC68020 Instruction Extension Words Above.

QQQ	: Specifies the data register used to hold the

quotient.

Z	: Size field. Selects a 32-bit or 64-bit dividend.

0 = 32-bit dividend in register Dq.

1 = 64-bit dividend in register Dr:Dq.

DDD	: Specifies the data register used to hold the

remainder. If Dr = Dq, then only the quotient

is returned (1st long form). If the size

field is 1, this specifies the data register

containing the high 32 bits of the 64-bit divi-

dend.

Addressing Modes : All EXCEPT An are allowed for the source

operand.

DIVUL	: Divide Unsigned Long.

Operation : dst/src -> dst

Assembler : DIVU.L <ea>,Dq	;32/32 -> 32r

DIVU.L <ea>,Dr:Dq	;64/32 -> 32r:32q

DIVUL.L <ea>,Dr:Dq	;32/32 -> 32r:32q

Attributes: Size = Word, Long.

Description	: Divide the destination operand by the source

operand, and store the result in the destina-

tion. The operation is performed using unsigned

arithmetic.

The first long form operates on a 32-bit divi-

dend (the destination) and a 32-bit divisor

(the source). The result is a 32-bit quotient

stored in the destination, and the remainder

is discarded.

The second long form operates on a 64-bit divi-

dend (the destination) and a 32-bit divisor (the

source). The result is a 32-bit quotient and a

32-bit remainder.

The third long form operates on a 32-bit divi-

dend (the destination) and a 32-bit divisor (the

source). The result is a 32-bit quotient and a

32-bit remainder.

Two special conditions may arise:

1) Division by zero : take the Zero Divide

exception.

2) Overflow detected & set before completion

of the instruction. If this occurs, then

the destination operand(s) are not affected.

Condition Codes : X N Z V C

- * * * 0

N : Set if the quotient is negative. Undefined if

overflow or divide by zero.

Z : Set if the quotient is zero. Cleared otherwise.

Undefined if overflow or divide by zero.

V : Set if division overflow is detected. Cleared

otherwise.

C : Always Cleared.

X : Not Affected.

Instruction Format :

0 1 0 0 1 1 0 0 0 1 M M M R R R

0 Q Q Q 0 Z 0 0 0 0 0 0 0 D D D

Instruction Fields :

MMM, RRR: See MC68020 Instruction Extension Words Above.

QQQ	: Specifies the data register used to hold the

quotient.

Z	: Size field. Selects a 32-bit or 64-bit dividend.

0 = 32-bit dividend in register Dq.

1 = 64-bit dividend in register Dr:Dq.

DDD	: Specifies the data register used to hold the

remainder. If Dr = Dq, then only the quotient

is returned (1st long form). If the size

field is 1, this specifies the data register

containing the high 32 bits of the 64-bit divi-

dend.

Addressing Modes : All EXCEPT An are allowed for the source

operand.

EXT	: Sign Extend (MC68020).

Operation : dst (sign-extend) -> dst

Assembler : EXT.W Dn	;extend byte to word

EXT.L Dn	;extend word to long

EXTB.L Dn	;extend byte to long

Attributes: Size = Byte, Word, Long.

Description	: Sign-extend the operand contained in the desti-

nation data register, according to the specified

size.

EXT.W copies bit 7 to bits [15:8] of the data

register. This instruction is available on all

MC680x0 CPUs.

EXT.L copies bit 15 to bits [31:16] of the data

register. This instruction is avaialble on all

MC680x0 CPUs.

EXTB.L copies bit 7 to bits [31:8] of the data

register. This instruction is avaialble on the

MC68020 onwards.

The instruction formats for EXT.W and EXT.L are

identical to the MC68000 EXT instruction. The

Op-Mode field provides for extensions to the

instruction set for higher processors such as

the MC68020.

Condition Codes : X N Z V C

- * * 0 0

N : Set if the result is negative. Cleared

otherwise.

Z : Set if the result is zero. Cleared

otherwise.

V : Always Cleared.

C : Always Cleared.

X : Not Affected.

Instruction Format :

0 1 0 0 1 0 0 M M M 0 0 0 D D D

Instruction Fields :

MMM	: Op-Mode field. Specifies the size of the

sign-extension operation.

010 : sign-extend byte to word.

011 : sign-extend word to long.

111 : sign-extend byte to long.

DDD	: Register field. Specifies which data register

to use.

MOVEC	: Move Control Register

(PRIVILEGED INSTRUCTION)

Operation : IF supervisor state

THEN

Rc -> Rn or Rn -> Rc

ELSE

TRAP (Privilege Violation).

Assembler : MOVEC Rc,Rn

MOVEC Rn,Rc

Attributes: Size = Long.

Description	: If the CPU is in supervisor mode, then copy the

contents of the specified control register to

the specified general register, or copy the con-

tents of the specified general register to the

specified control register. If the CPU is in

user mode, then take the Priivlege Violation ex-

ception.

The data transfer is ALWAYS a 32-bit transfer,

even though the register may be implemented with

fewer bits. Unimplemented bits are read as zeros

and ignored when written.

Condition Codes : NOT AFFECTED.

Instuction Format :

0 1 0 0 1 1 1 0 0 1 1 1 1 0 1 D

A R R R C C C C C C C C C C C C

Instruction Fields :

D	: Dr field. Specifies the direction of data

transfer.

0 = control register to general register.

1 = general register to control register.

A	: A/D field. Specifies the type of general

register.

0 = Data Register.

1 = Address Register.

RRR	: General Register field. Specifies which data

or address register is used.

CCC...	: Control Register field. Specifies the control

register used. Values are:

$000	: Source Function Code (SFC) Register.

$001	: Destination Function Code (DFC)

Register.

$002	: Cache Control Register (CACR).

$800	: User Stack Pointer (USP).

$801	: Vector Base Register (VBR).

$802	: Cache Address Register (CAAR).

$803	: Master Stack Pointer (MSP).

$804	: Interrupt Stack Pointer (ISP).

All other codes cause an illegal instruction

exception.

MOVES	: Move Address Space

(PRIVILEGED INSTRUCTION)

Operation : IF supervisor state

THEN

Rn -> dst[DFC]

or

src[SFC] -> Rn

ELSE

TRAP (Privilege Violation).

Assembler : MOVES <ea>,Rn

MOVES Rn,<ea>

Attributes: Size = Byte, Word, Long.

Description	: Move the operand, using the specified size, from

the specified general register to a location wit-

hin the address space defined by the destination

function code (DFC) register. Or, move the oper-

and, using the specified size, from the location

within the address space defined by the source

function code (SFC) register to the specified

general register.

This transfer is ONLY performed if the CPU is in

supervisor mode. If the CPU is in user mode, then

the Privilege Violation exception is taken.

For suitable values of the SFC and DFC registers,

see MC68020 Extra Registers above.

Condition Codes : NOT AFFECTED.

Instruction Format :

0 0 0 0 1 1 1 0 Z Z M M M R R R

A D D D X 0 0 0 0 0 0 0 0 0 0 0

Instruction Fields :

ZZ	: Size field. Specifies the size of the operation.

00 = Byte Operation.

01 = Word Operation.

10 = Long Operation.

MMM, RRR: See MC68020 Instruction Extension Words Above.

A	: A/D field. Specifies whether the general register

is a data or address register. 0 = data register,

1 = address register.

DDD	: Specifies which general register D0-D7 or A0-A7

is used.

X	: Direction field. 0 = from <ea> to general regis-

ter, 1 = general register to <ea>.

Addressing Modes : All EXCEPT Dn, An and #<data> are allowed for

the address operand in each case.

Notes : MOVES.x An,(An)+ or MOVES.x An,-(An) where the same address

register is used for both source and destination operands,

is an undefined operation. The value stored in memory is

undefined. On the MC68010 and MC68020 implementations, the

value stored is the increment or decrement value of An. This

implementation MAY NOT APPEAR on future devices.

MULS	: Multiply Signed (MC68020 Extended Version)

Operation : dst * src -> dst

Assembler : MULS.L <ea>,Dl	;32x32 -> 32

MULS.L <ea>,Dh:Dl	;32x32 -> 64

Attributes: Size = Long.

Description	: Multiply two operands using signed arithmetic,

and store the result in the specified destina-

tion.

In the first form, two 32-bit numbers are multi-

plied together, and a 32-bit result is stored in

the specified destination.

In the second form, two 32-bit numbers are multi-

plied together, these numbers being stored at the

source effective address and in the register des-

ignated Dl above. The result is stored as a 64-

bit value in Dh:Dl, Dh containing the high 32

bits, and Dl the low 32 bits of the result.

Condition Codes : X N Z V C

- * * * 0

N : Set if the result is negative. Cleared

otherwise.

Z : Set if the result is zero. Cleared other-

wise.

V : Set if overflow detected. Cleared other-

wise.

C : Always Cleared.

X : Not Affected.

NOTE : Overflow (V=1) can ONLY occur with the

first form of the instruction. This occurs when

the high 32 bits of the result are NOT EQUAL to

the sign-extension of the low 32 bits.

Instruction Format :

0 1 0 0 1 1 0 0 0 0 M M M R R R

0 L L L 1 Z 0 0 0 0 0 0 0 H H H

Instruction Fields :

MMM, RRR: See MC68020 Instruction Extension Words Above.

Define the addressing mode of the source oper-

and.

LLL	: Register Dl field. Specifies which data register

is to be used as the Dl register in the above

specification.

Z	: Size field. A 0 value indicates a 32-bit product,

a 1 value indicates a 64-bit product.

HHH	: Register Dh field. Specifies which data register

is to be used as the Dh register in the above

specification. If Dh = Dl and Sz =1, then the

result of the operation are undefined. If Sz=0,

this field is unused (generally set to 0).

MULU: Multiply Unsigned (MC68020 Extended Version)

Operation : dst * src -> dst

Assembler : MULU.L <ea>,Dl	;32x32 -> 32

MULU.L <ea>,Dh:Dl	;32x32 -> 64

Attributes: Size = Long.

Description	: Multiply two operands using unsigned arithmetic,

and store the result in the specified destina-

tion.

In the first form, two 32-bit numbers are multi-

plied together, and a 32-bit result is stored in

the specified destination.

In the second form, two 32-bit numbers are multi-

plied together, these numbers being stored at the

source effective address and in the register des-

ignated Dl above. The result is stored as a 64-

bit value in Dh:Dl, Dh containing the high 32

bits, and Dl the low 32 bits of the result.

Condition Codes : X N Z V C

- * * * 0

N : Set if the result is negative. Cleared

otherwise.

Z : Set if the result is zero. Cleared other-

wise.

V : Set if overflow detected. Cleared other-

wise.

C : Always Cleared.

X : Not Affected.

NOTE : Overflow (V=1) can ONLY occur with the

first form of the instruction. This occurs when

the high 32 bits of the result are NOT EQUAL to

the sign-extension of the low 32 bits.

Instruction Format :

0 1 0 0 1 1 0 0 0 0 M M M R R R

0 L L L 0 Z 0 0 0 0 0 0 0 H H H

Instruction Fields :

MMM, RRR: See MC68020 Instruction Extension Words Above.

Define the addressing mode of the source oper-

and.

LLL	: Register Dl field. Specifies which data register

is to be used as the Dl register in the above

specification.

Z	: Size field. A 0 value indicates a 32-bit product,

a 1 value indicates a 64-bit product.

HHH	: Register Dh field. Specifies which data register

is to be used as the Dh register in the above

specification. If Dh = Dl and Sz =1, then the

result of the operation are undefined. If Sz=0,

this field is unused (generally set to 0).

PACK	: Pack data into a single byte.

Operation : src (unpacked BCD) + adjustment -> dst (packed BCD)

Assembler : PACK -(Ax),-(Ay),#<adjustment>

PACK Dx,Dy,#<adjustment>

Attributes: Unsized.

Description	: The low four bits of two separate bytes are

adjusted and packed into a single byte.

When the source and destination operands are

data registers, the operation is performed as

follows:

src : xxxxabcdyyyyefgh +

adj : ....****....####

The highlighted bits (marked '*' and '#') are

then transferred to the destination as follows:

dst : ****####

When the source and destination operands are

predecrement operands, two bytes are fetched

from the source, concatenated to form a 16-

bit operand, and the adjustment added as above.

Then, the bits are transferred to the destina-

tion byte as above and the data pointers are

adjusted accordingly.

Condition Codes : NOT AFFECTED.

Instruction Format :

1 0 0 0 Y Y Y 1 0 1 0 0 M X X X

A A A A A A A A A A A A A A A A

Instruction Fields :

YYY	: Register Dy/Ay field:specifies the destina-

tion register used.

XXX	: Register Dx/Ax field:specifies the source

register used.

M	: Register/Memory Mode (R/M) field. Specifies

whether the addressing mode is data register

direct or predecrement.

0 : Instruction is PACK Dx,Dy

1 : Instruction is PACK -(Ax),-(Ay)

AAA...	: Adjustment field. Contains the immediate data to

be added to the source operand. Appropriate con-

stants can be used to translate ASCII or EBCDIC

strings to packed BCD.

Example : PACK D1,D0,#0

Before : D0 = $0000,

D1 = $3231 (= ASCII string "21").

Split word into bytes = $32, $31

Zero out upper nibbles = $02, $01

Concatenate these bytes to a word = $0201

Add adjustment : $0201 + $0000 = $0201

Now pack nibbles $2, $1 into a byte = $21

After  : D0 = $0021.

RTD	: Return And Deallocate Parameters

Operation : (SP) -> PC

SP + d -> SP

Assembler : RTD #<displacement>

Attributes: Unsized.

Description	: The program counter is pulled from the stack.

The previous value of the program counter is

lost. Then, the 16-bit displacement value is

sign-extended to 32 bits and added to the

stack pointer.

Condition Codes : NOT AFFECTED.

Instruction Format :

0 1 0 0 1 1 1 0 0 1 1 1 0 1 0 0

D D D D D D D D D D D D D D D D

Instruction Fields :

DDD...	: Displacement field. Specifies the 16-bit

displacement to sign-extend and add to the

stack pointer after pulling the program

counter from the stack.

RTE	: Return From Exception (MC68020)

(PRIVILEGED INSTRUCTION).

Operation : IF supervisor state

THEN

(SP) -> SR

SP + 2 -> SP

(SP) -> PC

SP + 4 -> SP

(SP) -> Internal Format Word register

Interpret format word and pull stack

frame off stack, restore state

ELSE

TRAP (Privilege Violation)

Assembler : RTE

Attributes: Unsized.

Description	: If the CPU is in the supervisor state, then

pull the status register from the stack,

followed by the program counter. Then pull

the stack frame format word from the stack,

and examine the format field to determine

if any extra words are to be pulled from

the stack, and how many. If extra words are

to be pulled from the stack, pull them and

restore to internal registers if needed.

If the CPU is in the user state, then the

CPU takes the Privilege Violation excep-

tion.

Condition Codes : ALL AFFECTED ACCORDING TO THE WORD PULLED FROM

THE STACK.

Instruction Format :

0 1 0 0 1 1 1 0 0 1 1 1 0 0 1 1

Stack Frame Format Word Format (pulled off the stack by the

RTE instruction) :

F F F F 0 0 V V V V V V V V V V

Stack Frame Format Word Fields :

FFFF	: Format field. Specifies the format of the

stack frame to be pulled from the stack.

Values interpreted are:

0000	: Short Format. Only four words are

removed from the stack (SR, PC and

this format word).

0001	: Throwaway Format. Four words are

removed from the currently active

stack, and SR only is updated from

this stack frame. Then, the CPU

begins executing the RTE from the

top of the active stack AFTER pul-

ling SR. This format is used to

mark the bottom of the Interrupt

Stack, and when the CPU is using

the Master Stack for exception

processing.

0010	: Instruction Error format. Six words

are pulled from the top of the stack.

The first four words are as for the

short format:the remaining two words

are discarded.

1000	: MC68010 Long Format. The MC68020 takes

a Format Error exception.

1001	: Coprocessor Mid-Instruction Format. 10

words are removed from the stack. Re-

sumes coprocessor instruction execution.

1010	: MC68020 Short Format. 16 words are re-

moved from the stack. Resumes instruc-

tion execution.

1011	: MC68020 Long Format. 46 words are re-

moved from the stack. Resumes instruc-

tion execution.

Any other values:the CPU takes a Format Error

exception.

VVV...	: Vector Offset field. This field contains the

exception number that was generated.

RTM	: Return From Module

Operation : Reload saved module state from stack.

Assembler : RTM Rn

Attributes: Unsized.

Description	: A previously saved module state (as saved by

the CALLM instruction) is reloaded from the

top of the stack. After the module state is

retrieved from the top of the stack, the

caller's stack pointer is incremented by the

argument count value in the module state.

Like CALLM above, this instruction requires

external hardware support, usually from the

MC68851 Paged Memory Management Unit (PMMU)

but also from other sources.

Condition Codes : ALL AFFECTED ACCORDING TO THE CONTENT OF THE

WORD ON THE STACK.

Instruction Format :

0 0 0 0 0 1 1 0 1 1 0 0 A R R R

Instruction Fields :

A	: D/A field. Specifies whether the module data

pointer is a data or an address register.

0 = Data Register.

1 = Address Register.

RRR	: Register Field. Specifies the register number

for the module data pointer which is to be

restored from the saved module state. If the

register specified is A7 (SP), the updated

value of the register reflects the stack poin-

ter operations, and the saved module data area

pointer is lost.

TRAP	: TRAP Instruction (MC68020)

Operation : SSP - 2 -> SSP

Format/Offset -> (SSP)

SSP - 4 -> SSP

PC -> (SSP)

SSP - 2 -> SSP

SR -> (SSP)

TRAP Vector Address -> PC

Assembler : TRAP #<vector>

Attributes: Unsized.

Description	: The processor initiates exception processing.

The vector number is generated to reference

the TRAP instruction vector specified by the

low order four bits of the instruction.

Sixteen TRAP instruction vectors (0-15) are

available.

Condition Codes : NOT AFFECTED.

Instruction Format :

0 1 0 0 1 1 1 0 0 1 0 0 V V V V

Instruction Fields :

VVVV	: Vector Field. Specifies which TRAP vector

contains the new value of the program coun-

ter to be loaded.

TRAPcc	: Trap On Condition

Operation : IF condition = TRUE

THEN

TRAP (TRAPV/TRAPcc Vector)

ELSE

continue normal execution

Assembler : TRAPcc

TRAPcc.W #<data>

TRAPcc.L #<data)

Attributes: Unsized or Size = Word, Long.

Description	: If the condition specified in the instruction is

true, then the CPU initiates exception processing.

The vector number is generated to reference the

TRAPcc exception vector. The stacked program val-

ue points to the next instruction after the TRAPcc.

If the condition specified is false, then no oper-

ation occurs, and execution continues at the ins-

truction immediately following the TRAPcc.

If an immediate data operand is specified, then

the word (or words) containing that data follow

the TRAPcc instruction immedaitely, and this data

is available to the programmer as a user-definable

data field for use within the trap handler. For a

full list of CPU conditions see typed_68000.doc.

Condition Codes : NOT AFFECTED.

Instruction Format :

0 1 0 1 C C C C 1 1 1 1 1 M M M

W W W W W W W W W W W W W W W W

L L L L L L L L L L L L L L L L

Instruction Fields :

CCCC	: Condition Code field. Selects one of the 16

possible conditions for the TRAPcc instruc-

tion. For a full list of conditions see the

companion file typed_68000.doc.

MMM	: Op-Mode field. Selects the instruction form.

010 : Instruction is followed by one operand

word.

011 : Instruction is followed by two operand

words.

100 : Instruction has no following operand

words.

UNPK	: Unpack BCD

Operation : src (Packed BCD) + adjustment -> dst (Unpacked BCD)

Assembler : UNPK -(Ax),-(Ay),#<adjustment>

UNPK Dx,Dy,#<adjustment>

Attributes: Unsized.

Description	: In the unpack operation, the byte operand is

split into its two component nibbles, and each

nibble is then zero-extended to a byte value.

These new byte values are concatenated into a

single 16-bit value. Then, the value of the ad-

justment is added to this 16-bit value, and the

two resulting bytes moved to the destination.

If the destination is a data register, then the

result is moved as a single 16-bit word to the

data register.

Condition Codes : NOT AFFECTED.

Instruction Format :

1 0 0 0 Y Y Y 1 1 0 0 0 M X X X

A A A A A A A A A A A A A A A A

Instruction Fields :

YYY	: Register Dy/Ay field:specifies the destina-

tion register used.

XXX	: Register Dx/Ax field:specifies the source

register used.

M	: Register/Memory Mode (R/M) field. Specifies

whether the addressing mode is data register

direct or predecrement.

0 : Instruction is UNPK Dx,Dy

1 : Instruction is UNPK -(Ax),-(Ay)

AAA...	: Adjustment field. Contains the immediate data to

be added to the source operand. Appropriate con-

stants can be used to translate packed BCD to

ASCII or EBCDIC strings.

Example : UNPK D1,D0,#$3030

Before : D1 = $0021 (BCD 21)

Unpack nibbles into bytes = $02, 01

Concatenate bytes to word = $0201

$0201 + $3030 = $3231

After  : D0 = $3132 (ASCII "21")