1.                Hard Disk Preparation program 4.00

KFH Assocaites, (c)  1985, 1986

2771, Middleborough

San Jose, CA 95132

FOR IBM PC   AND  XT & COMPATIBLES

AT needs HDPREPAT.exe

DOS 2.x/3.x

ALSO  XEBEC 1210A

DTC   5150BX

ADAPTEC 2002A

W DIGITAL 1002-WX2

ETC. PC CONTROLLER BOARDS

** If you think the program is useful  to you,

$35.00 donation to    H. King   at above

address will register for updating service

2.  The program READ in the hard disk drive parameters from the

controller to determine the available number of heads and cylinders

for each attached drive. This assumes the controller follows the IBM BIO

convention. Otherwise, the program defaults to 2 head and 100

cylinders for each drive. When this happens, the controller usually

has problem. You can use option 9 to change the settings. However,

the change is only temporary since the drive parameters are in the

controller ROM. If you have an Adaptec controller, the parameter will

be recorded to disk drive and later still can be recognized by the

DOS. Western Digital's Cobra board is not supported at this time.

3.  Hard disk preparation uses the controller default parameters in the

BIOS unless it is overwritten by option 9.

Instructions:

The program is manual driven and self explanatory. The following is a

summary.

Options:

1. Select the drive to be operated on. Drive 0 is the first hard disk

drive attached to the controller (usually the C:), Drive 1 is the

second one (usually the D:).

2. Perform the controller primary FORMAT of the specified drive by option 1.

See option 3. Run FDISK and Dos FORMAT after this step.

Bad track locations can be input at this step and the program

shall use the input to mark the bad ones accordingly. Option

is allowed to enter the file name that contains the defect track

location table. In this defect table file, each record has two

numbers. The first number is the head number and the second one

is the track (cylinder) location.

3. Perform the controller primary FORMAT of the specified drive by option 1

and scan the disk for bad tracks and mark them bad. Run FDISK and

DOS FORMAT after this step. This option 3 is preferred to option 2.

In the begining of this step, you can enter the bad tracks if you

already know them.  This step marks  any bad track found, plus

the ones you entered,  at the end of the process. As in the option 2,

the defect table file name can be used.

4. Move the head to track 0 for the specified drive.

5. Exit this program.

6. Set the interleave number for the controller primary FORMAT. The default is

6 which is used by IBM PC/XT. Do not use other number unless you

have enough knowedge on the controller. Adaptec controller can use

interleave 2.

7. If you know a specific track and head location is bad, you can use

this option to mark the track bad. Use DOS FDISK AND FORMAT after

this step.

Option allows the program to scan the drive and lists tracks that

already marked bad.

8. If you want to remark a bad track to reuse the track, you can use this

option. This option can not physically correct the defect track.

Option allows the program to scan the drive and lists tracks that

already marked bad.

9. Use this option to define the number of heads and cylinders for each

drive for this program to use. This option   does  overwrite

the controller defined parameter and affect the controller primary FORMAT

operation. The BIOS parameters are not affected. Warning: The DOS

FORMAT uses BIOS parameters.

However, the ADAPTEC 20xx  is an exception, the overwrite parameters

are recognized by DOS Fdisk and Format programs.

This option has a table of predefined drive parameters. You can select

a drive type from the table or input your own parameters.

A. The same operation as the option 3, except during the verify process

more data patterns are used. The process can take twice longer then

the option 3. Usually, the option 3 is good enough.

Note:

At option 2,3,7 and 8, head and track location input feature allows

the number entering easier and can be edited later. After the completion

of input process, the screen shall display the entered bad tracks information.

Once the total track  location display is on, you can use the cursor key to move the

pointer (reversed video) to different line/column and use command

to change (C),  remove (R), Re-enter every thing (N), go ahead to

proceed (Y). The editing options are displayed at the bottom of the

screen for reference.

If a new item need to be added, use cursor key to move the pointer

to a location that does not have any entry, hit "C" - change, head

number and cylinder number can then be added.