1.  INTRODUCTION

This documentation  explains how to use the SCHDRAW program.

SCHDRAW lets  you draw  schematics and  other drawings.  You

should have  already run  the  automatic  demo  program,  as

explained in  the file  READ.ME.   If not,  read  that  file

before continuing.

2.  GETTING READY

NOTE: In  the following  discussions, all text within quotes

is text  to be  entered by you, from the keyboard.  The text

"<Enter>" represents a single keystroke of the Enter key.

You must first install the Microsoft-compatible mouse driver

program that  was supplied  with your mouse.  See your mouse

manual for applicable information.

After  your  system  is  booted  and  the  mouse  driver  is

installed, insert  the demo  disk in  drive A and make A the

current drive: "A:<Enter>".

If you have not run the self-running demo program (IBMCOLOR,

IBMENHAN, HERCULES,  or AT&T), perform one of the following,

depending on the graphics adapter you have:

IBM Standard Color:  "COPY SCHEMA1.CFG SCHEMA.CFG<Enter>"

IBM Enhanced (EGA):  "COPY SCHEMA2.CFG SCHEMA.CFG<Enter>"

Hercules:            "COPY SCHEMA3.CFG SCHEMA.CFG<Enter>"

AT&T 6300:           "COPY SCHEMA4.CFG SCHEMA.CFG<Enter>"

You are  now ready  to run SCHDRAW.  First, we will walk you

through the use of some of the more common SCHDRAW commands.

After this,  you can  go  back  and  experiment  with  other

SCHDRAW commands.   At  the  end  of  the  discussion  is  a

complete list  of  SCHDRAW  commands  and  their  functions.

Since  all  disk  operations  have  been  removed  from  the

programs, there  is no  way you  can harm your system or the

disk - worst case, you may have to reboot.

3.  SCHDRAW

SCHDRAW is  SCHEMA's main program.  All schematic and object

creation is  done in  SCHDRAW.   To invoke  SCHDRAW  with  a

drawing, enter the following:

"SCHDRAW LOGIC<Enter>"

SCHDRAW briefly  displays  a  sign-on  message,  clears  the

screen, and  places a Command Window (with Command Boxes) on

the left side of the screen and a Drawing Window on the rest

of the  screen.   After this, it takes a few seconds for the

drawing and  objects to  load.  If an error message appears,

you probably  have not  installed the mouse driver correctly

or renamed the SCHEMAx file.  Go back and do this.

When the  drawing has  completed loading,  the cursor should

appear in  the middle  of the  Drawing Window.   Moving  the

mouse moves  the cursor.   If  the cursor  does not move, or

moves in an unusual manner, the mouse driver is probably not

installed.    You  should  exit  SCHDRAW  (CNTRL-BREAK)  and

reinstall the mouse driver.

If the cursor is working correctly, move it to the bottom or

right edge  of the screen.  The drawing pans to the right or

bottom.   You can pan the screen by moving the cursor to any

screen edge  (including  the  far,  left  edge).    Practice

panning the  drawing until  you feel  comfortable  with  the

panning feature.

Next, move  the cursor  over the  ZOOM Command Box and press

and release (click) the left mouse button.  (This process of

invoking a  command is  referred to  as 'selecting').    The

screen now  shows a  less detailed,  but larger  area of the

drawing.  Again, moving the cursor to a screen edge pans the

drawing.

Now, select  PAGE and  you will  see the  complete schematic

page on  the screen.   Go back and select ZOOM again and the

screen returns to the original viewing range.

Now, select DRAW.  You will see a new set of commands appear

in the  lower portion  of the Command Window.  These are the

DRAW Secondary  Commands.   Next, select  WIRE.    Move  the

cursor to  the center  of the  Drawing Window  and press and

hold the  left mouse  button.  Now, as you move the mouse, a

line is  drawn.  Right angle lines are easily drawn.  If the

cursor touches  a screen edge, the drawing pans and the line

continues.   When you  release the mouse button, the line is

completed.   Experiment with  lines by drawing several.  You

can change  from signal to bus lines by reselecting the WIRE

Command Box.   You will notice the thin line change to a fat

line in the WIRE Command Box.

Next, select OBJECT.  When OBJECT is selected, a Text Window

appears in  the lower  portion of  Drawing Window.   This is

where keyboard text is displayed.  Use the keyboard to enter

the following object name: "74LS86<Enter>".  The Text Window

is removed  and  the  disk  drive  is  selected  as  SCHDRAW

retrieves the  74LS86 object  from the object library.  Now,

move the  cursor to  the center  of the  Drawing Window  and

press and hold the mouse button.  The 74LS86 object appears.

As long  as you  hold the  mouse button,  you can  move  the

object around  the drawing.   If  you touch  a  border,  the

drawing pans.  Releasing the button places the object.  Also

at this  time, the  Text Window reappears, letting you enter

the name of another object.  If you enter: "? 74*<Enter>", a

list of  all objects  in the  demo library beginning with 74

appears.   Press "<Enter>"  to remove the directory from the

screen.   Experiment drawing  various objects.   By entering

"\n" after  an object  name, you  can call-up alternate pin-

outs for  objects, i.e.,  entering  74LS00\3  retrieves  the

third (C) section of a 74LS00 object.

Now select  LABEL.   Again the Text Window appears.  You can

enter any text. For example: "TEST<Enter>".  The Text Window

disappears.   Position the  cursor in the Drawing Window and

press and  hold the  mouse button.   The  label just entered

(TEST) appears.   As  long as  the mouse button is held, the

label can  be moved  anywhere on the drawing.  Releasing the

button places the label.

Now,  select  EDIT.    Another  set  of  Secondary  Commands

appears.   Select MOVE.  Position the cursor over one of the

objects you  added.   Now press  and hold  the mouse button.

(If a beep is generated, you have not placed the cursor over

the object.  Release the  mouse button  and try again.)  The

object is  now selected  and as  long as  you hold the mouse

button, the  object can  be moved.    Releasing  the  button

places the  object.   As you  see, no rubberbanding of wires

takes place  on object  moves.   However, rubberbanding does

occur on REGION moves.  Next, select COPY.  Repeat the above

sequence and  instead of  moving the  object,  you  are  now

making a copy of it.  Let's erase the object.  Select ERASE.

Again, position  the cursor  over the  object and  click the

mouse button.   The  object disappears.  During any of these

operations, you  can select  UNDO to  undo the operation you

have just done (you could have also selected UNDO during any

of the DRAW commands).

Now select  SCH ED.   A prompt appears on the screen.  Press

"<Enter>".   After a  few seconds,  a new  set  of  commands

appears in  the Command  Window.   You are now in the Object

Editor, used  to create  and edit  objects.  You will notice

some of  the  OBJ  ED  commands  are  identical  to  SCH  ED

commands.   Also  some  new,  object-oriented  commands  are

displayed.   Select OBJ  ED, and  press "<Enter>"  after the

prompt.   After a  few seconds,  you will  be  back  in  the

schematic editor.

Now select FILE, followed by QUIT.  A prompt appears - press

"<Enter>".  You are now back in DOS.

This completes  our brief introduction to SCHDRAW.  Try some

of the  other commands we have not discussed.  If you want a

blank piece of paper to practice with, enter with:

"SCHDRAW BLANK<Enter>"

This invokes  SCHDRAW  with  a  blank  drawing  page,  named

BLANK.P01.

For more information or to order SCHEMA, contact:

Omation, Inc

1701 Greenville Ave., #809

Richardson TX,  75081

214-231-5167

4.  SCHDRAW COMMAND SUMMARY

The following  is a list of all SCHDRAW commands.  Schematic

Editor commands  are listed first, followed by Object Editor

commands.

SCHEMATIC EDITOR COMMANDS

AREA -  Shows the  unZOOM area  when in the ZOOM mode.  Also

lets you position the unZOOM area.

CONFIG  -  Sets  drawing  editor  configuration  parameters.

Initially set via the configuration file.

C GRID  -  Activates  and  deactivates  the  invisible

cursor grid.

LIBR - Displays the names of current object libraries.

PAN - Sets the Drawing Window panning speed.

RPT -  Sets the  repeat X- and Y-offsets for use under

DRAW*REPEAT.

V GRID  - Activates and deactivates the visual drawing

grid field.

WI HDR  - Activates  and deactivates  the signal wire,

object, and  label  end-points  for  easier  schematic

creation.

XY - Turns the Cursor Coordinate Window on or off.

DRAW -  Used to add schematic information to drawings.  Most

of the information is interpreted by the post-processors.

ARC -  Adds right-angle  signal and  bus  connections.

Four different  orientations are available.  WIRE size

selects ARC type: signal or bus.

DOT - Adds connection dots to signal T-connections.

OBJECT -  Retrieves  objects  from  object  libraries.

Also, displays library directories.

LABEL - Adds labels to signal wires.

REPEAT - Steps and repeats the last command.

WIRE - Adds signal and bus wires.

EDIT - Used to edit existing drawing information.

COPY - Adds a copy of an existing image or region.

ERASE - Removes an existing image or region.

MOVE - Moves an existing image or region.

OBJECT - Edits an existing object's text line, for the

purpose of  changing reference  designators,  sub-part

identification, etc.

OBJ PM - Displays an object's required parameters.

REGION -  Defines an  area to  be  moved,  copied,  or

erased.

FILE - Various file operations.

LOAD - Loads a drawing page into the Schematic Editor.

Also used to view DOS directories.

PRINT -  Prints the drawing directly to the printer or

spools it  to disk for background printing.  ***NOT IN

DEMO VERSION***

REDRAW - Redraws the current drawing.

SAVE -  Saves the  current drawing to disk.  ***NOT IN

DEMO VERSION***

QUIT - Returns to DOS.

NOTES -  Used to  add non-schematic  lines  and  text  to  a

drawing.  Not interpreted by SCHEMA's post-processors.

LINE - Adds thin, thick, dotted and dashed lines.

TEXT - Adds small and large text.

PAGE - Displays the entire page of the current drawing.

UNDO - Reverses the last executed command.

ZOOM - Allows two different viewing perspectives.

OBJECT EDITOR COMMANDS

BIT -  Used to create or edit bit images.  Mode 1 selects an

existing bit  image.   Mode 2  is used  to create/edit a bit

image.  Mode 3 places the bit image in the object.

BODY - Assigns a body object to a top-level object.

C GRID  - Activates  and deactivates  the  invisible  cursor

grid.

DESC -  Defines the  four description fields.  These contain

BOM information, part numbers, in-house descriptions, etc.

DF PI  - Defines pin connection points, pin types, and names

for post-processor net extraction.

ERASE - Erases an image from the drawing window.

ID - Displays object subcomponents.

LINE - Adds thin, thick, dotted, and dashed lines.

LOAD - Loads preexisting objects into the Object Editor.

NEW - Clears the Object Editor.

OBJ - Adds a nested object to an object.

PARM - Defines object parameter locations and names.

PI HD - Adds pin headers.

SAVE -  Saves an  object to  a selected  library.  ***NOT IN

DEMO VERSION***

TEXT - Adds text.

UNDO - Reverses the last command.

ZOOM  -   Displays  objects   from  two   different  viewing

perspectives.

###########################################################################################################