NMI Version 1.01

Have you ever seen the message 'PARITY CHECK' on your display?  If

you have, you've probably noticed that it always seems to happen at the

most inopportune time.  You're cruising along putting the finishing

touches on something that you've worked long and hard, just to look up

from the keyboard to a blank display with only the words 'PARITY CHECK'

in the upper left corner.  Your heart sinks into your shoes, your face

gets flush, and palms begin to sweat, as you try franticly to figure a

way out of this one... but you KNOW that you've lost everything!  Your

only option is to power off and start over.

The IBM-PC is a great machine, and parity checking is a wonderful

concept, but I've always wished that I had been given a choice before

the machine was put to sleep.  Even more, I've wished that somehow the

machine could have told me where it hurts before it died.  I've never

gotten much of anything by wishing, so I sat down to attempt to remedy

the situation.

NMI is a resident program to replace the non-maskable interrupt

handler that the PC uses to handle parity errors.  In the event of a

parity error, NMI will interrupt the currently executing program(s) and

begin testing memory, looking for the error.  It will continue testing

until either it locates a failing address, or you interrupt it by

pressing a key.  If a failure is located, NMI will display the failing

64K bank, exact memory address, and bit(s).  You have the option to

continue or terminate testing.  If you interrupt the memory test, or

elect not to continue testing after an error is detected, you are given

the options to return to the interrupted program(s) with or without

parity checking enabled, or to reboot the system.

The memory test is of a non-destructive type, in that it will

preserve the current contents of memory.  It can be executed any time

without regard for concurrently executing processes.  It's also a very

thorough test, reading and writing each byte of memory with several

different bit patterns and checking for parity errors following each

read or write operation.  Every memory location is tested, including

the locations used by NMI.

This all sounds a bit like insurance doesn't it?  It doesn't do you

a bit of good unless you have a tragedy on your hands.  Well, I like

programs that you can play with, and since you've got this neat little

memory test just sitting there in memory, why not be able to use it?

NMI will execute the memory test any time you like, as many times as

you like!

If you're like me (a memory miser), the question in your mind right

now is... 'Just how much memory does this thing eat?'.  The answer,

approximately 1.8K including the Program Segment Prefix and a normal

size environment.  In English, that means about 1.8K TOTAL!  Even I can

afford that.

NMI (Non-Maskable Interrupt)

PROGRAM LOADING

---------------

NMI /N/I/T####

| | |

| | Test memory #### times.

| |

| |   If the number of test repetitions is 0 or blank, then the

| | memory test loops indefinitely until an error is detected

| | or you interrupt it by pressing a key.  This option may be

| | used to invoke the memory test from the command line or a

| | batch file.  NMI need not be installed to run the memory

| | test.

| |

| Install NMI.

|

|   This option must be selected the first time NMI is executed

| if you desire to replace the current non-maskable interrupt

| handler.  It need not, and should not be specified again.         |

|

No program logo display.

This option is selected to suppress display of the program

logo when NMI is installed.

*  If no parameters are specified, or the specified parameters are

invalid, NMI will display the program logo followed by a summary of

valid parameters.

*  Parameters need not be separated, however, they may be separated by

one or more spaces, tabs, commas, slashes, or semi-colons.

*  Alphabetic parameters may be upper or lower case.

*  Parameters may be entered in any order or combination so long as the

number of test repetitions follows the 'T' parameter and they are

not separated by another parameter.

NMI (Non-Maskable Interrupt)

+--------------------------+

|   TESTING ####K MEMORY   |

| Bank    Bit     Address  |

|  00  P76543210 0000:0000 |

|        0000 Passes       |

+--------------------------+

When the memory test is invoked, either as the result of a parity

error, or from the command line, the above window will be displayed in

the upper left corner of the screen.  This window indicates the size of

the memory being tested, as sensed by NMI, the 64K bank currently being

tested, and the number of successful test repetitions.  In the interest

of speed, the 'Address' field is updated only for each 64K bank in this

window.  The 'Bit' field is used to indicate failing bits and will not

change unless an error is detected.

If the 'Esc' key is pressed, the memory test will be terminated with

parity checking enabled, and control passed to the interrupted process.

Any other key will exit the memory test and display the termination

options.

The memory test takes approximately 35 seconds per 64K bank, longer

depending upon other system activity.

+--------------------------+

|   ** ERROR DETECTED **   |

| Bank    Bit     Address  |

|  00  P76543210 0000:0000 |

| Continue testing? _      |

+--------------------------+

In the event that an error is detected during the memory test, the

above window will be displayed.  This window indicates the failing 64K

bank, exact address, and bit(s).  Depending upon the display type and

video mode active at the time of the failure, the failing bit(s) will

be indicated either by blinking, or non-display.  These bit(s) directly

correspond to the failing chip(s) in the indicated 64K bank.

Pressing 'Y' (or #DDY) will continue the memory test starting with

the next byte.  The number of successful passes will be reset to zero.

Pressing 'N' will exit the memory test and display the termination

options.

NMI (Non-Maskable Interrupt)

+--------------------------+

| 1) Continue with parity  |

| 2) Continue w/o  parity  |

| 3) Reboot system         |

| Enter selection: _       |

+--------------------------+

The termination option window is displayed as a result of any key

other than 'Esc' being pressed during the memory test, or electing not

to continue testing following an error detection.  Selecting option 1

will return to the interrupted process with parity checking enabled.

Selecting option 2 will resume the interrupted process without parity

checking.  Option 3 will reboot the system.  Pressing #DDY will restart

the memory test.

RETURN CODES

------------

If NMI is invoked from within a batch file, or by another process, a

return code may be retrieved indicating any errors.  For batch files,

the return code may be interrogated using the 'IF ERRORLEVEL' batch

subcommand.  The following table indicates possible return codes for

the current release.

0 - Successful, no errors detected.

1 - Error detected in parameters, program aborted.

2 - NMI was invoked with the 'I' parameter, but is already

installed.  Installation aborted.

4 - Memory test already active, no memory test performed.  This

error can occur in multi-partition systems if memory test is

called while active in another partition.

It is possible for more than one error to be returned in the return

code.  For example, a return code of 6 indicates both errors 2 and 4

occurred.

PROGRAM DESIGN CONSIDERATIONS

-----------------------------

Certain performance and function trade-offs are made in the design

of every program.  These trade-offs are usually made in an attempt to

provide the most function within the constraints imposed by the

operating environment in which the program must run.  In this case, the

following criteria were taken into consideration.

1) All resident programs must use the smallest possible amount of

memory.

2) During the memory test, it is assumed that a memory failure has

occurred.  It is possible that the memory failure occurred in the

resident portion of DOS.  For this reason, DOS function calls must

not be used within the resident NMI code.

No attempt is made to save and restore the contents of

graphics screens.  This would have required additional code

and buffer space within the resident portion of the program.

If a disk file had been used to save the video buffer, DOS

function calls would have been required, increasing the

possibility of program check during the memory test.

3) The memory test should first attempt to find the failure using a

read operation, so that the original failure remains undisturbed.

If the read operation fails to produce the error, only then should

write operations be used.  Furthermore, the failure should be

located in a single pass through memory, if possible.  This method

will provide the best odds of locating an intermittent failure.

Memory test execution is slow, only because the test is

thorough.

#########################