PATCHING NOVELL V2.0A MANUALLY

For the /SFT versions of Novell 2.0A, modify the following files

found on the Novell Installation diskette:

COMPSURF.EXE

INSTALL.EXE

DISKED.EXE

LOADER.DAT

and the following additional files, found on the Genos-4 diskette:

NET$OS.OBJ

PREPARE.EXE

Use the procedure below to patch the above files:

1.  Use an editor capable of modifying executable files (e.g.,

Norton Utilities and AT86).

2.  Load the file into memory.

3.  Search the file for the hexadecimal string to be modified.

4.  Change the appropriate bytes as shown below.

NOTE: some of these strings may appear more than once in a single

file.

ORIG            CHANGE TO NEW HEX STRING FOR:

HEX

STRING

17 sect/trk   26 sect/trk   34 sect/trk   35 sect/trk   36 sect/trk

-------------------------------------------------------------------

8AC1241F      8AC1241F      8AC1243F      8AC1243F      8AC1243F

FE22          FE34          FE44          F346          FE48

EE22          EE34          EE44          EE46          EE48

3C12          3C1B          3C23          3C24          3C25

B911          B91A          B922          B923          B924

B9600BAF701   <-- Same      B90002BAF701  <---------------- Same

B90002BAF701  <-- Same      B90002BAF701  <---------------- Same

B90002BAF701  <-------------------------------------------- Same

The last change listed is in the LOADER.DAT file only.

80E11F        80E11F        80E13F        80E13F        80E13F

NOTE:  You must still modify your system BIOS to provide a drive

type table with the correct number of cylinders and sectors per

track for Novell v2.0a to run. If your system has the ACB-2320D

hard disk controller, after performing a low-level format, disable

the BIOS by setting J12 to having Pins 1 and 2 OUT and Pin 3 IN (IN

= shunt installed).

Novell v2.0a does not support disks with capacities greater than

250-MB. The system BIOS drive type table must specify less than

250-MB worth of storage. Use the following to calculate the maximum

number of cylinders that may be specified:

MAXIMUM NUMBER OF CYLINDERS =<R>   512,000 + (SECTORS PER TRACK x

HEAD COUNT)

Unpredictable results may occur when attempting to use a disk with

greater than 250-MB of storage.

PATCHING NOVELL V2.0A ELS-I MANUALLY

Modify the following files found on the Installation diskette:

COMPSURF.EXE

PREPARE.EXE

LOADER.DAT

INSTALL.EXE

and the following additional files, found on the OS ARCnet and OS

Ethernet diskettes, respectively:

NET$OS.OBJ

PREPARE.EXE

Use the procedure below to patch the above files.

NOTE: You cannot modify the .EXE files using the DOS debugger.

Therefore, you must first rename these files, patch them, and then

rename them back to their original names.

1. Use an editor capable of modifying executable files (e.g.,

Norton Utilities and AT86).

2. Patch two strings into the first four files, as follows:

a. Change B9 64 00 into B9 00 02.

b. Change 3D 00 C8 72 into 3D 00 00 72.

3. Change the string 3D 00 C8 72 (in the INSTALL.EXE file) into

<R>3D 00 00 72.

PATCHING NOVELL V2.12 ELS-II MANUALLY

The purpose of this patch is to locate four occurrences of the code

sequence 3D 00 C8 72 and to replace, in each occurrence of that

code sequence, the value C8 with the value 00.

Modify the following files found on their respective diskettes:

NDELS2-2.OBJ - OSOBJ Diskette

INSTOVL.EXE - ELSGEN Diskette

COMPSURF.EXE - UTIL-1 Diskette

VREP286.EXE - UTIL-2 Diskette

DISKED.EXE - UTIL-2 Diskette

Use the procedure below to patch the above files:

1. Make a working copy of the original diskette. Work only on the

working copy.

2. Use an editor capable of modifying executable files (e.g.,

Norton Utilities and AT86).

3. Patch the NDELS2-2.OBJ file.

a. Load the debugger and the NDELS2-2.OBJ file by entering the

following command:

DEBUG A:NDELS2-2.OBJ <<Enter>>

The NDELS2-2.OBJ file is fairly large and the code sequences we are

searching are spread over several 64-KB segments so the first thing

we need to do is find out the current value of the code segment

register CS by entering the command:

R <<Enter>>

The debugger displays the following format:

AX=0000 BX=0003 CX=0DC5 DX=0000 SP=FFEE BP=0000 SI=0000 DI=0000

DS=1157 ES=1157 SS=1157 CS=1157 IP=0100 NV UP EI PL NZ NA PO NC

1157:0100 820700           ADD  BYTE PTR BX ,00          DC:003=A0

b. Search for the hexadecimal code sequence 3D 00 C8 72 by entering

the command:

S RRRR:0 FFFF 3D 00 C8 72 <<Enter>>

where RRRR is the value of the CS register incremented by 2000. The

debugger should find two occurrences of that sequence and tell you

the address of the first byte of each sequence with a display

having the following format:

RRRR:XXXX

where XXXX and YYYY are the RRRR:YYYY addresses of the first byte

of each occurrence.

c. Repeat the search operation to find the two other occurrences of

the same sequence by entering the command:

S BBBB:0 FFFF 3D 000 C8 72 <<Enter>>

where BBBB is the value of the CS register incremented by 3000.

Again, the debugger should find two more occurrences of that

sequence and tell you the address of the first byte of each

sequence with the same format of display.

BBBB:UUUU where UUUU and WWWW are the BB:WWWW addresses of the

first byte of each occurrence.

d. Replace the value C8 by the value 00 in the sequence 3D 00 C8 72

by entering the command:

E RRRR:XXXX <<Enter>>

The debugger displays the value 3D. Press the SPACEBAR twice and

the debugger displays the value C8. Type in 00 and press ENTER. The

other occurrences must also be modified by entering the commands,

respectively:

E RRRR:YYYY <<Enter>>

E BBBB:UUUU <<Enter>>

E BBBB:WWWW <<Enter>>

and repeating the same sequence of keystrokes. To make sure the

patch was properly done, display the new code sequence by entering

the command:

D RRRR: YYYY <<Enter>>

You should now observe the sequence 3D 00 00 72.

e. Save the patched file on the diskette by entering the command:

W <<Enter>>

4. Patch the other executable files. The procedure for each file is

identical. Therefore, repeat the example of INSTOVL.EXE to patch

the other files. If you use the DOS debugger and because you cannot

patch on the .EXE file, you must rename the file with a different

extension.

a. Rename the file using the command:

REN A:INSTOVL.EXE A:INSTOVL.OBJ <<Enter>>

b. Load the file with the following command:

\DEBUG A:INSTOVL.OBJ <<Enter>>

c. Search for the hexadecimal code sequence by entering the

command:

S CS:0 FFFF 3D 00 C8 72 <<Enter>>

The debugger should find four occurrences of that sequence with the

following display:

XXXX:AAAA

XXXX:BBBB

XXXX:CCCC

XXXX:DDDD

d. Modify each occurrence with the following command:

E CS:AAAA <<Enter>>

The debugger displays the value 3D. Press the SPACEBAR twice, the

debugger displays the value C8. Type in the value 00 and press

ENTER.

e. Repeat the same process for three other occurrences with the

following commands:

E CS:BBBB

E CS:CCCC

E CS:DDDD

f. Save the patched file on the disk, after patching the four

occurrences, with the following command:

W <<Enter>>

g. Rename the file to its original name with the following command:

REN A:INSTOVL.OBJ A:INSTOVL.EXE <<Enter>>

h. Repeat this patch procedure for the three other executable files

(i.e., COMPSURF.EXE, VREP286.EXE, and DISKED.EXE).

PATCHING NOVELL V2.1X MANUALLY

The ATDISK.OBJ software patch allows the use of the ESDI-32's

primary format and autoconfiguration feature. This feature gives

you the capability to use all kinds of different disk types while

leaving the "setup" of the disk type definition to "Type 1." Also,

there is no need to modify the content of the motherboard BIOS.

Use the following procedure to patch the above file:

1. Make sure the ESDI-32 BIOS is enabled (factory default). Use the

SETUP program to select Drive Type 1.

2. Use the disk preparation program when formatting the drive for

the first time. This is mandatory for the autoconfiguration to work

properly. Later, you can reformat the disk using Novell's compsurf

program. Start the disk preparation program by entering the DOS

debug program and typing the command:

G=C800:5 <<Enter>>

a. Select option 0 to do the primary format.

b. Use option 1 to create the logical partition when the primary

format is finished. (You must use this in order to get a successful

autoconfiguration.

c. Select option 0 to create a volume and select volume number 1.

d. Enter the total amount of cylinders available. This amount is

displayed on top of the screen and can be larger than 1024.

e. Exit the disk preparation program by selecting option 3 and then

option 5.

3. Execute the ATDISK.OBJ patch, provided with the standard driver

program on the Novell v2.1x software. This file must be patched

before generating the utilities and Operating System, using the

standard DOS debug program.

a. Make a backup of the original diskette and work from the backup

diskette.

b. Load the debugger and the ATDISK.OBJ file by entering the

command:

Debug ATDISK.OBJ <<Enter>>

c. Search for the hexadecimal code sequence B9 64 00 by entering

the command:

S CS:0 FFFF B9 64 00 <<Enter>>

The debugger should find one occurrence of that sequence and tell

you the address of the first byte of the sequence with a display

having the following format:

XXXX:YYYY

where YYYY is the address of the first byte of the sequence

In the sequence of code B9 64 00, the value 64 must be replaced by

value 00 and the value 00 must be replaced by value 02. Do this by

entering the command:

E CS:YYYY <<Enter>>

The debugger displays the value B9. Press the SPACEBAR once the

debugger displays the value 64. Type in 00 and press the SPACEBAR.

The debugger now displays the value 02. Type in 01 and press ENTER.

To make sure the patch was properly done, display the new code

sequence by entering the command:

D CS:YYYY <<Enter>>

You should now observe the sequence B9 00 02.

d. Search for the hexadecimal sequence of code 3D 00 C8 72 by

entering the command:

S CS: 0 FFFF 3D 00 C8 72 <<Enter>>

The debugger should find TWO occurrences of that sequence and tell

you the address of the first byte of each sequence with a display

having the following format:

AAAA:BBBB

where BBBB and YYYY are the XXXX:YYYY addresses of the first byte

of each occurrence

In the sequence 3D 00 C8 72, the value C8 must be replaced by value

00. Do this by entering the command:

E CS:BBBB <<Enter>>

The debugger displays the value 3D. Press the SPACEBAR twice and

the debugger displays the value C8. Type in 00 and press ENTER.

Modify the second occurrence by entering the command:

E CS:YYYY <<Enter>>

Again, the debugger displays the value 3D. Press the SPACEBAR twice

and the debugger displays the value C8. Type in 00 and press ENTER.

To make sure the patch was properly done, display the new code

sequence by entering the command:

D CS:BBBB <<Enter>>

You should now observe the sequence 3D 00 00 72.

e. Save the patched file on the diskette by entering the command:

W <<Enter>>

4. Follow the standard Novell v2.1x software generation procedure,

that is, the link of the utilities (e.g., COMPSURF.EXE, DISKED.EXE,

and REPAIR.EXE) and the Operating System.

5. Run the newly-generated compsurf.exe program once the link and

generation is finished. Compsurf is a program supplied by Novell to

assess the disk data readability. It has the ability to primary

format and extensively verify the hard disk.

If you use the compsurf format option, enter the manufacturer

defect list. Compsurf has its own defect handling and maps defects

at a track level. (If a track contains one or several defects, all

blocks on that track are marked bad.)

6. Install the Novell v2.1x software by following the standard

installation procedure described by Novell once compsurf.exe is

done (displayed with the message "disk passed").