TO INSTALL THE REALPORT DRIVER FOR AIX 3.2.5 AND 4.X. ALSO HOW TO

ADD A PORTSERVER, TTY AND LP DEVICE TO THE SYSTEM.

TO INSTALL DRIVER;

From unix;

installp -a all

From smit

Software Installation and Maintenance

Install and Update Software

Install/Update Selectable Software

Install Software Products at Latest Level

Install New Software Product at Latest Level

INPUT device / directory for software

/dev/fd0 for diskette

or

Enter directory where driver file is located

SOFTWARE to install

digiasync.ncxa.obj

COMMIT software updates?

no

SAVE relpaced files?

yes

TO ADD PORTSERVER TO SYSTEM;

smit devices

Digi Portservers

Add a Portserver

Select appropriate PS type that is being added

INTERNET NAME

INTERNET ADDRESS

Enter PS node name or PS IP address, not both

A sa device will be created for the PS, make sure that it is available

TO ADD TTY DEVICES;

smit tty

Add a TTY

rs232 Asychronous Terminal

Select appropriate sa device

Enter appropriate entries then enter to save

TO ADD LP DEVICES;

smit printer

Printer/Plotter Devices

Add Printer/Plotter

Select appropriate printer

rs232

Select appropriate sa device

Enter appropriate entries then enter to save

TO REMOVE APPLIED DRIVER;

Remove all associated Realport tty, lp and sa devices;

rmdev -l <device> -d

smit

Software Installation and Maintenance

Manage Applied Software

Remove Applied Software Products

SOFTWARE name

digiasync.ncxa.obj