DigiWARE Release Notes 93000043M

Diskette(3.5")           P/N 40000309M  V2.2.1

Diskette(5.25")          P/N 40000310M  V2.2.1

Software Package         P/N 75000043M

Software Manual          P/N 92000043D

for

Digi PC/X, MC/X and Modem/X OS/2 2.x Device Driver

DGX.SYS     V 2.2.1

IBM OS/2 1.3, 2.00, 2.01, 2.1, 2.11, 3.0

9/11/95

1. Enhancements

- Added "/a:q" command line parameter to cause DosWrites to wait for

data to drain fron the card before returning. See Manual Errata.

- Made performance enhacements to decrease the latency between the

time Flow Control indicates transmit should resume and the time

it actually does.

2. Bug fixes

- Fixed bug where DosWrite()'s could block indefinately.

- Fixed bug where DosOpen() would take too long due to an unneeded

thread sleep.

- Fixed bug where GET_LINE_STATUS IOCtl would report transmitter

empty before all data actually left the card.

- Fixed bug in SET_DCB IOCtl where DTR and/or RTS would always get

set high if their Control Mode Setting was already enabled. This

should have happened only if SET_DCB was CHANGING their Control Mode

Setting to enabled. This would result in RTS and/or DTR being set

high unexpectedly.

- Fixed bug in SET_DCB IOCtl where on transitions from RTS and/or DTR

Handshaking to Control Mode disabled, DTR and/or RTS lines would

not always be turned off or brought low.

3. Known limitations

- A maximum of 32 ports are available using DGX.SYS in OS/2 sessions.

- Though many channels can be active at 1200-4800 baud, only a few

channels can sustain the higher baud rates of 9600 or more without

data loss and serious system performance degradation.

4. Manual Errata

- The included manual has not yet been updated to incorporate the

MC/X.  See the 'INSTALL.DOC' file on the distribution diskette.

In general, the MC/X not requiring per-card hardware arguments is

the only substantive difference.

- A new command line switch "/u:" permits the "U"ART start address

of daisy-chained boards to be specified so that multiple boards

may use the same IRQ and status port address. See 'INSTALL.DOC' for

more information on this feature.

- The new "/S:" option for disabling DTR and RTS control modes

is described in INSTALL.DOC

- A new command line switch "/a:q" allows the user to DISABLE the

default "quick write" feature. See "INSTALL.DOC".