Installation Guide

Reference Manual for

Universal QNX4 Device Driver

for DigiCHANNEL Intellegent Communications Boards

92000127A

Introduction

Dq.ser is a device driver that runs under the QNX4 "Dev" process,

providing the hardware-dependent component for DigiBoard intelligent

serial boards (DigiCHANNEL PC/Xi, PC/Xe, PC/Xem, PC/Xm,

COM/Xi and MC/Xi boards) and cluster controller systems

(DigiCHANNEL C/X System).

Once the Dq.ser driver has initialized the hardware and registered

itself with Dev, new serial device nodes will appear in the /dev

directory.  Dq.ser thereafter takes care of having these devices

emulate standard serial communications hardware.  When the nature of

the I/O requests permits it, Dq.ser will take full advantage of the on-

board buffering and intelligence of the DigiCHANNEL hardware.

Advantages include efficient, flow-controlled transmission of large

blocks of data; buffered inputs rendering data loss extremely unlikely;

and almost no interrupt-time processing that could interfere with other

Real Time Critical processes.

Installation

Dq.ser is a command line invoked device driver, so installation

consists of little more than copying the files into a directory on the hard

disk drive.

To copy the files, perform the following steps:

Step 1. Log in as super user (root).

Step 2.	Place the DigiBoard device driver diskette in Drive A

(/dev/fd0) or Drive B (/dev/fd1).

Step 3.	Type install (and press <Enter>).

The install program will extract all of the files from the DigiBoard

diskette and place them in the default DigiBoard directory

/etc/digi.

The files in /etc/digi are loaded when Dq.ser is invoked from the

command line.  Only those files that are appropriate to the specified

command line options are loaded, thus minimizing the amount of system

memory required to support the device driver.

The minimum requirements to run Dq.ser are:

*	All distribution files must be in the default DigiBoard directory.

*	Since devices will be mounted in /dev, Dev must have enough

space reserved for all the expected devices (see the section on Dev

in the QNX4 Administrator's guide).  For example, if 32 serial

ports are expected, the line in the sysinit file might be:

Dev -n 50 &

to have Dev reserve space for up to 50 nodes in /dev.

*	Settings for I/O port addresses, shared memory address and

concentrator daisy chaining configuration must be specified for each

card.

Invoking the Device Driver

Dq.ser is invoked from the command line (or from sysinit); all

DigiCHANNEL boards to be supported by the driver must be

described in the command line arguments.  After initialization, which will

take a few seconds, the device names will appear in /dev and are

ready for use.  See the QNX4 Administrator's Guide for information on

enabling ports for logins.

Command Line Arguments

The standard QNX4 usage utility use will give a brief overview of the

command line options.  The syntax of the command line is:

Dq.ser port, mem [,cfg] [port, mem [,cfg] ...] [options]

port and mem are mandatory fields which identify the I/O port address

and shared memory addresses of the boards.  cfg is required to identify

a board as a COM/Xi, PC/Xem, PC/Xm or C/X system).  If cfg is

absent, the board is assumed to be a PC/Xe or PC/Xi (ISA and EISA

systems) or an MC/Xi (Micro Channel systems).  The format for port,

mem and cfg is as follows:

port	Hexadecimal I/O port address of DigiCHANNEL intelligent

board.  All boards must have a unique I/O port address, and

that address must not be used by any other device in the

system.

mem	Hexadecimal starting address for shared memory.  All boards

except COM/Xi and Micro Channel boards may share the

same memory addresses.  The designated shared memory must

not be used by any other device in the system.

cfg		Identifies DigiCHANNEL board type or number of

concentrators on a C/X system.  If this parameter is absent,

board is assumed to be a PC/Xe or PC/Xi (on ISA or EISA

platforms) or an MC/Xi (on Micro Channel platforms).  Values

for cfg are:

p		Specifies PC/Xem or PC/Xm system.

c		Specifies COM/Xi board.

0 .. 8	Specifies C/X system and indicates the number of

concentrators attached to each RS-422 port on the

host adapter (e.g. 31 means 3 concentrators on line 1

and 1 concentrator on line 2; 40 means 4 concentrators

on line 1 and no concentrators on line 2).

The options field allows you to change some of the default configuration

settings:

-b baud rate	Set initial baud rate for all ports.  Default: 9600 bps.

-n name		Prefix for port names.  Default: dty

-d data path	Location of driver data files.  Default: /etc/digi

-v		Verbose.  Full banner including copyright, board names

and settings will be output upon initialization of the

driver.  Default: non-verbose (silent).

-a		Select ALTPIN configuration for all channels.

ALTPIN switches the Data Set Ready (DSR) and Data

Carrier Detect (DCD) signals so that DCD is available

on pin 1 of 8-pin RJ-45 connectors (normally a 10-pin

RJ-45 connector is required to support DCD).  Default:

ALTPIN not selected.

Multiple options must be separated by a space, and

each option must begin with a dash (-);  thus, to

select ALTPIN and Verbose, use -a -v, not -av.

Command Line Examples

Dq.ser 300,d0000

Board at I/O port 0x300, shared memory starting at 0x000D0000.

If installed in an ISA system, a PC/Xe or PC/Xi board is expected.

If installed in a Micro Channel system, an MC/Xi board is

expected.  All other settings are left at default values.  Devices will

be named /dev/dty1, /dev/dty2, etc.

Dq.ser 324,d0000,p -b 1200

PC/Xem (or PC/Xm) board at I/O port 0x324, shared memory

starting at 0x000D0000.  Set initial baud rate for all ports to 1200

baud.

Dq.ser 328,d8000,21 -n digi

C/X host adapter at I/O port 0x328, shared memory starting at

0x000D8000.  Two concentrators on line 1 and one concentrator

on line 2.  Devices will be named /dev/digi1, /dev/digi2,

etc.

Error Messages

Failed to RESET the I/O PORT

I/O port address may be set incorrectly or conflict with another board.

Verify setting, or try another address.

Failed to load data from disk

Failed to STAT BIOS file

Failed to allocate local space for BIOS file

Failed to read from BIOS file

Failed to find DIGIPATH directory

Improper installation.  Driver and/or data files not found in expected

directory.  Default DIGIPATH directory is /etc/digi.  All files must

be loaded from the distribution diskette to this directory.  If a different

directory is used, it must be specified with the -d option in the Dq.ser

command line.

Failed to allocate CardBase Memory Selector

System call to map physical memory has failed.  Too many processes in

the system may be competing for memory handles.  The QNX kernel

may need to be reconfigured to permit more memory selectors.

Failed to BOOT the BIOS

BIOS on DigiBoard will not respond to commands from the driver.

Usually means that the driver cannot reliably read/write to the dual

ported RAM.  This could be due to a cache conflict (if the board is

addressed above 1 meg) or a conflict with another device's memory.  It

could also mean that the wrong address was given in the Dq.ser

command line.

Failed to BOOT the uploaded FEPOS

Card type specification may be incorrect.  Could also indicate an

incorrect memory address specification or a dual ported memory

conflict.

Failed to read Initialization Information

Shared memory conflict.  Try a different address.

Failed to set up Event Queue

System resource exausted.