Installation Guide

for

DigiBoard Intelligent Communications Boards

SCO UNIX

DBI 92000154A

Software Installation

Installation of the DigiBoard device drivers is a two-part procedure:

First, the DigiBoard MenuPort Interface (MPI) package is installed onto

your hard disk. Second, the MPI package is run and the appropriate device

drivers for your system are installed and configured.

Software changes more rapidly than printed documentation

can keep up.  For this reason, some of the screens or prompts

may not appear exactly as shown.

Installing the MenuPort Interface (MPI) Package

SCO UNIX

1.	Log in as super-user (root).

2.	Run custom.  Select "New Product".  Insert the DigiBoard MenuPort

Interface device driver diskette when asked to do so.

3.	When the MPI installation is complete, quit custom.

Running the MenuPort Interface (MPI) Program

To run the MenuPort Interface program, log in as root and enter:

mpi

After the restricted rights legend, you will see the Main menu.  Items in

this menu are:

*   Config, which lets you install, change and remove device drivers for

specific boards and operating systems.

*   System, which lets you build a new kernel, update installed device

drivers and create an image of the kernel.

*   Monitor, which runs DigiBoard Port Authority (DPA), a software diagnostic

tool that allows users to easily monitor the status of the Front End

Processor/Operating System (FEP/OS), which runs on all DigiBoard

intelligent serial port boards. The DPA software also shows the status

of the individual ports. With DPA, system managers can tell instantly

if a board is no longer working, if its performance level decreases, of

if a port is non-functional, enabling them to work more productively and

solve system errors more quickly. DPA also provides a loop back test to

confirm that the board can transmit and receive data.

*   Notes, which displays the release notes for the supported device drivers.

MPI features context sensitive help screens; press <F1> at any

time for an explanation of the currently displayed information

and choices.

Device Names SCO UNIX

PC/Xe, PC/Xi, MC/Xe and MC/Xi Boards

Format used for port names: /dev/ttyibp, /dev/pribp

ttyi	Base name.

pri	Base name for DigiPrint transparent printing.

b	Board number (1-4).

p	Port identifier:

a-p     (lower case) are used for non modem control ports.  These

ports do not require DCD (Data Carrier Detect) to be asserted

before they can transmit data.  Primarily used with directly

connected terminals and printers.

A-P     (upper case) are used for modem control ports.  These ports

require DCD (Data Carrier Detect) to be asserted before they can

transmit data.  Primarily used with modems.

For example, Port 4 Board 2 will be /dev/ttyi2d. The same port with

modem control enabled would be named /dev/ttyi2D.

C/X, EPC/X and Xem Systems

Format used for port names: /dev/ttycp, /dev/prcp

tty	Base name.

pr	Base name for DigiPrint transparent printing.

c     	Concentrator or EBI module letter I.D:

a-z     (lower case) are used for non modem control ports.  These

ports do not require DCD (Data Carrier Detect) to be asserted

before they can transmit data.  Primarily used with directly

connected terminals and printers.

A-Z     (upper case) are used for modem control ports.  These ports

require DCD (Data Carrier Detect) to be asserted before they can

transmit data.  Primarily used with modems.

p	Port number on the individual concentrator or EBI module (01-64).

For example, Port 4 on concentrator or PORTS module "b"  will be

/dev/ttyb04.  The same port with modem control enabled would be named

/dev/ttyB04.

Enabling the New Ports SCO UNIX

1.  Connect terminals to the ports (using a null modem, if necessary) and

test the connections to each terminal by entering the following command

for each port added:

date > /dev/ttyi1a

(Assuming the terminal is connected to ttyi1a.)

(Please note that in the above, the "date" command is used as a simple test,

to provide text output that can be redirected; there is no other significance

to "date" in this test.)

*  If the date appears on the terminal's screen, the device is properly

connected.

*  If the date does not appear on the terminal's screen, then that terminal is

not receiving data; check the power, cables, connections, etc.

*  If nonsense characters are printed on the terminal's screen, check the baud

rates, data bits, stop bits, and parity setting on your terminal (UNIX

default parameters are 9600 baud, 8 data bits, 1 stop bit, and no parity).

Another test you can perform will verify that a port can both transmit and

receive data:

Connect the port's transmit and receive lines together (pins 2 and 3 of

a DB-25 connector, or the middle two pins of an RJ-45 connector), and

enter the following commands:

cat </dev/ttyi1a &

cat /etc/termcap > /dev/ttyi1a

The first command runs in the background, and directs all input from

ttyi1a to stdout (your screen).  The second command transmits the

termcap file to ttyi1a.  If the port is working, the data is sent out

the port, received back by the same port and displayed on your screen.

If the port fails, check another port.  If more than one port fails,

the problem is probably due to memory contention, rather than defective

hardware.  Try reinstalling the device driver with a different memory

start address. Once you can redirect output to a terminal with the test

above, perform the following steps to enable that port.

If the port tests above work, but you still can't respawn a

getty, it is probably because DCD is not asserted.

Remember that modem controlled ports (where the port ID is

a capital letter; e.g. ttyi1A) must have Data Carrier Detect

(DCD) asserted before they can function.  If DCD is not

connected to an active output line of the peripheral device, it

must be defeated, either by "tying it high" (e.g. connecting it

to the same port's DTR line) or by issuing the command

ditty forcedcd ttyi1A (assuming that the port in question is ttyi1A).

Another way to bypass the DCD requirement is to use non

modem control devices (where the port ID is a small letter;

e.g. ttyi1a).  Non modem control devices function

identically to modem control devices, except that they do not

require DCD to be asserted in order to function.

2.  The Terminal Control Database must be updated to include the new devices.

Add the DigiBoard device information using the following SCO UNIX

sysadmsh command sequence:

Accounts*Terminal:Create

Fill in the required information on the form displayed.

3.  To activate a port for use with a terminal, enter the following command:

enable ttyi1a

The above command will cause a login prompt to be sent to the terminal

connected to ttyi1a.  To activate the other ports, repeat the enable

command with the appropriate port name.

Serial Printers

To prepare a port for connection to a printer, enter the following command:

ditty printer [baud rate and flow control options] ttyi1a

This command forces the specified port to stay open, thus preserving any

"non-sticky" serial line characteristics. This command is recommended for

any port that has a printer connected to it. The command may be placed in

one of the following files so that it is automatically executed whenever

the system is booted. SCO UNIX:  /etc/rc.d/8/userdef

EXAMPLES:

The commands shown below result in 9600,8,1,N, with software flow control.

SCO UNIX:

ditty printer ixon opost onlcr ttyi1a

Uninstalling the MPI Software Package

Uninstalling the MPI software package is a two-part procedure.  First, you must

remove any device drivers that were installed via the MPI software, then you

remove the MPI software package itself.

Removing Device Drivers

1.  To run the MenuPort Interface program, log in as root and enter: mpi

2.  From the Main menu, select Config.

3.  Now select Remove, from the Config menu.

4.  From the Pick List, select the device driver you wish to remove.

Repeat this step for all installed device drivers.

Removing the MPI Software Package SCO UNIX

1.  Log onto the console as super-user (root), and enter the following

command: custom

2.  When the operating system menu asks, choose REMOVE.

3.  Select mpi.  The software will respond with:

Removing DigiBoard MenuPort Interface Package.

4.  Quit custom, and remove the perms file: rm /etc/perms/mpi

5.  Now perform a system shutdown and reboot to activate the new kernel.

----------------------------------------------------------------------------

Error Messages

The following error messages are generated by the driver:

***WARNING*** No board found at port 0x320, check switch settings.

or

***WARNING*** PC/Xi at port 0x320 not resetting, check switch

settings

What it means:

The board doesn't respond to reset.

Action to take:

Make sure the board is fully seated in the computer's slot.

Make sure the board's jumper settings and the device driver's settings match.

Potential hardware problem.

***WARNING*** Driver memory allocation failed for PC/Xi at port 0x320.

What it means:

The operating system would not allocate memory to the driver.

Action to take:

Adjust kernel resources.

***WARNING*** No memory at 0xd8000 for PC/Xe at port 0x320, check

switch settings

What it means:

The driver cannot read the board's dual ported memory.

Action to take:

Make sure the board is fully seated in the computer's slot.

Make sure the board's settings and the device driver's settings match.

Make sure the jumpers or DIP switches are in their proper positions.

Make sure that there is no memory contention with other devices.

Make sure you haven't selected an 8K address for a 64K board.

Potential hardware problem.

***WARNING*** PC/Xi at port 0x320 failed diagnostics. Error #(N1,N2).

What it means:

The driver encountered an error executing on-board BIOS.  N1 and N2 are

status bits at the time of the failure.

Action to take:

Make sure the jumpers or DIP switches are in their proper positions.

Make sure there is no memory contention with other devices.

Potential software problem.

Potential hardware problem.

***WARNING*** PC/Xi at port 0x320 failed BIOS function #X. Error #(N1,N2).

or

***WARNING*** FEPOS for PC/Xe at port 0x320 not functioning. Error #(N1,N2).

What it means:

The driver encountered an error executing on-board FEPOS.  X is the BIOS

function that failed, N1 and N2 are status bits at the time of the failure.

Action to take:

Make sure the jumpers or DIP switches are in their proper positions.

Make sure there is no memory contention with other devices.

Potential software problem.

Potential hardware problem.