| Field Note: 108 |  | Distribute freely |
| --- | --- | --- |

Subject:  	928Movie VL-Bus and IBM SLC 2 66

Date created:	May 27, 1994

Product: 	928Movie VL-bus

A recent problem arose on the installation of a 928Movie VL-Bus card in an Energy Smart Green PC Plus by Alaris.  The main processor in this PC is an IBM SLC 2 66 (it looks like a 386sx with a heat sink but is in actuality a 486).

Symptoms:

When going into Windows using the 928Movie driver the system would lock up after the initial Windows 3.1 logo and required a cold boot to re-start.  When a non 928Movie driver was used (i.e. VGA) the system was fine.

If an Intel 486/66 was placed in the Ziff-socket and the SLC 2 66 disabled the problem went away.

Reason:

The SLC 2 66 cannot deal with addresses above 16MByte.  This should create a problem for just about any VL-Bus graphics card (and probably other types of VL-Bus cards).  The Intel 486 chip does not have this limitation.  However, the i486 is not marketable as an energy conserving chip.

Solution:

There are 2 solutions to this problem:

Solution #1 -- In the system.inI file in the windows directory there is a [928Movie] section.  In this section there are two lines that allow the VL-Bus to use linear super high addressing:

ApertureBase=0

HighMemoryBase=0

Remark these 2 lines out by placing semi-colons before them (i.e.  ;ApertureBase=0 ).  Save the file and then power off the PC.  The next time Windows is run the driver should use local bus addresses below 1MB.

There may be a slight decrease in performance that is shown by the WinBench 4.0 graphics benchmark.

Solution #2 -- By disabling the SLC chip and placing an Intel 486 chip in the Ziff-Socket this problem is not present.  The Intel chip can handle VL-Bus super high linear addressing.  On the Alaris Energy Smart Green PC Plus (using an IBM Hurricane or Leopard motherboard) this is accomplished by placing a jumper pin on JP12 across pins 1 and 2, and placing a jumper pin on JP11.  This of course will differ from computer to computer.  Consult the manufacturer of the PC.

Peiter Zatko, Steve Everitt

July, 26