| Field Note: 230 |  | Distribute freely |
| --- | --- | --- |

Subject:  Apocalypse 3D and/or Captivator PCI -  Award BIOS and S3 968 Problem

Date Created: July 30, 2026

Product: Apocalypse 3D and/or Captivator PCI

Summary:

A number of customers are reporting problems with their Apocalypse 3D and or Captivator PCI causing a system reset or lock-up. These symptoms are caused by a system incompatibility between motherboards fitted with an Award BIOS and the S3 968 or S3 868 graphics controller.

Problem:

During PCI initialisation, graphics cards based on the S3 868 and 968 chips will request only 32Mbytes of virtual memory rather than the 64Mbytes they actually require. AWARD BIOS will allocate the requested 32 Mbytes to the graphics adapter and locate the add-in card’s memory range just above that, therefore within the memory range used (but not declared) by the S3 graphics card. Other BIOSes like MR-BIOS have a ‘workaround’ built in which will ignore the faulty memory request issued by those S3 adapters and allocate 64Mbytes by default, therefore not exhibiting the problem.

Result:

Attempts to use the Apocalypse 3D or Captivator PCI card will fail, typically causing the system to hang or reset. With the Captivator PCI card the error message ‘BT848 Capture Driver BT848 : Failure loading VXD driver’ may occur instead.

The problem is due to the illegal memory range overlap caused by the wrong memory size request of the S3 adapter.

Solution:

Manually changing the memory allocation for the add-in card to a range outside the 64Mbytes used by the graphics card will solve the problem. The memory reallocation can be done manually within the Windows 95 Device Manager as follows:

1) Go to the Control Panel, run ‘System’ and select the Device Manager

- Locate the affected device. For example, Apocalypse 3D users would select Sound, Video and Game Controllers and highlight ‘Apocalypse3D’

- Select ‘Properties’ and then ‘Resources’

- Remove check "Use Automatic Settings" to allow manual setup

- Choose Memory Range I from the list

6) Click on "Change Setting" and alter the range to FFBE0000 - FFBEFFFF

- Choose Memory Range II from the resource list

- Click on "Change Setting" and alter the range to FF400000 - FF7FFFFF

- Confirm everything by clicking OK

- Restart your PC

This assumes that there is no memory allocation conflict due to devices present at FF400000 - FFBEFFFF. One should cross-check for conflicts by checking the allocated memory ranges for other PCI devices by double-clicking the computer icon in the Device Manager and choosing "Memory".

This will allow you to run applications successfully for your add-in card and it is normally not necessary to reconfigure the memory settings again since Windows95 will remember the new settings every time it starts up and reallocate the memory automatically.

VideoLogic Support Team

July, 26