| Field Note: 37 |  | Distribute freely |
| --- | --- | --- |

Subject:  Why DVA-4000 /ISA cannot support more than 8-bit graphics

Date Created: July 30, 2026

Product: DVA-4000/ISA

The DVA-4000 was designed in the late '80s to coincide with the launch of IBM's new range of PS/2 machines, their new bus architecture (MCA) and the new graphics standard they had defined (VGA). Standard VGA was specified up to a maximum resolution of 640 x 480 in 16 colours, and a maximum colour depth of 256 colours (8-bit) at 320 x 240 resolution. The DVA was designed to match this architecture.

When VGA was adopted on the ISA bus via add-in cards, a feature connector was defined to allow the same digital graphics data to be provided from these cards as was available from the MCA video (graphics) bus. This feature connector provides 8 bits of digital graphics along with synchronisation information, and this is the ONLY graphics input to the DVA-4000 for ISA machines.

For this reason, if the graphics card is set to run at colour depths higher than 8-bit, the DVA-4000 does not have access to all the colour information required, and simply gets whatever the card passes to the feature connector - which is not defined in the specification of the feature connector for modes greater than 8-bit. The graphics from the DVA therefore do not match the graphics from the super VGA.

To allow DVA to work with higher bit depths would require a major redesign - taking the graphics information in the analogue domain direct from the super VGA.

Finally, to pre-empt the question "how come the DVA-4000 for the Macintosh can handle 16- and 24-bit graphics", the answer is that the feature connector on the SuperMac graphics card used was specified by VideoLogic, and includes not only 8 bits of digital graphics information, but the complete analogue signal as well. Unfortunately, we didn't have control of the specification when IBM originally came up with the ISA feature connector!

Colin Crawford

July, 26