| Field Note: 114 |  | Distribute freely |
| --- | --- | --- |

Subject:  DVA-4000/ISA and compatibility with modern SVGAs

Date Created: July 30, 2026

Product: DVA-4000/ISA

Some concern has been voiced recently about lack of DVA-4000 support for more modern graphics cards. There is one very important point to make clear:

THERE IS NO INTRINSIC REASON WHY DVA-4000 SHOULD NOT WORK WITH VESA OR PCI LOCALBUS GRAPHICS CARDS

However...

There are a number of reasons why DVA-4000 might not be compatible with a particular VESA or PCI localbus graphics card in a particular PC. Unfortunately, the chance of one of these reasons occurring has increased with the advances in graphics technology over the past few months.

The following is a list of issues (becoming more prevalent) which can cause incompatibility:

-	Some newer VGAs do not provide a feature connector (possibly because of cost, Radio Frequency Interference issues, PCB space).

-	Some graphics cards (or chips) actually disable output to the feature connector when run in graphics modes higher than standard VGA (e.g. Compaq QVision).

-	Some newer graphics cards do not support the refresh rates supported by DVA-4000.

-	Some PC systems when running a VESA Local Bus or PCI graphics card, do not propagate the palette update commands onto the ISA bus. In this case, the shadow palette on the DVA-4000 is not made aware of palette changes, and so the colours are all wrong.

Colin Crawford

July, 26