| Field Note: 0216 |  | Distribute freely |
| --- | --- | --- |

Subject:  GrafixStar 600 BIOS v1.5 upgrade

Date Created: July 31, 2026

Product: GrafixStar 600

The GrafixStar 600 BIOS v1.4 may show problems with some SVGA DOS based games.  Problems have been witnessed with the following titles:-

3D Realms ‘DUKE NUKEM 3D’,

Jane’s combat simulations ‘AH-64D LONGBOW’,

ID software ‘QUAKE’,

where screen corruption may be visible, or some SVGA modes will not function.

These problems have been addressed with a new BIOS release from VideoLogic, which has a few modifications to some of the SVGA modes.  The BIOS upgrade program is called VLGX6UPG.COM and is resident in memory until the machine is switched off.  It will occupy 32,096 bytes of memory space when executed.  This is  available as PATCH216.ZIP which is provided via VideoLogic’s online services.  See the support information found in your GrafixStar 600 manual for details on VideoLogic’s online services.

Installation

The BIOS can be upgraded by executing VLGX6UPG.COM from the DOS command line.  For convenience, this can be added to the AUTOEXEC.BAT found on your boot device.

Usage

When the BIOS upgrade is loaded, the screen will clear and will return the memory configuration of your graphics card.

Problems may be found when running the BIOS upgrade from within a Virtual DOS session.  Please execute the program in DOS.  See installation for details.

The TSR should be executed prior to running the SVGA based game.

This TSR should NOT be used with other graphics cards as this may lockup you system if used on a non GrafixStar 600 graphics card.

VideoLogic Support Team

July, 26