| Field Note: 204 |  | Distribute freely |
| --- | --- | --- |

Subject:  Award BIOS (Intel EtherExpress PRO/100 LAN Adapter)

Date Created: July 30, 2026

Product: GrafixStar 700

Award Software International have issued a Windows 95 Technical Bulletin (October 5,1995) stating a number of problems experienced with the Award BIOS and Windows 95 when implemented on some specific OEM motherboards.

One of the problems concerns the misreporting of Plug and Play functionality. This problem can cause an incompatibility between the GrafixStar 700 and other PCI devices such as the Intel EtherExpress PRO/100 LAN Adapter. Such an incompatibility lead’s to the system hanging during the initialisation phase of Windows 95 that occurs after boot-up.

To rectify the problem the Memory Range assigned to the Intel LAN Adapter must be user configured. To configure the Memory Range, the Adapter Card must be installed, thus necessitating that the GrafixStar 700 drivers are not currently active. It is recommended that the S3 ‘VISION 968’ drivers are used.

To user configure the Memory Range assigned to the Adapter Card -

- Load the ‘System Properties’ (right click on ‘My Computer’ and select ‘Properties’ from the Pop-Up menu or select ‘System’ from within the Control Panel).

- Select the Tab ‘Device Manager’ to view a list of current devices.

- Double Click on ‘Network adapters’ from within the list. A sub entry for the Intel 82557 based PCI Ethernet card should now be visible.

- Highlight the PCI Ethernet entry and click the ‘Properties’ button.

- From within the PCI Ethernet window select the Tab ‘Resources’. You should now be able to see the automatically assigned Memory Ranges.

- Click the check-box for ‘Use automatic Settings’ to Off.

- Highlight one of the two ‘Memory Range’ entries and click the button ‘Change Settings’. You should now see an Edit Memory Range dialogue box.

- The first digit of the boundary addresses of the Memory Range allocated to the card will probably begin with ‘F’. Edit the range so that the first digit for both address boundaries begin with ‘E’.

For example, change -

Memory Range :  F2100000 - F2100FFF

Memory Range :  F2000000 - F20FFFFF

to -

Memory Range :  E2100000 - E2100FFF

Memory Range :  E2000000 - E20FFFFF

- Repeat the last two steps for the other Memory Range entry.

- Close down System Properties and restart Windows. After verifying the Intel LAN adapter card is still functioning reinstall the GrafixStar 700 drivers.

Dave Marsland

July, 26