| Field Note: 208 |  | Distribute freely |
| --- | --- | --- |

Subject: Lock-up when using interlaced modes on the GrafixStar 700

Date Created: July 31, 2026

Product: GrafixStar 700

Interlaced modes on the GrafixStar 700 will cause your system to lock up or drop back to DOS when starting Windows 3.1 or 3.11 if you have also installed a Sound Blaster card.

The reason for this is that the GrafixStar drivers turn the software cursor on in interlaced modes by default. To force a hardware cursor to be used, add the following line to the [videologic svga] section of your SYSTEM.INI file

HardwareCursor=On

Colin Crawford

July, 26