| Field Note: 245 |  | Distribute freely |
| --- | --- | --- |

# Subject: Loss of sync with Compaq PCs

# Date Created: April 03, 1997

# Product: GrafixStar 700

Using GrafixStar 700 with certain Compaq PCs can cause loss of sync under DOS and Windows. This is because the GrafixStar 700 BIOS fails to set up one of the registers during a mode set, which causes the wrong clock to be used and so the monitor loses sync. Compaq PCs known to be affected are the Compaq Deskpro 4000 and 6000 series and the Compaq Prolinea 5120 series.

Patch 245 is a Terminate-Stay-Resident program (TSR) which fixes this problem. To install Patch 245, copy the file GX7CMPQ.COM to the root of your hard disk and add the following line to AUTOEXEC.BAT (assuming C is the hard disk from which you are running Windows):

C:\GX7CMPQ.COM

VideoLogic Support Team
April, 97