| Field Note: 42 |  | Distribute freely |
| --- | --- | --- |

Subject:  Improving MediaSpace throughput - more advice

Date Created: July 29, 2026

Product: MediaSpace

| 486-33 | EISA | Everex | RAID 5 system w/ 3 drives | EISA SCSI II fast |
| --- | --- | --- | --- | --- |
|  | local |  |  |  |
| 486-33 | ISA | Dell | Dell internal | Dell |
| 486-33 | EISA | Everex | Micropolis and/or Rare | BusLogic 747S |
| 486-50 | EISA |  | Micropolis 2112 11ms 1.2 Gig | AMI fast SCSI w/ 4 meg |
| 486-50 | ISA | Dell | Quantum 425 Meg SCSI <15ms | Adaptec 1542 |
| 33DX2 | EISA | HP | HP SCSI 9.8ms 1 Gig | Adaptec 1740 |

Here are some hints people came up with along the way to improve performance:

1. Turn off write-caching in smartdrv ("smartdrv c").

2. Format the hard drive with 4K blocks instead of512 byte blocks (use low-level format).

3. Substitute BusLogic caching software for smartdrv.

4. Get a 10 MB/sec subsystem instead of a typical 5.7 MB/sec subsystem.

5. Get a 5400 RPM drive with <10 ms access time.

6. Try shadowing controller card address space (C800 or CC00 usually).

7. Get controller software to update the FAT less frequently.

Colin Crawford

July, 26