| Field Note: 51 |  | Distribute freely |
| --- | --- | --- |

Subject:  Maximizing MediaSpace Throughput

Date Created: July 30, 2026

Product: MediaSpace

This field note is intended for anyone who cares about maximizing the quality of

MediaSpace movies, including:

End users

Resellers and sales people

Technical support

Windows application developers

The press.

This is version 1.4 of this document.  The topic is very complicated, and some of the

information in the appendicies is very detailed.  Version 2.0 of this document

is scheduled for July 1.  Please send comments and corrections to:

Internet: stever@videologic.com

Fax: 408 746 3630 (address fax to VideoLogic Developer Relations)

See also Field Note 1: "Video Compression Ratios" and Field Note 2: "Host Machine Performance."

TABLE OF CONTENTS

1. Spreadsheet: results and recommendations summary

2. Why Throughput is important

3. Why recommendations will change

4. A challenge

5. Recommendation: disk arrays

6. Recommendation: EISA bus

7. Recommendation: Fast SCSI II drive / controller

8. Recommendation: 486-50 CPU

9. Recommendation: Use a defragger

10. Recommendation: Config.sys settings

11. Recommendation: EISA controller settings

12. Recommendation: Big, fast disk characteristics

13. Warnings: Fast, new products can be unreliable

14. Appendix: Data Rate Calibrate technical

15. Appendix: How 3rd parties might improve throughput

16: Appendix: Further reading

17. Appendix: Spreadsheet: Maxtor's Benchmarks

2. WHY THROUGHPUT IS IMPORTANT

MediaSpace uses JPEG, a lossy compression method.  The less compression

MediaSpace is required to use, the less loss is introduced into movies

and the higher their quality.  If a computer can provide higher

throughput, MediaSpace parameters can be increased, producing better

quality.

A typical Dell 486-33 gives 300K/sec movie throughput off its internal

hard drive.  This translates into very good quality movies.  However, the

resultant quality is not the same as the VCR, camcorder, or laserdisc

source (especially if you single step through and examine individual

frames).  A jump to 400K/sec provides a large jump in quality.  Steps

up to 500K/sec and even 600K/sec are also very, very noticeable.  And

Video Editing afficionados want even higher data rates.

If you would like to improve the quality of movies you see on

MediaSpace, you may want to invest in a faster hard drive subsystem.

If you plan to use MediaSpace to edit movies, you should acquire a

Gigabyte or larger hard drive.

For compression hardware, it is usually harder to record (compress) than

play back (decompress).  Thus users can typically record movies on a

fast machine, but get away with playing them on a slower machine.

Issues of hard disk subsystem throughput are very complex.  Recent

articles in the press have contradicted the advice given by controller

and drive makers.

3. WHY RECOMMENDATIONS WILL CHANGE

Computer, controller, and hard drive manufacturers are always improving

their wares.  Thus any recommendations made may be out of date

within three months time.  In addition, the number of permutations of various

parameters that affect system performance is staggering.  When

benchmarking, a user/installer should experiment with about 10 different

combinations of parameter settings before they can feel confident

they've found the highest performance combination.  Arriving at these

settings is currently more of an art than a science.  Some

configurations may be too much for one particular component, and may

cause the system to temporarily mis-behave until parameter values are

lowered.  This an unfortunate side-effect of striving for high throughput.

A trial and error methodology should be followed.  A table of

configurations should be prepared so that conclusions can be drawn

about what helps and what doesn't.  It will be a good idea to read

the drive and controller manuals carefully, and it may be a good idea to

call the hard drive controller maker's technical support line for some

suggestions as to the best settings.

In addition, we have only checked a few configurations for MediaSpace

throughput.  There are other very fast systems available which we

haven't measured.

Throughput also depends upon the software application and

resultant pattern of hard disk i/o.  MediaSpace recording i/o is

characterized by repeated writes of huge sequential chunks; settings

optimized for MediaSpace may not be as effective for data base accessing

which involves much more random access.  The MediaSpace benchmark gives

different results than some other benchmarks.  The MediaSpace benchmark gives a

good indication of hard disk subsystem performance when recording Motion JPEG

movies; other benchmarks measure different things.

Popular benchmarks include: NCR, BIOSBNCH, CORETEST, PC LABS, BYTE,

NORTION SI, and BAPCo.  See the spreadsheet at the end of this paper (results

compiled by Maxtor) for these results.  Some of these benchmarks test the disk

reading the same track over and over again; which is an unrealistic model.  Note

that the Maxtor 1240S wins overall, but the Quantum drive is the clear winner for

the long sequential writes in the NCR benchmarks, which may be a good indication

of MediaSpace performance.

Fast SCSI II drives are rated at 10 MBytes/sec.  However, no application

ever gets close to this rate.  This is because there is a lot

of overhead involved with getting information off the disk.

In the future, operating systems such as Windows/NT will provide

increased file system performance and eliminate some of the overhead.

However, MediaSpace has a theoretical throughput limit of about 1 MByte/sec,

so no hard drive subsystem can ever overcome this limit.

4. A CHALLENGE

VideoLogic will send users a nice cheese basket for new Data Rate Calibrate records for each of the three buses:

ISA, EISA, VESA Local Bus.

So even if you get a lowly 480K/sec, if your machine has an ISA bus, you win!

If you feel you have a new record, contact VideoLogic Technical Support.  Before you call, run Data Rate Calibrate 3 times in a row and take down all three results (calibration results vary with each run depending on how the hard drive is laid out).  Technical Support will take detailed configuration information from you, including CPU, Bus, Motherboard, Drive, Controller, config.sys settings, controller settings, and any clever tricks you may have figured out.  The goal is to build up the spreadsheet so that all MediaSpace users can benefit.

Two $100 cheese baskets have already been sent (they are filled with other goodies as well) and another 8 are waiting to be claimed.

5. RECOMMENDATION: DISK ARRAYS

We heard of a record setting MediaSpace benchmark of 700K/sec on a

3 disk array system.  However, we do not have enough information to

recreate the configuration.  We would suggest that a disk array

can beat a single drive.

6. RECOMMENDATION: EISA BUS

There are two "pipes" involved with MediaSpace and hard drives.  The

main pipe is the host bus which shoots data around between the 486

cpu, the hard disk controller, and MediaSpace.  The second pipe connects

the hard disk controller with the hard disk.  Let's start with the

host bus.

The current PC buses are ISA (Industry Standard Architecture), EISA

(Extended ISA), and VESA Local Bus.  If you buy a machine with a fast

bus (EISA and/or Local Bus), you'll need to acquire a matching EISA or

Local Bus controller to squirt the data between the controller and the 486

at the faster bus rate.  Slower ISA controller cards will still run in EISA slots,

but will limit performance.

ISA runs at less than 5 MBytes/Sec, EISA runs at 33 MBytes/Sec, and VESA

Local Bus runs at 66 MBytes/sec.  In the future, PCI Local Bus machines

will appear with a bus speed of 265 MBytes/Sec.  The current

fastest bus is VESA Local; however, MediaSpace benchmarks on the current

batch of local bus hard drive controllers run slower than on EISA

controllers.  An EISA controller product manager confirms: "The UltraStore

34F local bus controller runs at about 60% of the speed of our EISA

controller."  So the current recommendation is to go with the EISA bus;

however, VESA Local Bus controllers should surpass EISA controllers

within the next few months (Local Bus configurations will

be more expensive).  There is currently a lack of VESA Local Bus controllers

which can "bus master," IE burst data at 66 MBytes/sec.

The cost differential between ISA and EISA machines is about $500 these

days, well worth the difference if you are serious about using

digital video.

VL sales people want to be able to recommend very cheap standard ISA

bus configurations from the top PC manufacturers for MediaSpace.

Unfortunately, the the current results show that it is better to buy a

specially configured computer:

- Standard 1 gigabyte PC models from the top makers come with

previous generation hard drives which are significantly

slower than the state of the art.

- EISA bus provides a 50% performance improvement over ISA.

7. RECOMMENDATION: FAST SCSI II DRIVE / CONTROLLER

Current drive specifications are: IDE, ESDI, SCSI I, SCSI II, and

Fast SCSI II.  IDE is more popular in simple computers, but

multimedia users need SCSI to access more peripherals.  SCSI is

much faster for reading or writing large sequential chunks than

IDE or ESDI.  ESDI is dying because of SCSI.  Be sure to get a

"bus mastering" controller.

Here are the max speeds: <Sorry, numbers are still forthcoming.>

IDE

ESDI

SCSI I

SCSI II: Can be 5 MBytes/sec or 10 MBytes/sec.  Ask for Fast SCSI II!

FAST SCSI II: 10 MBytes/sec

SCSI II Wide is another step past even Fast SCSI II.

Local computer dealers know a lot about fast drives and controllers.

Controller makers and drive makers can also be contacted:

CONTROLLERS:

Adaptec: Milpitas, CA  408 945 8600

UltraStore: Fremont, CA

AMI: Norcross, GA

DTC: Fremont, CA

BusLogic

MicroNet: Irvine, CA

Always: Westlake Village, CA

DRIVES:

Quantum: Milpitas, CA  800 624 5545, 44/344-780144

Seagate: San Jose, CA  408 452 0400

Micropolis: Chatsworth, CA  818 709 3300, 44/734-751315

Fijitsu

Maxtor: San Jose, CA  408 435 7884

AdStar: San Jose, CA

Liberty

MicroNet: Irvine, CA

PLI: Fremont, CA  510 657 2211

Storage Dimensions: Milpitas, CA

8. RECOMMENDATION: 486-50 CPU

The 486-33 DX2 "clock doubled" CPU runs at 66 MHz internally and 33 MHz

externally.  MediaSpace is a powerful parallel processing machine

with 3 special purpose CPUs on board; it does not rely on the 486's

internal processing for many calculations.  Rather, the 486's ability to transfer

data to and from external cards is more important.  Thus, a 486-50 DX is a better

machine than a 486-33 DX2 for MediaSpace.

9. RECOMMENDATION: USE A DEFRAGGER

Let's say you are a multimedia production house.  You will tend to record

your MediaSpace movies in a big batch.  On the night before each recording

session, defragment your hard disk.  This helps performance.  The

MediaSpace benchmarks that follow are on brand new hard drives where

the disk has one huge chunk of free disk space.  In this situation, the

next free block of disk space is always right after the last.  On a

fragmented disk, the drive subsystem has to seek to locate the next

free block, reducing performance.

Popular defraggers include:

Norton Utilities / Speed Disk

PC Tools

Paul Mace Utilities

10. RECOMMENDATION: CONFIG.SYS SETTINGS

The best approach is to experiment with these settings to discover

the highest throughput.

A. SCSI drives come with ASPI drivers.  Put these in as the first thing

in config.sys.  If you don't have these in, you're probably running

at half speed.  ASPI is a standard software interface for talking to

SCSI devices.

B. The use of smartdrv is puzzling.  It isn't always that smart.

We recommend not using doublebuffering and turning write caching off:

config.sys:

device=c:\windows\smartdrv.exe

autoexec.bat:

\windows\smartdrv.exe c d

The "c d" turns write caching off on drives c and d.  Controllers and

fast drives both do such an excellent job of caching in hardware that

software caching is unnecessary.  In fact, smartdrv's software write

caching actually slows things down.  This is because software is

duplicating the work of the hardware caching.  By buffering data

in software, smartdrv causes data items to be copied around in memory

unnecessarily.

Doublebuffering in config.sys looks like:

device=c:\=windows\smartdrv.exe /doublebuffer

This creates buffering in both low and high memory, which is very

bad for performance.  Note that everytime you re-install Windows,

you need to fix config.sys.

Stacker tech support: "smartdrv is dangerous.  Most caching programs

use a first in - first out discipline which is pretty safe.  In order

to be real fast, smartdrv uses a different strategy.  Sometimes smartdrv

will update your FAT but not update the actual file for a while.  If

you get a hard crash inbetween, you end up with a disk problem."

C. Don't use Stacker.  It slows things down.  Stacker uses one type

of compression that is very different than JPEG compression.  Stacker

cannot shrink JPEG movies because they are already very compressed.  What

stacker does is introduce unwanted overhead.

11. RECOMMENDATION: EISA CONTROLLER SETTINGS, ETC.

The three most popular types of EISA bios are Phoenix, MCS, and AMI.

All have EISA configuration utilities which allow for the installation

of EISA boards with settings set in software rather than with

jumper settings.  This is nice in theory; however, controller makers

have abused this privledge by creating too many changeable options

on their cards.

The following shows some examples of settings you may

face.  Not all will be available for a particular hardware configuration.

In your computer setup program, you may be able to bump up performance

a bit:

A. You can "shadow" your controller.  This improves its performance.

If your controller sits at C800 and you have AMI Bios, do the following:

Advanced CMOS Setup/ Enable: Adaptor ROM Shadow C800,32K

B. Fast SIMMs (which increase your computer's RAM from 8 MBytes to 16 MBytes

for instance) are probably worth it.  There may be some setup settings

to tweak such as: DRAM Speed, DRAM Write CAS Pulse Width, DRAM Interleave,

Cache Write BAck, Cache Write Cycle, Cache Burst Read Cycle, Bus Clock

Speed.

In your EISA computer EISA Configuration Utility (ECU), check the

following parameters:

C. EISA Bus Burst - enable this

This means data can burst from the controller to the 486 at 33 MBytes/sec.

D. Sync Negotiation - enable this

This controls the speed of data from disk to controller:

Asynchronous:  2.0 MBytes/sec

Synchronous:  5.0 to 10.0 MBytes/sec

E. Maximum sync data rate (from disk to controller) allowed:

10 MBytes/sec is better than 5 MBytes/sec.

F. We recommend you increase "read ahead" if this parameter is available.

G. Low overhead when issuing instructions is good (say Adaptec).  Make

sure to turn Adaptec's Enhanced mode on.

H. If you have only one drive, disable "disconnect."  This may improve

performance by up to 15% (says Adaptec).

I. Increasing "bus on" from 11 to 15, and decreasing "bus off" from 4 to

1 yields a 20% performance improvement (says Adaptec).  Be warned, this may

prevent your computer from booting.

J. General advice:

- Lots of cache on controllers is good.

- 32 bit access is good.  16 bit access is bad.

K. The controller may come with an installation program that allows for

a special low-level format.  We suspect that all gigabyte drives have

a 1 to 1 interleave.  However, a low-level format that could produce

a blocksize of 4K versus the typical 512 bytes could provide a huge

performance boost.  However, 4K blocks mean that 33 byte files

take up 4K bytes.  MicroNet has a special low-level format.  Future

Domain is also rumoured to have this capability.

Industry experts do not all agree on block size issues.  Some people

have claimed that the DOS high level format generates 4K blocks already.

-----------------------------------------------------------------

Fast controllers retail in the $450 price range.

SCSI controller manufacturers call their controllers "host adapters"

because SCSI drives do much of their own controlling.  However, SCSI

controller boards control SCSI drives from a higher level of

abstraction.  Therefore, the person who came up with the term "host

adapter" should be shot for trying to create still more confusion.

12. RECOMMENDATION: BIG, FAST DISK CHARACTERISTICS

A. 10 MBytes/sec Fast SCSI II transfer rate is good.  5.7 MBytes/sec is slow.

B. RPM: 4500 to 6300+ is good.  3600 or 2400 is slow.

C. Note that buffering on a hard disk is more important than

raw speed.  An intelligent buffer on the drive that buffers

writes and reads ahead is crucial.  A large cache is good.

D. 8 or 10 ms access time is great.  15 ms is slow.

E. Low command overhead is good.

F. Drives that "thermally re-center their heads" are bad.  Some

drives that use this technique have to stop every so often to re-calibrate.

We've seen MediaSpace movies stop for 4 seconds during

playback while this occurs.  Drive manufacturers are loathe to

brag about this in their product sheets.  If you find this out about

a drive, put it on your "sell" list.

G. Comments heard in passing:

- Conner Cougar is line is real fast.

- Fujitsu drives are good.

- MicroNet resells Seagate.

- Liberty resells Quantum.

- Adstar (IBM) drives are good.

- Maxtor's 6300 RPM drive is a screamer.

- Recommendation from chip maker: EISA machine, Adaptec or MCT

controller, Micropolis SCSI II drive.

A fast gigabyte drive can be purchased at retail for as little as $1300.

Many many of these Gig drives are 3.5" and "half height."

13. WARNINGS: FAST, NEW PRODUCTS CAN BE UNRELIABLE

The latest and greatest products may not be as reliable as those which

have been on the market for a while.  Revision 1.0 drives have been

known to crash.  Tech support responsiveness and documentation

quality varies dramatically between controller and drive manufactureres:

Adaptec: Good doc, excruciating phone waits.

DTC: Medium doc, short phone waits.

AMI: Medium doc, short phone waits.

Micronet: Good doc, short phone waits.

In addition, gigabyte drives are marvels of technical innovation with

mind bogglingly small tolerances.  Treat your drive with the utmost

care.  Run Norton Disk Test once a month to mark any sectors which

may have gone bad on you.

Buying a tape backup unit is a smart investment.  However, a tape

backup unit is yet another challenging device to install and use

on your computer.

Because settings may have been pushed to the limit, don't congratulate

yourself for increasing your Data Rate Calibrate score until you've

made a few large movies and your system has proven itself to be

stable.

14. APPENDIX: VL DATA RATE CALIBRATE TECHNICAL

MediaSpace users run VideoSnap in the MIC System II group of Windows

to record movies.  Under Control/Data Rate Calibrate, users can benchmark

their hard drive.  Here's what happens:

The target data rate is set to "system," i.e. to maximum possible.

Quality is set to 100%.

Sample is set to 30 fps.

A DVA still image that is very complicated and hard to compress is

loaded into the DVA framestore.  This frame compresses to 40K.

Record is set to "video only" - i.e. no audio is recorded.  During

video only recording, two files are written to.  JPEG video frames

go into one file, an index in the other.  The index is

written in 16K chunks every 200 frames.  At the end of the

recording, the index is appended to the end of the JPEG video

frame file.

Priority is qualty not frames, i.e. frames get compressed and written

at much less than 30 fps, but frames don't get discarded when

time boundaries are missed.

Calibrate records/compresses this same frame for about 25 seconds.

At the end, the file size (jpeg frames + index) is divided by

25 to get KBytes/sec throughput.  This number is then multiplied

by 92% to provide a little slack so that users who are really

pushing the limit (red lining) don't start dropping frames when

the incoming video has a complexity spike.  (Incoming video

varies in its "complexity" or ability to be compressed based on

a number of factors including color distribution, number of visual

elements, and sharpness of transitions between visual elements.)

MediaSpace's theoretical throughput limit is somewhere between 900K/sec

and 1 MByte/sec.  A good way to test your machine's limit is with a

large 28 Meg RAMdrive.  You'll need 32 MBytes of RAM for a valid calibration

with MIC II v2.1.  In config.sys, try:

device=c:\windows\ramdrive.sys 28000 /e

MediaSpace is an ISA bus board.  In the limiting case, 900K of compressed

data goes from MediaSpace to host memory at ISA rates (even when the

board is running on an EISA or Local Bus computer).  From host RAM,

the data goes to disk.

15. APPENDIX: HOW 3RD PARTIES MIGHT IMPROVE THROUGHPUT

A 3rd party such as a independent software vendor ("ISV") writing a video

editor might be willing to supply some additional hard disk driver

software to improve MediaSpace performance and gain a competitive

advantage over other software vendors.  Below are two methods which

could be pursued.

15A. VL's MIC II driver for Mediaspace runs as a virtual machine

alongside Windows.  MIC II bumps itself up to 90% priority and

drops Windows down to 10% during record and playback of movies.

During record, MIC II might want to write out a large chunk of

data.  In DOS, the MS C 7.0 call, write(handle, ptr, length),

goes through DOS INT 21 which calls BIOS INT 13 which does 16 bit

disk i/o.  An ASPI driver installs itself in INT 13 and speeds

up the INT 13 call:

write(handle, ptr, length)

|

INT 21

|

INT 13

|

ASPI driver

ISVs often complain that standard DOS i/o is quite stupid.  The

FAT is updated much too often, for instance.

With Windows, the INT 21 call gets intercepted by the Windows

kernel.  Windows 3.1 has an optimized device driver for controllers

using the standard Western Digital Chipset for IDE.

An ISV can supply a fast virtual machine at this level to replace

the Western Digital driver.  This information can be found in the

Windows 3.1 Device Driver Kit: Virtual Device Adaptation Guide,

Chapter 9: Fast Disk Devices.  A 32 bit i/o, ring 0 (highest priority)

driver could be built.

15B. VL would consider working with an ISV to test the feasiblity of

using an ISV supplied library of disk i/o calls.  Once feasibility

was established, harder issues covering funding, maintenence, and

productization would have to be addressed before the ISV could

deliver an accelerated solution to end users.

VL uses standard C 7.0 file i/o calls to transport data throughout

MediaSpace.

VL would require a generally DOS compatible file-system. eg. access(),

rename(), and delete() work normally. Other applications would need to

be able to access the files quite normally.

The specific calls required by the transport code are described below.

Note that the transport will still do some normal DOS IO but the calls

identified below are those that carry the bulk data.

OPENs

File = open( STRING, O_BINARY | O_RDWR | O_CREAT | O_TRUNC,

S_IWRITE | S_IREAD);

File = open( STRING, O_BINARY | O_RDONLY);

File = open( STRING, O_BINARY | O_RDWR);

DATA

wBytesRead = read((vINT) hMed->lData1, hpBuffer, (vWORD) dwBytes);

Generally the sizes of reads are variable >= 8K and < 64K (for

high data rate movies).

wBytesWritten = write((vINT) hMed->lData1, hpBuffer, (vWORD) dwBytes);

Generally the sizes of writes are less variable >= 16K and < 64K

lNewOffset = lseek((vINT) hMed->lData1, Offset, SEEK_SET);

lNewOffset = lseek((vINT) hMed->lData1, Offset, SEEK_CUR);

Seeks are done on play-back to skip over sections of data because

play-back is falling behind. Hence lseek MUST NOT have a

dramatically bad effect on read performance, ie. increased read

performance must NOT be achieved at the cost of much degraded

random access performance.

Seeks are also used intensively during reverse play-back. In

this mode the above random access issue is critical.

CLOSE

close((vINT) hMed->lData1);

16. FURTHER READING:

ADAPTEC ARTICLES - these are very helpful, contact Adaptec to order them.

Evaluating and Understanding SCSI Host Adapter Performance

Fast SCSI is Here

Enhanced Mode SCSI

Intelligent Caching - An analysis of SCSI performance

Building a Superior PC Server

OTHER ARTICLES

Comuter Technology Review has over 300 articles on hard drive

subsystems.  They have a back order service where you can

get an article listing and then order articles for a price.

PC Magazine

April 8, 1991. Ben Myers.  "SCSI Controllers Difficult

To Configure."  And how!

Feb 23, 1993: "Tutor: Answers to Five Common Questions

About Hard Disks."  Note that many of the 5 answers

are incorrect for MediaSpace users.  The author

thinks IDE and SCSI are of equal speed.  Wrong! The

author also thinks smartdrv is smart.

In 1993, magazines are still getting it wrong.

Special thanks to Jacqui R for risking her eyesight by typing in

the Maxtor Benchmarks and Steve F for a proofread.

Steve Raney

July, 26