| Field Note: 77 |  | Distribute freely |
| --- | --- | --- |

Subject:  Mediator LC and PAL TVs using SCART

Date Created: July 29, 2026

Product: Mediator LC

Mediator LC can be used successfully with many recent PAL TVs which have RGB SCART sockets.

A special cable is required to connect the Mediator LC's RGB output to the TV's SCART input.  This cable can be made up for you by PC dealers or resellers and should meet the following requirements:

Mediator LC Connector type: 9-pin male D-Type

Analogue RGB out: pins 1(red),2(green),3(blue)

Analogue Returns: pins 6(red),7(green),8(blue)

Composite Sync:   pin 4 (4V output - attenuate to 300mV at SCART end)

Sync Return:      pin 9

The SCART signal 'RGB status' must be driven high to enable RGB input.  The TTL vertical sync signal on pin 5 of the D-Type may be suitable for this purpose.

If you are having the cable made up for you, you should provide specific information regarding the SCART setup for your particular TV model. This can usually be found either in the TV manual or from the manufacturer.

Please note that vertical synchronisation may not be achieved with older TVs.  This is not a Mediator fault.

Lachman Dhalliwal

July, 26