| Field Note: 84 |  | Distribute freely |
| --- | --- | --- |

Subject:  	Using Mediator LC with Macintoshes

Date Created: 	July 29, 2026

Product: 	Mediator LC

Using Mediator LC with Macintoshes

Mediator LC can be used successfully with those Macintoshes which are capable of outputting VGA 640x480 60Hz graphics - including the MAC LC, the Quadras and some PowerBooks.

You need a Mac to VGA adaptor or adaptor/cable between the Macintosh and the Mediator LC.  These adaptors are available from a number of sources, and are sold to allow VGA and multifrequency monitors to be used with suitable Macs.

The Mac monitor sense wires are linked inside the connector to force the Mac graphics into VGA emulation mode.

Most Macs output 'Sync-on-Green', which can cause a strong green tint on some non-Apple monitors.  Software may be available to switch off 'Sync-on-Green', but this is NOT needed for Mediator LC.

If you wish to make your own Mac to Mediator LC cable, here are the connections:

MAC 		            VGA

2  	- RED  -    	1

1  	- RGND -    	6

5  	- GRN  -    	2

6  	- GGND -    	7

9  	- BLU  -    	3

13  	- BGND -    	8

15  	- HSYNC-    	13

14 	- HGND -    	10

12  	- VSYNC-    	14

11  	- GND  -    	5

7   	-I

10  	-I SENSE LINK

NB: Wire link from pin 7 to pin 10 on the Mac connector is essential.

Please use good engineering practice to maintain signal quality and comply with applicable EMC standards."

The adaptor was tested with the following Macintoshes and functioned correctly:

Macintosh LC

Centris 650

Macintosh IIcx

Quadra 700

Powerbook 170

Lachman Dhalliwal

July, 26