| Field Note: 165 |  | Distribute freely |
| --- | --- | --- |

Subject:	Using Mediator XLC with Windows 95

Date Created:	July 30, 2026

Product:	Mediator XLC

The Mediator XLC functions under Windows 95.  To take full advantage of all the features of the Mediator XLC it is recommended that the following is considered.

The Mediator XLC can accommodate an output of 640x480@60Hz operation from any graphics card.  To use the software controls that are designed into the Mediator XLC driver, a small batch file needs to include the following, and also executed in the autoexec.bat from your Windows 95 configuration.

Sample batch file to be inserted in the AUTOEXEC.BAT @echo off cls echo. echo VideoLogic Mediator XLC driver menu selection. echo ---------------------------------------------- echo. echo (1) Load Mediator XLC PAL driver. echo (2) Load Mediator XLC NTSC driver. echo. echo (N) Continue without loading XLC drivers. echo. echo. choice /c:12n Choose an option /t:n,5 if errorlevel 3 goto Cont if errorlevel 2 goto NTSC if errorlevel 1 goto PAL :PAL c: cd\mediator vga2pal goto Cont :NTSC c: cd\mediator vga2ntsc goto Cont :Cont echo.

This batch file relies on the CHOICE.EXE program.  Once executed, the batch file gives a menu of options, and defaults to bypass, if no selection is made within the programmed time of 5 seconds approx.

Alan Lee

July, 26