| Field Note: 178 |  | Distribute freely |
| --- | --- | --- |

Subject:	Using Mediator XLC with DVA-4000

Date Created:	July 31, 2026

Product:	Mediator XLC

This Field Note addresses DVA-4000 usage with Mediator XLC v2.11.

This document assumes that the user has a DVA-4000 running with Windows 3.1 using MIC system II v2.2.1

PAL mode output

When using the Mediator XLC in PAL mode (Using VGA2PAL.EXE) the DVA-4000 exhibits a rolling video within the video window,  this is regardless of PAL or NTSC source.

The problem lies with the fact that the DVA-4000 operates in 60Hz mode, and although the Mediator XLC driver is used, it only  alters the graphics card refresh output to 50Hz operation (of which the graphics card must be supported by the Mediator XLC driver, please see the readme.txt for further information) hence complying with the PAL standard.  The driver does not change the DVA-4000 output.

Using Patch 97 will alleviate the problem by the use of an utility which is executed under Windows called SYNCSW.EXE,  this utility can also be found if the user has MIC system II developers kit.  The utility is located on diskette 2 labelled ‘developers’ in the directory a:\developr\syncsw\syncsw.exe.

Once a video source has been connected to the DVA and configured with MIC system II, show the ‘video window’ via the ‘live window’ button within MIC system II Control Panel.  The rolling video should be present regardless of input colour source.  Execute the utility SYNCSW.EXE, the utility should show PAL Video (50Hz) radio button selected and the video should stabilise.  See below for a screenshot of the MIC SyncSwitch.

µ §

NTSC mode output

When using the Mediator XLC in NTSC mode (Using VGA2NTSC.EXE) the DVA-4000 exhibits no known problems with the video window with both PAL and NTSC source.

Alan Lee

July, 26