| Field Note: 8 |  | Distribute freely |
| --- | --- | --- |

Subject:  Controlling Pioneer videodisc players from DOS

Date Created: July 29, 2026

Product: Peripherals - videodisc players

The following information may be of interest to support staff...

To communicate with a Pioneer videodisc player from DOS, you can issue the following commands:

C> mode com1: 4800,n,8,1

C> echo >com1 PL    {this will cause unit to play}

C> echo >com1 OP    {this will cause the door to open}

All the Pioneer videodisc players use the same commands except for the LD-V6000.  The LD-V6000 requires the commands to be entered in hexadecimal.

The parameters given to the mode command will depend on which communication port you have the player connected to,  and the communications parameters set up on the player's switch bank(s).

mode <port>: <baud>,<parity>,<data>,<stop>

<port> = COMx (where x will probably be 1 or 2)

<baud> = the baud setting of your player (e.g. 1200, 2400, 4800, 9600)

<parity> = n = no parity; e = even parity; o = odd parity

<data> = the number of data bits used (usually 7 or 8)

<stop> = the number of stop bits used (usually 1)

The same principle should also work when communicating with other controllable devices.

Colin Crawford

July, 26