| Field Note: 55 |  | Distribute freely |
| --- | --- | --- |

Subject:  InfoWindow Control Program compatibility level

Date Created: July 29, 2026

Product: MIC InfoWindow Emulator

The MIC InfoWindow Emulator is a multimedia control system which allows applications originally written to run on the IBM InfoWindow System to also run on DVA-4000 systems. It was developed in 1990 to be compatible with the most recent specification of the IBM InfoWindow System - this being defined by version 1.3 of the InfoWindow Control Program.

Since then, the InfoWindow Control Program has been modified to include new functionality and greater support for graphics resolutions, the most significant difference being support for the standard VGA resolution of 640 x 480 at 16 colours. The Emulator does not support this extra functionality.

To summarise...

The MIC InfoWindow Emulator is functionally equivalent to version 1.3 of the IBM InfoWindow Control Program.

The Emulator cannot support applications which were written for a later version of the Control Program.

The DVA-4000 and the Emulator provide hardware compatibility in the following areas: graphics, video overlay, touchscreen control and videodisc control (simultaneous control of only a single player). It cannot emulate the IBM InfoWindow ROM speech synthesis module or external speech.

As defined in version 1.3 of the Control Program, the Emulator supports graphics resolutions up to 640 x 350 with no video overlay, and video overlay in resolutions up to 640 x 200.

Colin Crawford

July, 26