| Field Note: 60 |  | Distribute freely |
| --- | --- | --- |

Subject:  User codes on various videodisc players

Date Created: July 30, 2026

Product: MIC System I

For historical reasons, the value returned from a REPORT USERCODE (REP UC) command in MIC System I will be different for the same videodisc depending upon the manufacturer of the videodisc player it is being played in.

The differences are highlighted in the table below.

| Usercode | Philips | Sony | Pioneer |
| --- | --- | --- | --- |
| 001A | 0=01: | 0=0£A | 0=01: |
| 001B | 0=01; | 0=0£B | 0=01; |
| 0000 | 0=000 | 0=000 | 0=000 |
| 0001 | 0=001 | 0=001 | 0=001 |
| 1DFF | XXXXX | 1=D¤F | XXXXX |

The value returned for any particular videodisc will be consistent across all the players in manufacturers range.

The = character in all the codes above is inserted by MIC.

The £ character is 91 (hex)

The ¤ character is A6 (hex)

For this reason, developers of systems which need to test for a usercode should check for both returned values.

Colin Crawford

July, 26