| Field Note: 235 |  | Distribute freely |
| --- | --- | --- |

# Subject: MMSYSTEM262 error or system lock-ups with Windows 95 OSR2

# Date Created: February 27, 1997

# Product: MPEGStar

When using MPEGStar under Windows 95 OEM Service Release 2 (otherwise known as OSR2) you might get a MMSYSTEM 262 error message or experience a system lock-up when trying to play an MPEG clip.

This is because OSR2 supports IRQ (Interrupt ReQuest) steering, which is disabled by default, but which must be enabled for MPEGStar to work correctly. IRQ steering is the ability to intercept requests issued for an IRQ and route them to the next available IRQ.

## Enabling IRQ Steering

To enable IRQ steering:

1.	In the BIOS setup of your PC, set the Plug and Play PCI configuration to “ICU” or “Automatic Configuration”. Refer to the documentation that came with your PC on how to do this, or contact your PC vendor.

2.	In Windows 95, open the Control Panel window by clicking the Start button, pointing to Settings and clicking Control Panel.

3.	Double click the System icon to open the System Properties window.

4.	Click the Device Manager tab.

5.	Double click System Devices, and click PCI Bus.

6.	Click Properties.

7.	Click the Settings tab.

8.	In the Device Enumeration section, select the “Use BIOS” check box.

9.	Click the IRQ Steering tab, and select the “Use IRQ steering” and “Get IRQ table from PCBIOS 2.1 call” check boxes.

10.	Click OK and OK again to close the System Properties window.

11.	Restart your computer.

## Checking IRQ Steering

To check that IRQ steering is working:

1.	In Windows 95, open the Control Panel window by clicking the Start button, pointing to Settings and clicking Control Panel.

2.	Double click the System icon to open the System Properties window.

3.	Click the Device Manager tab.

4.	Double click System Devices, and click PCI Bus.

5.	Click Properties.

6.	Click the IRQ Steering tab

If IRQ steering is working, you should get the following IRQ routing status:

IRQ steering enabled
IRQ table read from PCI BIOS
IRQ Miniport Data processed successfully
IRQ Miniport loaded successfully

VideoLogic Support Team
February, 97