| Field Note: 43 |  | Distribute freely |
| --- | --- | --- |

Subject:  	TV output and encoders

Date Created:	January 20,1993

Product: 	Rapier 24

Background

Rapier 24 can produce NTSC or PAL output, encoded in RGB format.  This type of encoding can be fed directly to many TV monitors (especially those with SCART / Euro AV connectors) and video display systems.  The quality is very sharp due to the high-bandwidth RGB signals and 24-bit truecolour.

All domestic and most professional video recorders require CVBS composite encoding or S-Video Y/C encoding input.  There are some ultra-professional video recorders that accept RGB or YUV (Betacam) encoding.  These are used in video production studios.

The main uses for recordable Rapier 24 output are:

3D Studio animation to Diaquest tape controller

Windows presentation apps. such as:

Persuasion, PowerPoint, Harvard Graphics, Action! etcetera

Animation, TIGA-based or Windows apps.

How to use TV output

Run the CONFIG program in the RAPIER directory and select "TV line rate RGB" as the Monitor Type.  NTSC or PAL formats are selected in the Resolution section.  There are 2 resolutions for PAL and 2 for NTSC.

These are the special "square-pixels"; which guarantees that images retain the aspect ratio when viewed on the TV system, and standard CCIR-601; which stretches computer graphics sideways.

| Resolution | Frame rate Hz | Scan rate kHz | Comment |
| --- | --- | --- | --- |
| 640 x 480 | 59.9 | 15.7 | NTSC square pixels |
| 720 x 486 | 60 | 15.7 | NTSC CCIR-601 |
| 768 x 576 | 50 | 15.6 | PAL square pixels |
| 720 x 576 | 50 | 15.6 | PAL CCIR-601 |

Table 1.  TV resolutions

Use a VGA to BNC monitor cable or make a Rapier to SCART cable, using the connections shown in Table 2.

Remember that most computer monitors CANNOT display TV pictures and so a separate VGA or multi-sync monitor is required for connection to the VGA card.

The picture on the TV monitor is interlaced at 50 or 60Hz and so flicker can be very noticable.  This is normal for still pictures in TV formats.  To minimise the effect:

Draw any images with wide lines and large text

Use 24-bit anti-aliasing in the image editing software

Use moving images

Use VideoLogic Mediator instead of Rapier's TV modes (see below).

Encoders

A TV encoder can convert RGB-encoded NTSC/PAL to composite-encoded NTSC/PAL.  They cost from £300 to £1000.  Some can also output S-video for better quality.  They are simpler and cheaper than scan converters (such as VideoLogic's Mediator) because the graphics card produces interlaced TV scan timing rather than computer monitor non-interlaced output.

Simply connect the Red, Green, Blue, Ground and Composite Sync signals from Rapier's output to the encoder inputs.  If an encoder supports Separate Horizontal and Vertical syncs, then the Rapier 24 CONFIG program must select  the "-ve Separate Syncs" option in the Sync Type section.

Many encoders, such as the Neotech NE0061, are designed for connection to a Macintosh graphics card.  In this case, the Rapier 24 VGA-to-Mac adapter cable (supplied)  MUST be used, in conjunction with a standard Macintosh monitor cable.  In addition, the Rapier 24 CONFIG program must select  the "-ve Comp Syncs on V" option in the Sync Type section.

| Rapier output | SCART plug | Signal |
| --- | --- | --- |
| 1 | 15 | Red video |
| 2 | 11 | Green video |
| 3 | 7 | Blue video |
| 6 | 13 | Red ground |
| 7 | 9 | Green ground |
| 8 | 5 | Blue ground |
| 10 | 17 | Sync ground |
| 13 | 20 | Composite Sync |

Table 2.  SCART cable connections

Rapier can be set to its 640 x 480 resolution at 60Hz non-interlaced mode (VGA monitor compatible) and connected to Mediator or Mediator LC.  This provides a flicker-free output, in NTSC / PAL and Composite / S-Video format.

TV studios

For professional TV studio production, video sources must be stable with temperature, have bandwidth filtering and GenLock to be used with a video mixer.  These features are beyond the scope of this product but it could be used with a GenLock scan converter that has these special features.

Steve Everitt

January, 93