| Field Note: 73 |  | Distribute freely |
| --- | --- | --- |

Subject:  	New Windows drivers in release 1.2.2

Date Created: 	16-August-1993

Product: 	Rapier 24

The software release version 1.2.2 for Rapier 24 has Windows drivers that can be optimised for different applications.  The default installation will give the user the best performance for most applications, but the default Windows driver, called "Rapier (small/large font)", cannot run the following applications:

Adobe Illustrator v 4.0 or 4.01

Lotus Notes v2.5

An alternative Windows driver is available on the install floppy that allows these applications to run correctly.  This is called "Rapier no BMC (small/large font)" and is different to the default driver because it has no bitmap caching.  It is slower than the default driver and should only be used when it is essential to run the applications listed above.

Note that Adobe Illustrator v4.02 will run with the default driver and is expected to be available Q4 1993.

## How to install the no bitmap caching driver

This is done in the normal way, using the Windows Setup program.

1.  Install the software and configure Rapier as described in the Installation Guide.

2.  Before running the Windows Setup program, change to the floppy drive with the Rapier install disk inserted.  Then run ADOBEFIX.BAT, e.g.:

C:\RAPIER>  a:  <cr>

A:\>  ADOBEFIX  <cr>

This changes the OEMSETUP.INF file so that the "Rapier no BMC" driver is also available.  ADOBEFIX must be run when the floppy drive is the current directory.

3.  Change back to the Windows directory on hard disk and run the Setup program, e.g.:

A:\>  c:  <cr>

C:\WINDOWS>  setup  <cr>

If you run the network version of Windows then remember to enter setup /n instead.

4.  Select "Display" from the menu and scroll to the end to select "Other (requires disk provided by a hardware manufacturer)".  Press Enter.

5.  Setup then asks which floppy drive to read the display driver disk.  Enter your drive letter, e.g. a:\

6.  The program will show 4 choices:

Rapier (large font)

Rapier (small font)

Rapier no BMC (large font)

Rapier no BMC (small font)

Select the "Rapier no BMC" option, large or small font, and follow the rest of the instructions on the screen in Setup.

If Setup asks for the display driver disk again, then enter the floppy drive letter once more.

7.  When Setup has returned to the DOS prompt then Windows can be started.

If Setup hangs, then reboot and try again with SETUP /I .

## Extra notes

Image North Technologies ImageQ v2.0 has a slide viewer which can have a problem with the picture sliced into strips ("venetian blind") when using the default driver.  This problem is fixed by using the "no BMC" driver as described above.

It is possible to use the Windows Setup application in the Windows Main group to install the "no BMC" driver, and then restart Windows.

To change between the default driver and the "no BMC" driver anytime, use Setup from DOS or from Windows.

The ADOBEFIX.BAT program will return to the default choice of only one Rapier driver if it is run a second time.

The default driver uses the files RAPIER.DRV and RAPIER.RLM.

The "no BMC" driver used the files RAPIERB.DRV and RAPIERB.RLM.

## Special driver switches

Two switches are available to control the Windows driver.  These are the "ImageQ" and the "FastPolygons" switches.  They are controlled in the WIN.INI file, in the [Rapier] section.

ImageQ switch:

This switch only works with the default Rapier driver and has no effect on the "no BMC" driver.  This table shows the effect of the switch on different applications:

| Entry in WIN.INI [Rapier] section | Image North Technologies ImageQ v2.0 | Aldus Persuasion v2.1 | Adobe PhotoShop v2.5 |
| --- | --- | --- | --- |
| ImageQ=ON (default setting) | Runs OK | Bitmaps in Slide Sorter are distorted. Workaround: use monochrome or resize slides in Slide Sorter | Brush Options run OK |
| ImageQ=OFF or switch not present | Causes GPF, cannot run. | Bitmaps in Slide Sorter are OK | Brush Options cause GPF, cannot run |

N.B.  GPF is General Protection Fault

FastPolygons switch

This switch works with both the default driver and the "no BMC" driver.  This table shows the effect of the switch on different applications:

| Entry in WIN.INI [Rapier] section | Corel Systems CorelDraw! v3.0 and v4.0 | TI Wintach v1.2 |
| --- | --- | --- |
| FastPolygons=ON | Fine line fills run at standard speed | SpreadSheet benchmark runs fast |
| FastPolygons=OFF or switch not present (default setting) | Fine line fills run 3x to 6x faster. Examples are SNOWBARN and GRASHOPP files. | SpreadSheet benchmark runs slightly slower. RPM is about 5% less. |

We recommend that the default values of these switches are used for normal use, i.e. "ImageQ=ON", and "FastPolygons=" not present in WIN.INI

Steve Everitt

August, 93