| Field Note: 81 |  | Distribute freely |
| --- | --- | --- |

Subject:  DVA-4000 not compatible with PS/1 model 2133-471

Date Created: July 30, 2026

Product: DVA-4000 & Rapier 24

The feature connector on the IBM PS/1 model 2133-471 has 32 pins and is not compatible with the DVA-4000 or Rapier 24 (or any other add-in card expecting a standard VGA feature connector). The signals provided on the feature connector are reminiscent of older, EGA-style, systems, but the connector set-up itself is not familiar to any VideoLogic engineers - this PS/1 is (we believe) an older model.

Although this particular PS/1 is not DVA-4000 compatible, IBM assure us that this is not necessarily true of the whole PS/1 range which uses a number of different motherboards and feature connectors.

Colin Crawford

July, 26