| Field Note: 166 |  | Distribute freely |
| --- | --- | --- |

Subject:  	Windows 95 with Rapier 8-24

Date Created: 	July 31, 2026

Product: 	Rapier 8-24

As the Rapier product family has been discontinued, VideoLogic will not be developing native drivers for Windows 95.  However, Rapier 8-24 works under Windows 95 with its existing drivers in certain display modes and as long as you follow the instructions below.

All 8 and 16 bit colour modes are supported under Windows 95.  When a 24 bit display mode is selected the Windows graphics are corrupt.

To install the Rapier 8-24 drivers start your computer into Windows 95 DOS mode:  On boot up when the message "Starting Windows 95" appears, press F8 and choose "DOS Prompt only" from the menu.  You can then run a:\INSTALL from the DOS Prompt.  Follow the instructions in the Rapier 8-24 User Guide.

Restart your computer.

You must now add the Windows 3.x driver to the system.  Go into Win95 Control Panel, "Add new hardware" and choose “no” to search.  Choose "Display Adapters", insert the Rapier 8-24 v1.0 disk into your floppy drive and press "Have disk".

Press “Finish” and the Rapier 8-24 Windows drivers are now being installed.

Silvia Ohr

August 1995