Orchid Fahrenheit Display Driver Installation Instructions

for IBM OS/2 2.0

Driver Release (Beta) 3.0

OS/2 versions supported

-----------------------

This driver release is intended for use with IBM OS/2 2.0 General Availability

(GA), OS/2 2.0 with the application of the IBM OS/2 2.0 Service Pack. Support

for OS/2 2.1 Beta (all versions) is being enhanced. (The drivers have

been tested with OS/2 2.1 Beta 12/92 release and no problems were noted.

Drivers to support OS/2 2.1 GA are under development and will ship at the same

time, or shortly after, OS/2 2.1 becomes a retail (non-beta) product.)

Adapters supported

------------------

This driver release is intended for all Orchid Fahrenheit adapters

(ISA & local bus) using the S3 86C911, 86C924, 86C801 and 86C805 chips.

All supported adapters must have at least 1Mb of video memory installed.

Adapters with 512Kb video memory are not supported.  Memory upgrade

kits are available from Orchid Technology Customer Service

Department.

Video Resolutions supported

---------------------------

F10X8.DLL supports a resolution of 1024 x 768 with 256 colors at

vertical refresh rates of 45Hz (interlaced), 60Hz or 70Hz (both

non-interlaced).

F8X8.DLL supports a resolution of 800 x 600 with 256 colors at

vertical refresh rates of 56Hz, 60Hz, or 72Hz.

Adapter configuration

---------------------

No special configuration settings are needed.

Installation Procedure

----------------------

Before installing new drivers, it is recommended that a full back up

of the OS/2 system partition be performed.

For OS/2 2.0 (GA) with ServicePak installed, the installation is

automatic using DSPINSTL.EXE  With OS/2 installed for VGA or previous

driver release, select DSPINSTL.EXE from DRIVES folder (on drive A:

or B: containing Orchid OS/2 driver disk) and select appropriate

options from menu.

When DSPINSTL is complete, please verify on your OS/2 boot drive

(C:\OS2) system directory, that a new SVGADATA.PMI file has been

created.  If not, create a SVGADATA.PMI file by running SVGA.EXE.

SVGA can be run in a DOS full screen session (the preferred choice),

an OS/2 full screen session, or a real DOS environment. For the first

two choices

svga on

should result in the creation of a SVGADATA.PMI file located in the

C:\OS2 subdirectory.

If SVGA.EXE is run in a real DOS environment, the following command

line should be used

svga on dos

In a "real" DOS environment, SVGA.EXE creates a file SVGADATA.DOS

which must be renamed to SVGADATA.PMI. SVGADATA.PMI must reside in

the C:\OS2 subdirectory located on the OS/2 boot volume. If the boot

volume is not located on C:, then a dummy \OS2 subdirectory must be

created and a copy of the SVGADATA.PMI must be placed in it.

Manual Installation

-------------------

For OS/2 2.0 the installation has to be performed manually.  Follow

the procedure described below.

Note: in the following examples, the Orchid Fahrenheit OS/2 drivers

disk is assumed to be in drive A: and the OS/2 system files are

assumed to be on drive C:. If your situation is different, please

substitute the approriate drive letters.

Installing DISPLAY.DLL

----------------------

Create a subdirectory under C:\OS2\DLL on the OS/2 boot partition.

md c:\os2\dll\orchid

cd c:\os2\dll\orchid

Then copy the appropriate driver file from the Orchid Fahrenheit OS/2

driver diskette to this directory using the OS/2 UNPACK command.

(UNPACK is supplied with OS/2 and can be found in the \OS2

subdirectory.) For example, to install the 800x600 256 color driver

use

UNPACK a:\f8x8.dl@

To install the 1024x768 256 color driver use

UNPACK a:\f1x8.dl@

OS/2 CONFIG.SYS file changes

----------------------------

Use a text editor and modify the LIBPATH statement in the OS/2 2.0

CONFIG.SYS to put the ORCHID subdirectory in the LIBPATH path. It

should be placed before any other LIBPATH entries.

As an example

LIBPATH=.;C:\OS2\DLL;C:\OS2\MDOS;C:\;

should be changed to

LIBPATH=C:\OS2\DLL\ORCHID;.;C:\OS2\DLL;C:\OS2\MDOS;C:\;

While editing the CONFIG.SYS file, add the following line

DEVICE=C:\OS2\MDOS\VSVGA.SYS

Remove or comment out the following line

DEVICE=C:\OS2\MDOS\VVGA.SYS

Also, add the line

SET VIO_VGA=DEVICE(BVHVGA,BVHSVGA)

and remove or comment out line

SET VIO_VGA=DEVICE(BVHVGA)

BVHSVGA (Base Video Handler Super VGA) support file installation

----------------------------------------------------------------------

In the C:\OS2\DLL subdirectory, rename file BVHSVGA.DLL to

BVHSVGA.RTL.  Copy the BVHSVGA.DL@ from the Orchid Fahrenheit OS/2

driver diskette to this subdirectory using the UNPACK command

UNPACK a:\bvhsvga.dl@

VSVGA (Virtual Device Driver - VDD) support file installation

-------------------------------------------------------------------

Switch to the C:\OS2\MDOS subdirectory. If the file VSVGA.SYS exists, rename

it to VSVGA.RTL. Copy the Orchid VSVGA.SY@ file from the Orchid Fahrenheit

OS/2 Driver diskette to this subdirectory using the UNPACK command

UNPACK a:\vsvga.sy@

Return to the C:\OS2 subdirectory and if the file SVGA.EXE exists,

rename it to SVGA.RTL. Copy the SVGA.EX@ file from the Orchid

Fahrenheit OS/2 Driver diskette into the C:\OS2 subdirectory using

the UNPACK command.

UNPACK a:\svga.ex@

Create a SVGADATA.PMI file by running SVGA.EXE. SVGA can be run in a

DOS full screen session (the preferred choice), an OS/2 full screen

session, or a real DOS environment. For the first two choices

svga on

should result in the creation of a SVGADATA.PMI file located in the

C:\OS2 subdirectory.

If SVGA.EXE is run in a real DOS environment, the following command

line should be used

svga on dos

In a "real" DOS environment, SVGA.EXE creates a file SVGADATA.DOS

which must be renamed to SVGADATA.PMI. SVGADATA.PMI must reside in

the C:\OS2 subdirectory located on the OS/2 boot volume. If the boot

volume is not located on C:, then a dummy \OS2 subdirectory must be

created and a copy of the SVGADATA.PMI must be placed in it.

DOS settings

------------

Switch-Notification

Recommended setting for most reliable operation is OFF. Setting this

setting to OFF forces the VDD to assume complete responsibility for

saving or restoring the video hardware state upon a background or

foreground switch.

I/O Trapping

When turned ON, accessess to the S3 86C911/924/801/805 enhanced I/O

ports are trapped and shadowed by the VDD. This trapping and

shadowing will affect performance, but is necessary when the

Fahrenheit adapter is being used in one of the enhanced modes and the

state is required to be restored completely.

Known Shortcomings or Problems

------------------------------

For Fahrenheit VA and VLB (S3 86C80X) boards, the SVGA VESA mode support

in full screen DOS modes has not been fully verified.

Support & Feedback

------------------

Driver support questions or feedback can be addressed to the Orchid

Technical Support Department by voice phone: 510 683 0323; CIS EMAIL

(use Orchid's Customer Service ID 76467,1424); or by FAX (510 683

0379 or 510 683 0355).

If submitting a problem report, or just supplying feedback, please

have ready a complete system hardware description along with detailed

steps to re-create the symptoms. If submitting problem or feedback

reports via CIS EMAIL or FAX, please include a daytime phone number

and a best time to call in case Orchid Technical Support needs more

information.

Warranty Disclaimer

-------------------

The software and documentation is a beta release and one or both may

be modified  in subsequent releases. Performance and reliability of

this software release may not be at the level usually associated with

Orchid display drivers. Driver and documentation are supplied "as

is" basis.