Modem Status Reporting Feature

We have added a feature ‘AT&V2’ which prints out a string that contains a modem status report.  By setting bit 4 of s202 (i.e. s202=16), the modem status string will automatically display before every connect message and after each hang-up.  The string is organized in the following manner:

BEGINaa26ab26ac26ad26ba11bb64 … pa18END

- Where the only capitalized letters are BEGIN and END, signifying the start and stop of the message.

- Where the only small letters are for fields described later.

- Where the only numbers are the values of the fields.

- All values are initialized to 255 with the exception of  ‘ma’ which is initialized to 21. This is done on power up, on atz, and on start of each handshake .

Here is a description of the &v2 command fields:

aa	Current transmit speed

ab	Minimum transmit speed

ac	Maximum transmit speed

ba	Current receive speed

bb	Minimum receive speed

bc	Maximum receive speed

ca	Error correction protocol

cb	Compression type

cc	Modulation

da	Transmit carrier frequency

db	Receive carrier frequency

ea	Symbol rate

fa	Last AGC gain value

ga	Transmit level

gb	Transmit Level Reduction

ha	Last EQM value

hb	Minimum EQM value

hc	Maximum EQM value

hd	Number of times EQM became negative

ia	SNR ratio

ib	Minimum SNR value

ic	Maximum SNR value

ja	Number of local retrains

jb	Number of remote retrains

jc	Number of local rate renegs

jd	Number of remote rate renegs

ka	Transmit Non-linear Encoding

kb	Receive Non-linear Encoding

kc	Transmit precoding

kd	Receive precoding

ke	Transmit shaping

kf	Receive shaping

kg	Trellis mapping

kh	Pre-emphasis

ki	Upper band edge

kj	Lower band edge

kk	Round trip delay high byte

kl	Round trip delay low byte

la	Highest transmit state

lb	Highest receive state

ma	Connection time (Hours)

mb	Connection time (Minutes)

mc	Connection time (Seconds)

na	Auto gain amplitude

nb	Auto gain attenuation

oa	Digital impairments high byte

ob	Digital impairments low byte

oc	Dual PCM and 120 Hz detection

pa	Disconnect reason

Here is a brief description of the fields and tables to decode the values:

(These values are for the US profile, other profiles are not taken into account)

aa	Current transmit speed

ab	Minimum transmit speed

ac	Maximum transmit speed

ba	Current receive speed

bb	Minimum receive speed

bc	Maximum receive speed

These fields display the data rates. The speeds all use the same table.  These speeds can be decoded using the following table.

| SPEED | VALUE |
| --- | --- |
| 0-300 | 0 |
| 600 | 1 |
| 1200 | 2 |
| 2400 | 3 |
| 4800 | 4 |
| 9600 | 5 |
| 12000 | 6 |
| 14400 | 7 |
| 7200 | 8 |
| 16800 | 9 |
| 19200 | 10 |
| 21600 | 11 |
| 24000 | 12 |
| 26400 | 13 |
| 28800 | 14 |
| 31200 | 15 |
| 33600 | 16 |
| 32000 | 17 |
| 34000 | 18 |
| 36000 | 19 |
| 38000 | 20 |
| 40000 | 21 |
| 42000 | 22 |
| 44000 | 23 |
| 46000 | 24 |
| 48000 | 25 |
| 50000 | 26 |
| 52000 | 27 |
| 54000 | 28 |
| 56000 | 29 |

ca	Error correction protocol

This field displays the type of error correction chosen.

| PROTOCOL | VALUE |
| --- | --- |
| NONE | 70 |
| LAP-M | 77 |
| MNP4 | 80 |
| MNP10 with cellular | 81 |

cb	Compression type

This field displays the type of compression chosen.

| COMPRESSION | VALUE |
| --- | --- |
| NONE | 69 |
| MNP 5 | 66 |
| V.42 bis | 67 |

cc	Modulation

This field displays the modulation type chosen.  This field displays the conf (See data pump manual).

| MODULATION | VALUE |
| --- | --- |
| V.34 | 192-206 |
| V.17 | 177-184 |
| V.23 | 161-164 |
| V.21 | 160 or 168 |
| K56flex (K56Plus) | 144-157 |
| V.22 bis | 130-132 |
| V.32 | 112-120 |
| Bell 212 | 98 |
| Bell 103 | 96 |
| V.22 | 81-82 |
| V.FC | 64-76 |
| V.33 | 49-56 |
| Bell 208 | 35 |
| V.29 | 17-20 |
| V.27 | 1-2 |

da	Transmit carrier frequency

db	Receive carrier frequency

These fields are only for V.34 connections.  All other connections see data pump manual for the carrier frequency.  A one signifies the high frequency and a  zero signifies the low frequency.  These are dependent on the symbol rate which can be found in field “ea”.

| SYMBOL RATE | HIGH FREQUENCY (Hz) | LOW FREQUENCY (Hz) |
| --- | --- | --- |
| 2400 | 1800 | 1600 |
| 2800 | 1867 | 1680 |
| 3000 | 2000 | 1800 |
| 3200 | 1920 | 1829 |
| 3429 | 1959 | 1959 |

ea	Symbol rate

This field is only for V.34 and K56flex connections.  All other connections see data pump manual for symbol rate.

| SYMBOL RATE | VALUE |
| --- | --- |
| 2400 | 0 |
| 2800 | 2 |
| 3000 | 3 |
| 3200 | 4 |
| 3429 | 5 |

fa	Last AGC gain value

This field displays the decimal value of the AGC gain high byte.  This value can be used to calculate the Receive level with the following calculation.

| CONFIGURATION | CALCULATION |
| --- | --- |
| K56flex, V.34, V.33, V.17, V.29, V.27 | RL = N/2.7 - 52 dB |
| V.32 bis, V.32 | RL = N/2.7 - 53 dB |
| V.22 bis, V.22, Bell 212, V.23/1200 | RL = N/2.7 - 48 dB |
| V.21, V.23/75 | RL = N/2.7 - 54 dB |
| Bell 103 | RL = N/2.7 - 51 dB |

* Where: N is the number displayed

ga	Transmit level

This field displays the transmit level based on S91 and offset.

Formula: Transmit Level = S91 - ($A48)

The value given is in negative dB’s.

gb	Transmit Level Reduction

This field indicates the transmit level reduction as requested by the remote modem.

ha	Last EQM value

hb	Minimum EQM value

hc	Maximum EQM value

These fields display the values from the last, maximum and minimum EQM readings.

hd	Number of times EQM became negative

This field will display the number times EQM values became negative.  The negative value is an invalid EQM reading and will cause a rate renegotiation.

ia	SNR ratio

ib	Minimum SNR value

ic	Maximum SNR value

These fields display the SNR values obtained during the V.34 probe.  It needs to be converted in the following manner:

SNR in db = 1.7 (X) - 7 	; where X is the value displayed

ja	Number of local retrains

jb	Number of remote retrains

These fields display the number of retrains which occurred during the connection.

jc	Number of local rate renegotiations

jd	Number of remote rate renegotiations

These fields display the number of rate renegotiations which occurred during the connection.

ka	Transmit non-linear Encoding (Warping)

kb 	Receive non-linear Encoding

These fields display a one if non-linear encoding is on and a zero if non-linear encoding is off.

kc	Transmit precoding

kd	Receive precoding

These fields display a one if precoding is on and a zero if precoding is off.

ke	Transmit shaping

kf	Receive shaping

These fields display a one if shaping is on and a zero if shaping is off.

kg	Trellis mapping

This field displays the number of type of trellis mapping used.

| TRELLIS MAPPING | VALUE |
| --- | --- |
| 16 state | 0 |
| 32 state | 8 |
| 64 state | 24 |

kh	Pre-emphasis

This field displays the number of the pre-emphasis filter chosen during the call.  The value will be between 0 and 10.

ki	Upper band edge

kj	Lower band edge

These fields display the upper and lower band edges.  The values need to be converted using the following equations:

Lower -3dB edge (Hz) = 37.5(X) +150

Upper -3dB edge (Hz) = 37.5(X) + 2400

* Where X is the decimal value displayed.

kk	Round trip delay high byte

kl	Round trip delay low byte

These fields display the values needed to calculate the round trip delay using the following calculation:

RTD = (N / X) - 3.5 (for V.34, K56flex, and V.FC)

RTD = (N / 2.4) - 29 (for V.32 bis)

where

RTD is the round trip delay in ms

N is the decimal value of the 16 bit word achieved by the

combination of the hex values of the two fields.

(i.e. kk32kl127… convert 32 to 20h and 127 to 7Fh

to get the 16 bit word 207Fh.  Now convert this to

it’s decimal equivalent 8319 and this is the value of N)

X is the symbol rate found in the field ‘ea’ divided by 1000

We have plans in future releases to print the actual round trip delay value in this field.

la	Highest transmit state

lb	Highest receive state

These fields display the highest transmit and receive spx states reached.  A description of these states can be found in the data pump manual.  The values must be changed to hex format.

ma	Connection time (Hours)

mb	Connection time (Minutes)

mc	Connection time (Seconds)

These fields combine to give the total time of connection.

na	Auto gain amplitude

This field displays the auto gain value chosen for the connection.  The value is determined by the following table.

| AUTO GAIN (dB) | VALUE |
| --- | --- |
| 0 | 0 |
| 10 | 2 |
| 15 | 4 |
| 20 | 6 |

nb	Auto gain attenuation

This field displays the auto gain attenuation.  The amount attenuated can be found using the following table.

| ATTENUATION(dB) | VALUE |
| --- | --- |
| 0 | 0 |
| 4 | 16 |

oa	Dual PCM and 120 Hz detection

This field indicates if dual PCM and/or 120 Hz tones were detected.

Convert this value to its hex value.

If bit 0 is set then 120 Hz tones were detected.

If bit 1 is set then dual PCM was detected.

ob	Digital impairments high byte (for client only)

oc	Digital impairments low byte   (for client only)

These fields display the digital impairments found during the connection.

These fields need to be converted into a 16 bit hex word.

(i.e. oa32ob127… convert 32 to 20h and 127 to 7Fh

to get the 16 bit word 207Fh)

bits 0-5:	 show the RBS pattern detected before pad.

bit 6:		if set, ½ RBS detected

if reset, ½ RBS not detected

bits 8-D:	RBS pattern detected after pad

bits E-F:	shows the digital pad detected

| bit F | bit E | Pad |
| --- | --- | --- |
| 0 | 0 | None |
| 0 | 1 | 3dB |
| 1 | 0 | 6dB |

ob	RBS Pattern 		     (for Server only)

This field displays the total RBS pattern found during the connection.

oc	Rate drop due to RBS   (for Server only)

This field displays the rate dropped due to RBS (in Kbps).

pa	Disconnect reason

This field displays a code which describes the reason for disconnection.  The values can be decoded using the following table:

| DISCONNECT REASON | VALUE |
| --- | --- |
| LOCAL REQUEST | 0 |
| LOCAL REQUEST | 1 |
| LOCAL REQUEST | 2 |
| LOCAL REQUEST | 3 |
| CARRIER LOSS | 4 |
| NO ERROR CORRECTION | 5 |
| NO ERROR CORRECTION | 6 |
| INCOMPATIBLE PROTOCOL | 7 |
| INCOMPATIBLE PROTOCOL | 8 |
| INCOMPATIBLE PROTOCOL | 9 |
| INCOMPATIBLE PROTOCOL | 10 |
| INCOMPATIBLE PROTOCOL | 11 |
| LINK DISCONNECT | 12 |
| EXCESSIVE RETRANSMISSIONS | 13 |
| INCOMPATIBLE PROTOCOL | 14 |
| DTR LOSS | 15 |
| GSTN CLEARDOWN | 16 |
| INACTIVITY TIMER | 17 |
| INCOMPATIBLE SPEEDS | 18 |
| BREAK DISCONNECT | 19 |
| KEY ABORT | 20 |
| NONE | 21 |
| NO CONNECT | 22 |
| 3 RETRAINS | 23 |

This document was written for V1.054.  Previous and subsequent versions may be different.