Last modified: March 6, 1998  (draft version )

Workstation using an ISDN Terminal Adapter to Route IPX over L2TP tunnel

This application note explains how to set up a workstation using an ISDN terminal adapter (TA) to connect to local ISP and route IPX through a L2TP tunnel created between the local ISP_A to corperate’s Prestige P128+(B).  The ISP_A must provide L2TP service in order to create tunnel to corperate’s network. The ISP’s router will be acting as LAC (L2TP Access Concentrator) to answer calls from dial-in users, and the Prestige P128+(B) will be LNS ( L2TP Network Server) to accept L2TP tunnel request from LAC.  Note that Prestige can be both LAC and LNS, and this application shows that P128+(A) is the LAC in ISP_A, and P128+(B) is the LNS located in corperate network. P128+(B) is setup as Proxy mode L2TP.

When a call from Win95 to ISP_A’s P128+(A), P128+(A) will start PPP LCP negotiation with Win95, and also undertake partial authentication to see if the dial in user need to setup a tunel to his/her corperate network or just a local ISP user. If the user is defined as a L2TP user, then P128+(A) will setup a tunnel to P128+(B). When the tunnel is up, P128+(A) will pass all PPP packet from Win95 to P128+(B). Win95 will work as if the call is made directly to P128+(B), and route IPX.   ( Note, if you configure IP in Dialup network protocol, then the IP will be able to route over the tunnel to the corperate network too. ) The Windows 95 dial-up network should use the structured user name as suggested in L2TP RFC (e.g. username@company.com ).  P128+(A) will use the ‘company.com’ to decide if the user is a L2TP user or not.

In this configuration, the workstation has NetWare client software so that you will be able to log into the NetWare server.  Three configuration setup needed, and they are the workstation and the Prestige P128+(A) and P128+(B).

I.	Workstation Setup (NetWare Client)

To set up the workstation (W/S), you will need to set the following parameters:

- Server Name - In some cases, you may need the name that has been configured for the NetWare server you wish to login to.  In the previous example, that would be ‘Server_1’.

The procedure for configuring these parameters for your workstation may differ depending on the type of IPX client software you are using on your workstation.  If you are unfamiliar with how to set these parameters, you can refer to the technical notes corresponding to your software.

- Stampede in a Windows 3.X PC

- Windows 95/NT - see section ‘Remote Access, Telecommute’

II.	Prestige P128+(A) Setup

In order to configure your Prestige for this application, make sure you have the following menus configured correctly.

Note that the fields are indicated in bold type.

Menu 1:

Menu 1 - General Setup

System Name= ISP_A

Location= ISP

Contact Person's Name= admin

Route IP= Yes

Route IPX= No

Bridge= No

Press ENTER to Confirm or ESC to Cancel:

- In menu 1, make sure the Route IP fields are set to ‘Yes’.

Menu 10:

Menu 10 - Tunnel Endpoint Setup

1. zyxel.com

2. hitech.com

3. ________

4. ________

Enter Node # to Edit:

- Create one tunnel to the corperate’s router P128+(B).

- The tunnel name is used to identify which tunnel P128+(A) should create to when a L2TP dial-in user dialed in. If a user’s login name is userA@zyxel.com, the P128+(A) should create a tunnel to the 1st configured endpoint as shown above. If a user login as userA@hitech.com, then P128+(A) should create the tunnel to the 2nd configured tunnel endpoint.

Menu 10.1:

Menu 10.1 - Tunnel Endpoint Profile

Endpoint Name= zyxel.com

Active= Yes

My Host Name= isp_a

Peer Host Name= p_corp

shared Secret= ********

IP Address= 200.1.1.175

Press ENTER to Confirm or ESC to Cancel:

- Specify a name for identification of the tunnel.

- Specify the ‘Host Name’ information for identify which endpoint.  Specify the ‘Secret’ for the tunnel, and note that the secret must be the same configured in both P128+(A) and P128+(B).

- Specify the IP address that the tunnel is going to create to.  This IP address must be a legal IP address from the Internet.  Note: In this example that ISP_A needs to reach the network in corperate, so you must provide an IP address that P128+(A) knows how to route the tunnel setup packet to.

Menu 14:

- No user information needed to be configured for L2TP dial-in users.

II.	Prestige P128+(B) Setup

In order to configure your Prestige for this application, make sure you have the following menus configured correctly.

Note that the fields are indicated in bold type.

Menu 1:

Menu 1 - General Setup

System Name= P_CORP

Location= cupertino

Contact Person's Name= ll

Route IP= Yes

Route IPX= Yes

Bridge= No

Press ENTER to Confirm or ESC to Cancel:

- In menu 1, make sure the Route IPX and Route IP fields are set to ‘Yes’.

Menu 10:

Menu 10 - Tunnel Endpoint Setup

1. isp_a

2. ________

3. ________

4. ________

Enter Node # to Edit:

- Setup one tunnel endpoint to P128+(A).

Menu 10.1:

Menu 10.1 - Tunnel Endpoint Profile

Endpoint Name= isp_a

Active= Yes

My Host Name= p_corp

Peer Host Name= isp_a

shared Secret= ********

IP Address= 128.1.2.1

Press ENTER to Confirm or ESC to Cancel:

- Specify a name for identification of the tunnel

- Specify the ‘Host Name’ information for identifing which endpoint.  Specify the ‘Secret’ for the tunnel, and note that the secret must be the same configured in both P128+(A) and P128+(B).

- The IP address should be the IP address of P128+(A) in ISP_A.

Menu 11:

Menu 11 - Remote Node Setup

1. ISP_B

2. ________

3. ________

4. ________

Enter Node # to Edit:

- Setup one remote node to access Internet.

Menu 11.1 (Remote Node to ISP)

Menu 11.1 - Remote Node Profile

Rem Node Name= ISP_B

Active= Yes                          Route= IP

Call Direction= Outgoing             Bridge= No

Tunneling Mode= None

Endpoint Index= N/A                Edit PPP Options= No

Rem IP Addr= 0.0.0.0

Incoming:                            Edit IP/IPX/Bridge= No

Rem Login=                         Telco Option:

Rem Password= ********               Transfer Type= 64K

Rem CLID= N/A                        Allocated Budget(min)= 0

Call Back= N/A                       Period(hr)= 0

Outgoing:                            Session Options:

My Login= user                       Input Filter Sets=

My Password= ********                Output Filter Sets=

Authen= CHAP/PAP                     Call Filter Sets=

Pri Phone #= 30001                   Idle Timeout(sec)= 300

Sec Phone #=

Press ENTER to Confirm or ESC to Cancel:

- Normal IP remote needs to setup the Tunneling Mode to ‘None’.

Menu 13:

Menu 13 - Default Dial-in Setup

Telco Options:                       IP Address Supplied By:

CLID Authen= None                    Dial-in User= Yes

IP Pool= No

PPP Options:                             IP Start Addr= N/A

Recv Authen= CHAP/PAP                  IP Count(1,2)= N/A

Compression= No

Mutual Authen= No                  IPX Net Num Supplied By:

PAP Login= N/A                     IPX Pool= Yes

PAP Password= N/A                    IPX Start Net Num= 12345678

Multiple Link Options:                 IPX Count(2,16)= 2

Max Trans Rate(Kbps)= 128

Session Options:

Callback Budget Management:            Input Filter Sets=

Allocated Budget(min)= 0             Output Filter Sets=

Period(hr)= 0                        Idle Timeout= 300

Press ENTER to Confirm or ESC to Cancel:

- If you want to assign a specific IPX network number to the workstation, set IPX Pool to ‘Yes’.

- Enter the starting IPX network number in the IPX Start Net Num. field.  The Prestige will assign the network numbers from this pool.  For example, ‘12345678’.

Menu 14:

Menu 14 - Dial-in User Setup

1. userA@zyxel.com

2. test

3. win95@zyxel.com

4. ________

5. ________

6. ________

7. ________

8. ________

Enter Menu Selection Number:

- Confiugre dial in user as if a direct call into P128+(B).

Menu 14.1:

Menu 14.1 - Edit Dial-in User

User #: 1

User Name= userA@zyxel.com

Active= Yes

Passwd= ********

Callback= No

Phone # Supplied by Caller= N/A

Callback Phone #= N/A

Rem CLID=

Idle Timeout= 300

Press ENTER to Confirm or ESC to Cancel:

- The User Name and Passwd fields should be set to the login user name and password that the workstation will provide when dialing in to the ISP_A Prestige P128+(A).

- Set the Active field to ‘Yes’.