OPSD BIOS ENGINEERING

Intel Corporation, 5200 NE Elam Young Parkway
Hillsboro, OR  97214

DATE:	July 30, 2026

TO:	OMCO Business Unit

CC:	iPG / OPSD BIOS Engineering

FROM:	OPSD BIOS Engineering

SUBJECT:	4L3TT0X0.86 Custom P03-0007

About This Release

P03-0007

- Banner Changed to P03-0007.

- Fixed an issue where the system would hang in OSR2 if the system was configured with an LS-120 drive but no legacy floppy.

B-0007

- Banner changed to B-0007

- Added Intel Copyright sign on string.

- Added fix for Hauppauge WinCast TV card fails in WIN95 when PnP OS=YES

- Fixed issue where an IDE Channel that is not populated with a drive may erroneously create a boot delay looking for drives.

P02-0006

- Banner changed to release P02-0006.

- Added the quickBoot feature

- Removed setup option to allow enabling/disabling of 32 bit FPIO of the channels.

- PIIX4 app note 5. Bus-master DMA support for hard drives and CDs, and CD PIO modes.

- Changed POST memory test to display in MByte increments

- Fix for suspend in 1999, resume in 2000 with timer

P01-0006

- Banner changed to release P01-0006.

B-0006

- Banner changed to release B-0006.

- Fixed a problem where the mouse was not reported correctly in win95. In Win95 device manager a red "X" was displayed even if the mouse works OK.

- Fixed issue of system hanging when it has been powered up via a modem ring and another ring comes in during POST.

Beta-0005

- Fixed Bootblock issue with Samsung/NEC SDRAMS

- Fixed Keyboard Keyclick feature

- Allow hard disk access to prevent Standby timeout

- Enabled 64MBit Technology SDRAM support

- Fixed check for "old scan type" user ROM.

- Fixed IntelPCIDiag APM test failure for Resume on Ring

Beta 0001

Features/Errata Fixed In This Release

Features Included In This Release

Setup Changes

Beta-0004

- Fixed Keyboard Keyclick feature

Password Changes

Language Support

Hard Disk Changes

- Allow hard disk access to prevent Standby timeout

Plug and Play (Windows 95, ISA Configuration Utility, etc.) Changes

APM  changes

Beta-0004

- Fixed IntelPCIDiag APM test failure for Resume on Ring

New Int 15h Extensions

Other Changes

Known Errata With This Release

Features Still To Be Added / Comments About This Release