Intel® LB440GX 2U Rack Server Platform
Supported Processors

Two things determine processor speeds that are supported on the LB440GX 2U rack mount server platform: Product code and BIOS version. The product code is located on the bottom of the chassis on a white label, or on the white label located on the short edge of the shipping box. You can also determine the product code by taking the top of the chassis off and comparing the PBA# of the baseboard to the following table. The PBA# is found on a small white label near the outer edge of the baseboard.

| 704293-501 or greater | Product code is SLWBRF5 |
| --- | --- |
| 704293-409 or less | Product code is SLWBR |

Once you know the product code, use the following table to determine the supported processors. You will need the BIOS version listed (#) or better to support the particular speed. For example, to support the Pentium® III 600E processor, the product code will need to be SLWBRF5 and the BIOS will need to be version 9.1 or better.

| Pentium® III Processors |  |  |  |  |
| --- | --- | --- | --- | --- |
| Speed | Product Code |  |  |
|  | SLWBR | SLWBRF5 |
| 800EB MHz | NO | NO |
| 800 MHz | NO | NO |
| 750 MHz | NO | NO |
| 733 MHz | NO | NO |
| 700 MHz | NO | YES (9.2) |
| 667 MHz | NO | NO |
| 650 MHz | NO | YES (9.2) |
| 600EB MHz | NO | NO |
| 600E MHz | NO | YES (9.2) |
| 600B MHz | NO | NO |
| 600 MHz | YES (7.3) | YES (7.3) |
| 550E MHz | NO | NO |
| 550 MHz | YES (7.0) | YES (7.0) |
| 533EB MHz | NO | NO |
| 533B MHz | NO | NO |
| 500E MHz | NO | NO |
| 500 MHz | YES (7.0) | YES (7.0) |
| 450 MHz | YES (7.0) | YES (7.0) |

| Pentium® II Processors |  |  |  |  |
| --- | --- | --- | --- | --- |
| Speed | Product Code |  |  |
|  | SLWBR | SLWBRF5 |
| 450 MHz | YES (7.0) | NO |
| 400 MHz | YES (7.0) | NO |
| 350 MHz | YES (7.0) | NO |