- WPD SW ENGINEERING

Intel Corporation, 2800 Center Dr.
DuPont, WA 98327-5050

DATE:	Oct. 30, 1998

TO:	WPD Business Unit

CC:	

FROM:	WPD SWE Engineering

SUBJECT:	4M4SG0X0.86E Standard BIOS

- About This Release

- 4M4SG0X0.86E.0026.B.9810301106

- Features/Errata Fixed In This Release

- B-0026 (Tanner 500mhz support BIOS)

- Forced C6C/CK1 cacheable address range to 64GB always (was 512mb).

- Improved L2 knobs for Katmai/C6C (Tanner) 450 and 500MHz.

- Improved L2 knobs for Deschutes/C6C (P2XP) 400 and 450MHz.

- Added D46 errata workaround for Pentium II Xeon processor.

- P07-0025

- Changed 400MHz and 450MHz L2 latency to 10,1,1,1

- Fixed DRAM throttling control registers

- Fixed El Torito formatted CD-rom boot

- Fixed Recovery Mode when On PME set to Power On

- Fixed NMI enable

- Fix for WHQL PNP BIOS test failure

- Fixed APM standby problem in Win95

- Updated processor microcode for Intel Pentium ® Xeon ™ processors.

- Fixed intermittent wake On PME

- Fix for WHQL unreported memory failure under ACPI

- Added quiet boot support (small user logo support).

- Added SMBIOS 2.2 compliance.

- P06-0023

- Fix for WHQL APM 1.2 test failure (Suspend Resume Timer via RTC Alarm)

- Fix for WHQL APM 1.2 test failure for Suspend using IBM 10G IDE Drive

- Fix for WHQL ACPI test failure (Unreported memory 0FEC0 APIC)

- P05-0020

- Fixed ACPI wake on KBD

- Fixed wake on Ring

- Fixed STBL manufacturing test

- Fixed SMBIOS type 17 structures

- P04-0019

- Changed Manufacturing Default for the AC Power Loss CMOS bit to “Always On”.

- P03-0018

- Fixed problem that would cause PCI configuration space corruption resulting in system hangs.

- Fixed ACPI IRQ routing for USB device.

- Fixed ACPI/RTC compliance issue.

- Fixed issue related to IRQ 12 availability with disabled PS/2 mouse.

- Fixed SETUP access security inconsistencies for unattended start, clear user password and user setup access.

- Clarified boot order message in SETUP help text.

- Added Memory Decreased error logging and functionality.

- Removed ECC memory SETUP option.  The system now requires ECC memory.

- Fixed SETUP hard drive menu corruption.

- Fixed shutdown hang with some non-compliant ATAPI devices.

- Fixed SETUP display for HDD > 8.4 G.

- Fixed DMI memory structures.

- P02-0017

- Boot time for certain ATAPI devices reduced.

- Locked cmos.

- Hang when Win98 or Win95 shutdown fixed, related to ATAPI/IDE devices.

- ACPI ring indicator functional.

- Hard Disk Drive menu corruption fixed.

- ECC/Non-ECC removed from Setup, Only ECC system memory allowed.

- Error message for memory decreased since last boot added.

- P01-0016

- Updated processor microcode for Intel Pentium ® Xeon ™ processors.

- Fix IRQ routing problem.

- B-0015

- Fix for IRQ9/AP CPU thrash

- Fix for PC97b WHQL v.7.6 in ACPI mode fails unreported I/O test failure.

- Fix for PC97b WHQL v.7.6  in ACPI mode fails on MS440GX  ECP printer port tests PC9x test failure

- Fix for PC97 ACPI test failure

- GX support for 2G added

- PnP OS setup question default to "NO"

- Removal of the user option to turn ACPI off within BIOS setup

- Added in Y2K

- Fix for ECC DMI event logging

- Fix for AC Power Loss

- Removed Option to boot in config mode

- B-0014

- Fix for APM hang at 400+ MHz

- B-0013

- Support for B0 Intel Pentium ® Xeon ™ Processors update

- B-0012

- Fix for MTRR programming above 512M

- Fix for PC97 compliance HCT test. Unreported Memory & IO test now pass.

- ACPI fan control now working and Soft SMI capability.

- Manufacturing Defaults for Resume on Ring changed.

- BIOS Spec Update rev 2.5 item E2 changed for 440GX chipset. (Performance increase)

- Resource file fix for CS4235 Audio chip

- Removed memory hole from setup

- Support for B0 Intel Pentium ® Xeon ™ Processors including strings.

- 440GX register programming incorporated for revision id checking and x11 column addressing.

- B-0011

- Readjusted fix for functional test issue with cache being disabled on first boot as stated below.

- Changed Mfg default for AC Loss to always Power on.

- B-0010

- Fixed Mfg Functional test issues with cache being disabled on first power up of board.

- Fixed Resume on ACLoss hang for “Stay Off” option.

- B-0009

- New Boot Block to support SPD added. (Fixes anomaly with Registered DIMM’s not working).  This Boot block also supports LS-120 drives with 3.5 inch legacy floppy media only.

- B-0008

- Changes for Fab B SMBUS addresses (DIMMs and Alertpack) and GPIO changes

- Note: Beta BIOS will not be supported on Fab A.

- A-0007

- Added support for Alert Pack

- Fix for Event Logging

- New Setup Organization

- Fix for Audio setup question not working. (Was always enabled)

- Fix for “On LAN”, “On PME”, and “On Modem Ring” setup questions functionality.

- Fix for “On AC Loss” setup question functionality.

- Fix for Flash always being write enabled.

- Fixed all DMI information and DMITEST passage.

- Fixed APMTEST hang problem.

- Fixed Invalid CPU speed settings in Configuration Mode.

- A-0006

- Boot block change only to allow for 440GX to work. (NOTE: Testing shows that only un-registered DIMMS seem to work with any Boot block released to this point. This includes all 440BX board builds as well.)

- A-0005

- New Features/Fixes

- Significant Phoenix Core changes incorporated. (VCM)

- Spanish and French Language Support

- Hard Disk Pre-delay

- Embedded PXE Support

- Legacy USB Support

- Extended Security Support

- 440BX Update S20, S28

- Wake on LAN

- ECC support (chipset only)

- Features not yet implemented

- Alert Pack

- GX Chipset

- ACPI

- Known Bugs

- DMI Event Logging does not function correctly

- Some DMI strings are not present

- BEL APM Test hangs the system

- Mixing DIMMS hangs the system at post code 28h

- Cache anomalies when put in configuration mode via jumper

- Resume on AC Loss - Stay Off test fails

- APM Front Panel Sleep Button

- A-0004

- Incorporated items in 440BX BIOS Spec. Update rev 2.3 (S3, S11, S17, S19, S27, S29, E2)

- A-0003

- Corrected GPO outputs for SIO 307 to TTL levels.

- A-0002

- Default L2ECC to enabled. Setup question removed

- Processor ucode id 19 added.

- SPD memory detection vs. Dynamic

- Fixed GPO for SDRAM EPROM write protect.

- Defaults for LAN, audio, and heceta changed to enabled.

- GPO’s for SIO 307 changed to TTL level outputs

- A-0001

- Using DYNAMIC memory detection to work due to problems with SPD which will eventually be put back into the bios.

- Changed the GPIO value for the SW_PCI_RESET signal to the Arbiter to a TTL compatible mode verse Open Drain