SoundFontÒ Technical Specification

Ò

Appendix A3.1:

SoundFont Management System

Applications Programming Interface (API)

For Microsoft Windows 95

# About this Document

## Revision History

Revision History:

| Version | Date | Description |
| --- | --- | --- |
| 1.1 | October 7, 1997 | Add SF_GetLoadedBankPathname |
| 1.0 | July 28, 2026 | Initial Release |

## Disclaimers

THIS SPECIFICATION IS PROVIDED “AS IS” WITH NO WARRANTIES WHATSOEVER INCLUDING ANY WARRANTY OF MERCHANTABILITY, FITNESS FOR ANY PARTICULAR PURPOSE, OR ANY WARRANTEE OTHERWISE ARISING OUT OF ANY PROPOSAL, SPECIFICATION, OR SAMPLE.

A LICENSE IS HEREBY GRANTED TO COPY, REPRODUCE, AND DISTRIBUTE THIS SPECIFICATION FOR INTERNAL USE ONLY.    NO OTHER LICENSE EXPRESS OR IMPLIED, BY ESTOPPEL OR OTHERWISE, TO ANY OTHER INTELLECTUAL PROPERTY RIGHTS IS GRANTED OR INTENDED HEREBY.

AUTHORS OF THIS SPECIFICATION DISCLAIM ALL LIABILITY, INCLUDING LIABILITY FOR INFRINGEMENT OF PROPRIETARY RIGHTS, RELATING TO IMPLEMENTATION OF INFORMATION IN THIS SPECIFICATION.    AUTHORS OF THIS SPECIFICATION ALSO DO NOT WARRANT OR REPRESENT THAT SUCH IMPLEMENTATION (S) WILL NOT INFRINGE ON SUCH RIGHTS.

This preliminary document is being distributed solely for the purpose of review and solicitation of comments.    It will be updated periodically.    No products should rely on the content of this version of the document.

SoundFontÒ and the SoundFont logo are registered trademarks of E-mu Systems, Inc.    E-mu Systems licenses the “SoundFont Compatibility” logo for a nominal fee; please contact E-mu’s SoundFont administrator by FAX at (408) 439-0392 for more information.    Users of the information contained herein should refer to applications conforming to the specification and sound banks used within those applications as “SoundFont Compatible,” with appropriate acknowledgment of trademark ownership.

Sound BlasterÒ is a registered trademark of Creative Technology, Ltd. Advanced Wave Effects, AWE32 and AWE64 are trademarks of Creative Technology, Ltd. WindowsÒ and Windows 95 are registered trademarks of Microsoft Corporation.

## Updates and Comments

Please visit http://www.soundfont.com for specification updates, and please send comments via e-mail to soundfont@emu.com.

## Table of Contents

0.	About this Document	1

0.1	Revision History	1

0.2	Disclaimers	1

0.3	Updates and Comments	2

0.4	Table of Contents	3

1.	Introduction	5

1.1	Scope and Intended Purpose of this Document	5

1.2	Document Organization	5

1.3	SoundFont API Definition and Objectives	5

2.	SoundFont API Overview	5

2.1	SoundFont API Description	5

2.2	SoundFont Management Services API	5

2.3	SoundFont Sound Event Control Services API	6

3.	SoundFont Management System API Details	6

3.1	SoundFont Management Services API Specification	6

3.1.1	Theory of Operation	6

3.1.1.1	Overview	6

3.1.1.2	Linking and Receiving of the Entry Point Function	6

3.1.1.3	SoundFont Master Manager Data Structures	7

3.1.1.4	Manager MIDI Space Index Ranges	7

3.1.2	Error Codes	8

3.1.2.1	General Error Messages	8

3.1.2.2	API Error Messages	8

3.1.2.3	Invalid Parameter Messages	9

3.1.2.4	Resource Contention Messages	10

3.1.3	API Version Functions	10

3.1.3.1	SF_QueryInterface	10

3.1.4	Global Functions	11

3.1.4.1	SF_GetNumDevs	11

3.1.5	Device Information and Contention Messages	12

3.1.5.1	SF_GetDevCaps	12

3.1.5.2	SF_GetRouterID	13

3.1.5.3	SF_IsDeviceFree	13

3.1.5.4	SF_Open	14

3.1.5.5	SF_Close	14

3.1.6	Synthesizer Emulation Functions	15

3.1.6.1	SF_GetAllSynthEmulations	15

3.1.6.2	SF_GetSynthEmulation	16

3.1.6.3	SF_SelectSynthEmulation	16

3.1.7	Bank Loading Management Functions	17

3.1.7.1	SF_LoadBank	17

3.1.7.2	SF_GetLoadedBankDescriptor	18

3.1.7.3	SF_GetLoadedBankPathname	19

3.1.7.4	SF_ClearLoadedBank	20

3.1.8	Preset Loading and Management Functions	21

3.1.8.1	SF_LoadPreset	21

3.1.8.2	SF_GetLoadedPresetDescriptor	22

3.1.8.3	SF_ClearLoadedPreset	23

3.1.9	Waveform Loading and Management Functions	24

3.1.9.1	SF_LoadWaveform	24

3.1.9.2	SF_ClearLoadedWaveform	25

3.1.10	Miscellaneous Functions	26

3.1.10.1	SF_IsMIDIBankUsed	26

3.1.10.2	SF_QueryStaticSampleMemorySize	26

3.2	SoundFont Master MIDI Data Router API Specification	27

3.2.1	Theory of Operation	27

3.2.2	Routing Command Overview	28

3.2.2.1	Routing (System Exclusive Message) Command Format	28

3.2.2.2	Routing Command General Description	28

3.2.3	Global Routing Commands	29

3.2.3.1	Reset All Routings to Default	29

3.2.4	SoundFont Compatible Device Routing Commands	29

3.2.4.1	Route All Events to SoundFont/MIDI Device	29

3.2.4.2	Route MIDI Channel to SoundFont/MIDI Device	29

3.2.4.3	Route Bank and Program Data to SoundFont/MIDI Device	30

4.	Glossary	31

# Introduction

## Scope and Intended Purpose of this Document

This document is the definitive source of the SoundFont Management System API, used in conjunction with the SoundFont Management System for Microsoft Windows 95, an extension of the SoundFont 2.x standard.    This document should provide complete and accurate information to allow any application to correctly utilize the features provided by a SoundFont Compatible Devices running under the Microsoft Windows 95 Operation System.

It is expected that the reader of this document has read and understood all documents prior to this one, including the SoundFont Technical Specification, and Appendix A1 describing SoundFont Management System.

This document is not intended to provide any information on the design or implementation of music synthesizers or applications.

## Document Organization

This document is organized such that sections 1 and 2 give introductory information about the SoundFont API. Section 3 provides a detailed description of the SoundFont API.    This section will ultimately serve as reference, but can be scanned in order to provide sufficient detail for any level of understanding.    The alphabetical glossary in section 4 can be used as a reference for any unfamiliar or confusing terminology.

## SoundFont API Definition and Objectives

The SoundFont Application Programming Interface (API) is the single mechanism through which any application may utilize the features that SoundFont Compatible Devices provide, regardless of which or how many SoundFont Compatible Devices are running on the same system.

This specification defines the mechanism with which a Windows 95 application may query the availability of SoundFont Compatible Devices which may exist on a system, identify individual devices, and use the SoundFont features on any number of SoundFont Compatible Devices. The mechanism also allows such applications to use MIDI control features on any MMSystem compatible Device.

An overview of the SoundFont Management System is described Appendix A1.

# SoundFont API Overview

## SoundFont API Description

The SoundFont API is split into two parts:

- SoundFont Management Services

- SoundFont Sound Event Control Services

## SoundFont Management Services API

SoundFont Management Services is defined as those functions required for applications to choose which SoundFont based sounds are to be used with their application at a given time, how they may be loaded and unloaded, and how the capabilities and current state of a SoundFont compatible device may be queried.

SoundFont Management Services are provided through an API provided through a table of functions exported by the SoundFont Master Manager (SFMAN32.DLL).

## SoundFont Sound Event Control Services API

SoundFont sound event control is defined as those functions required for applications to select among a palette of currently loaded SoundFont sounds, turn such sounds on and off, and control their timbre in real-time.

SoundFont Sound Event Control Services are provided via MIDI through the SoundFont MIDI Data Router. Commands passed to the SoundFont MIDI Data Router are in turn relayed to one or more SoundFont and/or MIDI Compatible Devices on the system.

The SoundFont MIDI Data Router also intercepts special MIDI System Exclusive Messages which handle the routing of this data. These messages are not relayed to SoundFont Compatible Devices on the system, rather they are commands to the MIDI Data Router to filter ordinary MIDI data and route specific data to a selected set of SoundFont and/or MIDI Compatible Devices. The routing selection may be sensitive to either MIDI channel or current Bank/Program selection.

In other words, a single MIDI data stream may be used to send data to one or more devices. Data corresponding to individual MIDI channels in that stream may be sent to any of those devices.

Also, MIDI data following specific Bank and Program select settings may be sent to any of those devices.

The latter allows an application to load the span of all SoundFont sounds used in a particular application or sequence into any of a number of SoundFont compatible device and anywhere in MIDI space. With that, a pre-defined standard type-0 MIDI sequence stream (with pre-determined MIDI space information and no individual device information) may be used to trigger and control the correct sounds. The routing commands, which must be sent dynamically based on how the SoundFont based sounds are actually loaded in the various SoundFont Compatible Devices, provides the logic necessary to make this possible.

# SoundFont Management System API Details

## SoundFont Management Services API Specification

### Theory of Operation

#### Overview

The API provides parent applications with a set of functions, accessing the features of any number of properly installed SoundFont Compatible Devices. The interface to these sets of functions is the SoundFont Master Manager, or SFMAN32.DLL.

The data structures and APIs defined in the following section may be found in the <sfman.h> header file. The APIs are supplied through C function tables.

It is anticipated that the SoundFont Master Manager may require multiple non-backward compatible revisions of APIs as SoundFont support matures. It is also MANDATORY that the SoundFont Master Manager support all previous released APIs with any such revision.

#### Linking and Receiving of the Entry Point Function

The SFMAN32.DLL must be dynamically linked with the application using the LoadLibrary command defined in <windows.h>.

The SFMAN32.DLL exports a C compatible function table, SFManager. This function table exports a single function called SF_QueryInterface. This function may be used to determine which API version(s) the SFMAN32.DLL supports, and may be used to receive function tables corresponding to past or present revision of the SoundFont Master Manager API which was ever supported by SFMAN32.DLL.

As of the date of this document, there is one API revision available. The SF_QueryInterface function may be used to obtain the function table for API revision 1.0. Applications can access services provided by the SFMAN32 though the API revision 1.0 function table as specified in the sections below.

Once the application has obtained the entry point for the version 1.0 function table, it can access to the services provided.

#### SoundFont Master Manager Data Structures

Some of the parameters to the message function take the form of pre-defined structures. The user applications will have to allocate these structures before passing in as parameters. These structures have the following forms.

##### The MIDI Location Data Structure

typedef struct {

WORD                            m_BankIndex;

WORD                            m_PresetIndex;

} CSFMIDILocation, *PSFMIDILOCATION;

The CSFMIDILocation structure is used to specify locations in MIDI space for various functions.

##### The Buffer Object Data Structure

typedef struct {

DWORD                          m_Size;

LPSTR                          m_Buffer;

DWORD                          m_SizeUsed;

WORD                            m_Flag;

} CSFBufferObject, *PSFBUFFEROBJECT;

The CSFBufferObject structure is used to pass arbitrary data used with the various functions. Not all of the fields within the structure will be used every time. Unused members must be set to zeroes explicitly. It is the duty of both application and DLL to retrieve and set the values from the appropriate parameters.

##### The SoundFont Compatible Device Capabilities Structure

typedef struct {

WORD              m_SizeOf;

DWORD            m_DeviceRouterID;

DWORD            m_DevCaps;

DWORD            m_MoreDevCaps;

DWORD	    m_RomID;

DWORD	    m_RomVer;

CHAR              m_DeviceName[40];

CHAR              m_DeviceManagerName[20];

CHAR	    m_DeviceManagerEntry[20];

} CSFCapsObject, *PSFCAPSOBJECT;

The CSFCapsObject structure can be used to identify the capabilities a given SoundFont compatible device on the system. For details on these fields, please see the description of SF_GetDevCaps.

All the above mentioned structures are already provided in a header file called SFMAN.H, which should accompany this document.

#### Manager MIDI Space Index Ranges

This section describes the valid ranges of MIDI space indices used within the device manager API.

The Manager API supports loading SoundFont banks in to MIDI space corresponding to the MSB (CC0) value. The LSB (CC32) value is not supported. Valid bank indices span the values 0 through 127, plus the percussive bank values.

When a bank index specifies percussive MIDI space as a SOURCE (such as a sound encapsulated in a file) or as a TARGET (location in MIDI space in which a sound is loaded), the bank index value used should be SFMAN_PERCUSSIVE_BANK.

Examples:

Load:

Sound indicated in file by Bank 1 Program 2 into

MIDI Space location indicated by Bank 3 Program 4

Source Bank = 1, Source Preset = 2,

Target Bank = 3, Target Preset 4

Load:

Sound indicated in file by Percussive Section Program 0 into

MIDI Space location indicated by Bank 1 Program 2

Source Bank = SFMAN_PERCUSSIVE_BANK, Source Program = 0,

Target Bank = 1, Target Preset = 2

Load:

Sound indicated in file by Bank 1 Program 2 into

MIDI Space location indicated by Percussive Section Program 5

Source Bank = 1, Source Program = 2,

Target Bank = SFMAN_PERCUSSIVE_BANK, Target Preset = 5

### Error Codes

The error codes generated by a SoundFont Manager are described below.    They can be divided into three categories:

- General Error Messages

- Invalid Parameter Messages

- Resource Contention Messages

#### General Error Messages

SFERR_NOERR	No error message.    This message will be display most of the time to indicate a successful operation.

SFERR_UNDEFINE_ERROR	A reserved error message indicating an error not defined during development.

#### API Error Messages

SFERR_LOAD_DLL_FAIL	Indicates that the attempt to load a SoundFont Compatible Device Manager failed, therefore that device may not be used.

SFERR_GET_ENTRYPOINT_FAIL	Indicates that the attempt to receive the entry point of a SoundFont Compatible Device failed, therefore that device may not be used

SFERR_DEVICE_NOT_FOUND	Indicates that the index to the SoundFont Compatible Device was out of range.

SFERR_INTERFACE_NOT_SUPPORTED	Indicates that the function table corresponding to the requested version number is not supported in this version of SFMAN32.DLL.

#### Invalid Parameter Messages

SFERR_SYNTH_INVALID	An invalid Synthesizer Emulation index is specified.    This error occurs when the SoundFont Compatible Device does not have a definition for the specified Synthesizer Emulation.

SFERR_BANK_INDEX_INVALID	An invalid User Bank index is specified. This error occurs when the application points to a bank index higher or lower than the legal range. This range is contingent on the capabilities and restrictions of the individual device.

SFERR_PATHNAME_INVALID	An invalid path name to a file is specified.    This happens when an operation, which requires a user file or a predetermined path, cannot locate the desired file.

SFERR_FORMAT_INVALID	A user object module (such as a file corresponding to a full path to a bank name) has an incorrect or unrecognized data format.

SFERR_PRESET_INDEX_INVALID	An invalid MIDI Program or preset index is specified.    This index is only supported within the range of 0 to 127, i.e. 128 presets.

SFERR_ACCESS_NOT_PERMITTED	Access is denied to the SoundFont Compatible Device, likely because another application is using the device.

SFERR_VERSION_INVALID	Indicates that one or more of the SoundFont Compatible Device components are invalid.

SFERR_SUPPORT_INVALID	Indicates that the SoundFont Compatible Device does not support the specified function.

SFERR_DEVICE_INVALID	Indicates that the SFDEVINDEX value passed into the API function was invalid.

SFERR_INVALID_SOUND_ROM	Indicates that the SoundFont selection is dependent on a wave-table ROM that the SoundFont Compatible device either does not have or does not support.

SFERR_NOT_STATIC_MEMORY	Indicates that the synthesizer doesn't support static type memory, and therefore does not support the Query Sound RAM function.

#### Resource Contention Messages

SFERR_DLL_BUSY	Notifies the application that another application is already accessing a given SoundFont Compatible Device Manager and is still accessing it.

SFERR_DEVICE_BUSY	Indicates that the SoundFont Compatible Device MIDI Output driver is in use by another application.

SFERR_RESOURCE_INSUFFICIENT	Indicates that the calling application has provided insufficient memory space that is necessary to complete the current query operation.

SFERR_SYSMEM_INSUFFICIENT	Indicates that the SoundFont Manager is unable to allocate any more memory from Windows for internal use.

SFERR_SOUNDMEM_INSUFFICIENT	Indicates that the SoundFont Compatible Device has insufficient resources to store the waveforms required to complete a load operation.

### API Version Functions

#### SF_QueryInterface

Syntax

LRESULT SF_QueryInterface ( INTERFACEID id, PDWORD pdwTable )

Actions

An application calls this function when it wishes to receive the function table for a particular API version.

Parameters

id

Set this to the version number of the API which the client wishes to use. Each interface version has a corresponding unique ID value which is defined in the header file for that interface version.

pdwTable

The Manager fills this location with a pointer to the function table for the corresponding API.

Remarks

This document corresponds to API version 1.0, or id=ID_SFMANL100API.

Parameters passed in as pointers must be valid memory allocated by the parent application.

This is the ONE AND ONLY function which is available in the SFManager function table. All other functions in this document are contained within the table returned by this function.

The SoundFont Master Manager will support any and all publicly released API versions less than or equal to the version number returned by this function.

Return

The return value is SFERR_NOERR if the operation is successful, and an error code otherwise. Possible error messages are:

SFERR_INTERFACE_NOT_SUPPORTED

### Global Functions

#### SF_GetNumDevs

Syntax

LRESULT SF_GetNumDevs ( PWORD pwNumDevs )

Actions

An application calls this function when it wishes to determine how many SoundFont Compatible Devices are available and active on the system.

Parameters

pwNumDevs

The SFMAN will    fill this location with the number of SoundFont compatible devices available on the system.

Remarks

Parameters passed in as pointers must be valid memory allocated by the parent application.

This function returns with the TOTAL number of active and working SoundFont compatible devices available in a system.

Return

The return value is SFERR_NOERR if the operation is successful, and an error code otherwise.

See Also	SF_GetDevCaps

### Device Information and Contention Messages

All functions (except SF_Close) may be called without specifically opening a particular device using SF_Open first.

#### SF_GetDevCaps

Syntax

LRESULT SF_GetDevCaps ( SFDEVINDEX sfIdx, PSFCAPSOBJECT pCaps )

Actions

An application calls this function when it wishes to know the capabilities of a particular SoundFont Compatible Device.

Parameters

sfIdx

This is the index to the SoundFont compatible device. The index value ranges from 0 to one less than the return of SF_GetNumDevs.

pCaps

Specify a pointer to CSFCapsObject (PSFCAPSOBJECT) data type. SoundFont manager will fill this structure with the capabilities of the device.

Remarks

Parameters passed in as pointers must be valid memory allocated by the parent application.

The m_SizeOf field of CSFCapsOject structure must be set before calling this API. This is to ensure that future versions of this API will properly initialize or ignore entries of the data structure, given bi-directional compatibility situations. The members of this structure are:

Member		Remarks

m_SizeOf	Contains the size of the CSFCapsObject in bytes.

m_RouterID	Filled with Router ID for the SoundFont Device

m_DevCaps	Filled with the features supported by SoundFont compatible device.
m_MoreDevCaps

m_RomID	Filled with the ROM ID of a device.

( This is only valid for device that uses ROM samples )

m_RomVer	Filled with the ROM version of the device.

( This is only valid for device that uses ROM samples )

m_DeviceName[40]	Filled with the name of the SoundFont Compatible Device.

m_DevMgrName[20]	Filled with the file name of the SoundFont Device Manager

m_DevMgrEntry[20]	Filled with the name of the SoundFont Device Manager Entry Point

The values used in m_DevCaps and m_MoreDevCaps have the following meaning:

m_DevCaps Bit 31: Dynamic Sample Memory Capability: When set to 1, the device stores samples in the same hardware memory pool as that of the application using the sample manager. When set to 0, the device stores samples in dedicated sample memory which cannot be shared with the application.

m_DevCaps Bit 30: Software Synthesis Capability: When set to 0, this indicates that the majority of the fundamental synthesizer activity occurs on a separate processor than that running the application software. When set to 1, this indicates that the majority of the fundamental synthesizer functionality occurs on the same processor as that running the application software.

m_DevCaps Bit 29: Hidden Device Capability: When set to 1, the device uses a “hidden” MIDI Output Device Driver, or on which is not accessible through the MMSystem.

m_DevCaps Bits 0 through 28 and m_MoreDevCaps Bits 0 through 31: Reserved: These bits indicate capabilities which are not documented as of the date of this specification. The default condition for all of these capabilities is 0, which indicates that the device is NOT capable of performing a particular function.

Return

The return value is SFERR_NOERR if the operation is successful, and an error code otherwise.

See Also	SF_GetNumDevs

#### SF_GetRouterID

Syntax

LRESULT SF_GetRouterID ( SFDEVINDEX sfIdx, PDWORD pdwDevID )

Actions

The application calls this function to obtain the value of the SoundFont MIDI Data Router ID used in the Routing Commands.

Parameters

sfIdx

This is the index to the SoundFont compatible device. The index value ranges from 0 to one less than the return of SF_GetNumDevs.

pdwDevID

The Manager fills this location with the proper Routing Device ID given the wIndex. This ID may in turn stripped down to byte size chunks for use in the Routing Commands. The SF_GET_ROUTING_INDEX macro is provided to simplify this process.

Remarks

Parameters passed in as pointers must be valid memory allocated by the parent application.

Return

The return value is SFERR_NOERR if the operation is successful, and an error code otherwise. Possible errors are :

SFERR _DEVICE_INVALID

See Also	SF_GetNumDevs,

SF_GetDevCaps

#### SF_IsDeviceFree

Syntax		LRESULT SF_IsDeviceFree ( SFDEVINDEX sfIdx )

Actions

An application calls this function to check whether a specific SoundFont compatible device is free for use by that application.

Parameters

sfIdx

This is the index to the SoundFont compatible device. The index value ranges from 0 to one less than the return of SF_GetNumDevs.

Remarks

None.

Return

The return value would be SFERR_NOERR if the operation is successful, and an error code otherwise Possible errors are:

SFERR _DEVICE_INVALID

See Also	None.

#### SF_Open

Syntax

LRESULT SF_Open ( SFINDEX sfIdx )

Actions

An application call this function when it wishes to acquire a specific SoundFont device.

Parameters

sfIdx

This is the index to the SoundFont compatible device. The index value ranges from 0 to one less than the return of SF_GetNumDevs.

Returns

The return value would be SFERR_NOERR if the operation is successful and an error code otherwise.    Possible error messages are:

SFERR_DEVICE_DRV_INVALID

SFERR_DLL_BUSY

SFERR_SYSMEM_INSUFFICIENT

SFERR_ACCESS_NOT_PERMITTED

SFERR_VERSION_INVALID

See Also	SF_Close,

SF_GetDevCaps

#### SF_Close

Syntax		LRESULT SF_Close ( SFDEVINDEX sfIdx )

Actions

An application calls this function when it wishes to release control of an acquired SoundFont device.

Parameters

sfIdx

This is the index to the SoundFont compatible device. The index value ranges from 0 to one less than the return of SF_GetNumDevs.

Remarks

The close function will not necessarily unload the SoundFont Device Manager from memory. Unloading depends on whether any more applications are still accessing the Device Manager.

Return

The return value is SFERR_NOERR if the operation is successful, and an error code otherwise.    Possible error message is:

SFERR_DEVICE_INVALID

See Also	SF_Open

### Synthesizer Emulation Functions

#### SF_GetAllSynthEmulations

Syntax		LRESULT SF_GetAllSynthEmulations ( SFDEVINDEX sfIdx,

PSFBUFFEROBJECT pBufObj )

Actions

An application calls this function to request the names of the supported Synthesizer Emulations.

Parameters

sfIdx

This is the index to the SoundFont compatible device. The index value ranges from 0 to one less than the return of SF_GetNumDevs.

pBufObj

Specify a pointer to CSFBufferObject, PSFBUFFEROBJECT. When the application calls the manager, the following fields are used:

Member	Remarks

m_Size	Indicates the size of buffer available.

m_Buffer	Pointer to a buffer area.

Upon returning, the manager will update the following fields:

Member	Remarks

m_SizeUsed	Number of characters from buffer used.

m_Flag	Number of entries written into the buffer.

m_Buffer	String entries of synthesizer emulation.

The buffer area will be filled by the manager with the entries of Synthesizer Emulation supported by the SoundFont driver.    These entries are in string format, each delimited by a '\0'.    The last entry of the list will have two consecutive '\0's.    Using the above example, we have:

"General MIDI\0GS\0MT 32\0User Custom Synth\0\0"

The size returned includes the '\0's used to delimit two entries.

Remarks

Parameters passed in as pointers must be valid memory allocated by the parent application.

If the m_Buffer is not sufficient to complete the operation, the SoundFont Manager will not update string entries into the buffer and will result in an error.    However, the m_SizeUsed field will still be updated which indicate the amount of memory required within m_Buffer for the operation to succeed.

Return

The return value would be SFERR_NOERR if the operation is successful, and an error code otherwise.    Possible errors are:

SFERR_DEVICE_INVALID

SFERR_RESOURCE_INSUFFICIENT

SFERR_SYSMEM_INSUFFICIENT

See Also	SF_SelectSynthEmulation,

SF_GetSynthEmulation

#### SF_GetSynthEmulation

Syntax

LRESULT

SF_GetSynthEmulation (SFDEVINDEX sfIdx, PSFBUFFEROBJECT pBufObj )

Actions

An application calls this function to query for the current Synthesizer Bank and its descriptor.

Parameters

sfIdx

This is the index to the SoundFont compatible device. The index value ranges from 0 to one less than the return of SF_GetNumDevs.

pBufObj

Specify a pointer to CSFBufferObject, LPSFBUFFEROBJECT. When the application calls the manager, the following fields are used:

Member	Remarks

m_Size	Indicates the size of buffer available.

m_Buffer	Pointer to a buffer area.

Upon returning, the manager will update the following fields:

Member	Remarks

m_SizeUsed	Number of characters from buffer used.

m_Flag	The synthesizer emulation index.

m_Buffer	String of current Synthesizer Emulation descriptor.

The manager fills this buffer with the descriptor of the current Synthesizer Emulation.    The m_Flag will contain the Index to the currently set Synthesizer Emulation.

Remarks

Parameters passed in as pointers must be valid memory allocated by the parent application.

If the m_Buffer is not sufficient to complete the operation, the SoundFont Manager will not update string entries into the buffer and will result in an error.    However, the m_SizeUsed field will still be updated which indicate the amount of memory required within m_Buffer for the operation to succeed.

Return

The return value would be SFERR_NOERR if the operation is successful, and an error code otherwise.    Possible errors are:

SFERR_DEVICE_INVALID

SFERR_RESOURCE_INSUFFICIENT

See Also	SF_GetAllSynthEmulations,

SF_SelectSynthEmulation

#### SF_SelectSynthEmulation

Syntax		LRESULT SF_SelectSynBank ( SFDEVINDEX sfIdx,

WORD wSynIdx)

Actions

An application calls this function to select the Synthesizer Emulation in which the application wishes a SoundFont Compatible Device to run.

Parameters

sfIdx

This is the index to the SoundFont compatible device. The index value ranges from 0 to one less than the return of SF_GetNumDevs.

wSynIdx

This word contains the desired Synthesizer Emulation Index.

Remarks

None.

Return

The return value would be SFERR_NOERR if the operation is successful, and an error code otherwise.    Possible errors are:

SFERR_DEVICE_INVALID

SFERR_DEVICE_BUSY

SFERR_SYNINDEX_INVALID

SFERR_PATHNAME_INVALID

SFERR_SYSMEM_INSUFFICIENT

SFERR_SOUNDMEM_INSUFFICIENT

See Also	SF_GetAllSynthEmulations,

SF_GetSynthEmulation

### Bank Loading Management Functions

#### SF_LoadBank

Syntax

LRESULT SF_LoadBank (SFDEVINDEX sfIdx, PSFMIDILOCATION psfLoc, PSFBUFFEROBJECT pBufObj )

Actions

An application calls this function to inform the SoundFont    Manager that it wishes to load a SoundFont Bank into MIDI Space.    The source can be either in a file or in system memory.

Parameters

sfIdx

This is the index to the SoundFont compatible device. The index value ranges from 0 to one less than the return of SF_GetNumDevs.

psfLoc

This indicates the bank index to which a SoundFont bank is to be loaded. Section .1.1.4: Manager MIDI Space Index Ranges for details on valid ranges.    The m_PresetIndex portion is ignored with this command.

pBufObj

Specify a pointer to CSFBufferObject. When the application calls the manager, the following fields are used:

Member	Remarks

m_Size	Indicates the size of buffer area.

m_Flag	Indicates if buffer contains a path name.

m_Buffer	Pointer to a buffer area.

The value of m_Flag should be either SFFLAG_OPER_MEMORY or SFFLAG_OPER_FILE. SFFLAG_OPER_MEMORY indicates that the bank is contained in system memory. In this case, m_Buffer contains a pointer to the memory location in which the SoundFont bank resides. SFFLAG_OPER_FILE indicates that the bank is contained in a file. In this case, m_Buffer contains a full path to the SoundFont Bank file.

Remarks

Parameters passed in as pointers must be valid memory allocated by the parent application.

Return

The return value will be SFERR_NOERR if the operation is successful, and an error code otherwise.    Possible errors are:

SFERR_DEVICE_INVALID

SFERR_DEVICE_BUSY

SFERR_PATHNAME_INVALID

SFERR_FORMAT_INVALID

SFERR_BANK_INDEX_INVALID

SFERR_RESOURCE_INSUFFICIENT

SFERR_SYSMEM_INSUFFICIENT

SFERR_SOUNDMEM_INSUFFICIENT

See Also	SF_GetLoadedBankDescriptor,

SF_ClearLoadedBank

#### SF_GetLoadedBankDescriptor

Syntax

LRESULT SF_GetLoadedBankDescriptor (	SFDEVINDEX sfIdx,

PSFMIDILOCATION sfLoc,

PSFBUFFEROBJECT pBufObj)

Actions

An application calls this function to the SoundFont Manager to request information describing sounds that may be loaded as banks in MIDI Space.

Parameters

sfIdx

This is the index to the SoundFont compatible device. The index value ranges from 0 to one less than the return of SF_GetNumDevs.

sfLoc

This indicates the MIDI location that is being queried. Section .1.1.4: Manager MIDI Space Index Ranges for details on valid ranges.    The m_PresetIndex portion is ignored with this command.

pBufObj

A pointer to a CSFBufferObject, PSFBUFFEROBJECT. When the application calls the manager, the following fields are used:

Member	Remarks

m_Size	Indicates the size of buffer available.

m_Buffer	Pointer to a buffer area.

Upon returning, the manager will update the following fields:

Member	Remarks

m_SizeUsed	Number of characters from buffer used.

m_Buffer	String of current bank descriptor.

The manager fills the buffer with the descriptor of the bank index in question.    The string ends with two consecutive '\0's.    Size returned includes the '\0's.

Remarks

Parameters passed in as pointers must be valid memory allocated by the parent application.

If the m_Buffer is not sufficient to complete the operation, the SoundFont Manager will not update string entries into the buffer and will result in an error.    However, the m_SizeUsed field will still be updated which indicate the amount of memory required within m_Buffer for the operation to succeed.

Return

The return value would be SFERR_NOERR if the operation is successful, and an error code otherwise.    Possible errors are:

SFERR_DEVICE_INVALID

SFERR_BANK_INDEX_INVALID

SFERR_RESOURCE_INSUFFICIENT

See Also	SF_LoadBank,

SF_ClearLoadedBank

#### SF_GetLoadedBankPathname

Syntax

LRESULT SF_GetLoadedBankPathname (	SFDEVINDEX sfIdx,

PSFMIDILOCATION sfLoc,

PSFBUFFEROBJECT pBufObj)

Actions

An application calls this function to the SoundFont Manager to request the full path to the file whose sounds were loaded into MIDI space.

Parameters

sfIdx

This is the index to the SoundFont compatible device. The index value ranges from 0 to one less than the return of SF_GetNumDevs.

sfLoc

Indicates the bank index that is to be queried. Section .1.1.4: Manager MIDI Space Index Ranges has details on valid ranges.

pBufObj

A pointer to a CSFBufferObject, PSFBUFFEROBJECT. When the application calls the manager, the following fields are used:

Member	Remarks

m_Size	Indicates the size of buffer available.

m_Buffer	Pointer to a buffer area.

Upon returning, the manager will update the following fields:

Member	Remarks

m_SizeUsed	Number of characters from buffer used.

m_Buffer	String of current bank descriptor.

The manager fills the buffer with the descriptor of the bank index in question.    The string ends with two consecutive '\0's.    Size returned includes the '\0's.

Remarks

Parameters passed in as pointers must be valid memory allocated by the parent application.

If the m_Buffer is not sufficient to complete the operation, the SoundFont Manager will not update string entries into the buffer and will result in an error.    However, the m_SizeUsed field will still be updated which indicate the amount of memory required within m_Buffer for the operation to succeed.

Note that some banks may contain presets that are “children” of other banks. Application may ascertain this condition by to comparing the value of pwBankIndex before and after a call to SF_IsMidiBankUsed. See Section .1.11.1:  for details.

Return

The return value would be SFERR_NOERR if the operation is successful, and an error code otherwise.    Possible errors are:

SFERR_DEVICE_INVALID

SFERR_BANK_INDEX_INVALID

SFERR_RESOURCE_INSUFFICIENT

See Also	SF_LoadBank,

SF_ClearLoadedBank

SF_IsMIDIBankIndexUsed

#### SF_ClearLoadedBank

Syntax

LRESULT SF_ClearLoadedBank ( SFDEVINDEX sfIdx,

PSFMIDILOCATION psfLoc)

Actions

An application calls this function to clear a bank of sounds which have been loaded into a given bank location in MIDI Space.

Parameters

sfIdx

This is the index to the SoundFont compatible device. The index value ranges from 0 to one less than the return of SF_GetNumDevs.

psfLoc

This indicates the location in MIDI Space containing a loaded SoundFont Preset that is to be cleared. Section .1.1.4: Manager MIDI Space Index Ranges for details on valid ranges. The m_PresetIndex portion is ignored with this command.

Remarks

Parameters passed in as pointers must be valid memory allocated by the parent application.

Return

The return value would be SFERR_NOERR if the operation is successful, and an error code otherwise.    Possible errors are:

SFERR_DEVICE_INVALID

SFERR_DEVICE_BUSY

SFERR_PRESET_INDEX_INVALID

SFERR_BANK_INDEX_INVALID

See Also	SF_LoadBank

SF_GetLoadedBankDescriptor

### Preset Loading and Management Functions

#### SF_LoadPreset

Syntax

LRESULT SF_LoadPreset ( SFDEVINDEX sfIdx,

PSFMIDILOCATION psfSource,

PSFMIDILOCATION psfTarget,

PSFBUFFEROBJECT pBufObj )

Actions

An application calls this function to load a single Preset located in a SoundFont bank into a Bank/Program location in MIDI Space.

Parameters

sfIdx

This is the index to the SoundFont compatible device. The index value ranges from 0 to one less than the return of SF_GetNumDevs.

psfSource

Indicates the MIDI location within the source bank that indicates the preset is to be loaded. Section .1.1.4: Manager MIDI Space Index Ranges for details on valid ranges.

psfTarget

Indicates the target MIDI location to which the preset is to be loaded. Section .1.1.4: Manager MIDI Space Index Ranges for details on valid ranges.

pBufObj

Specifies a pointer to a CSFBufferObject data type, PSFBUFFEROBJECT.    When the application calls the manager, the following fields are used:

Member	Remarks

m_Size	Indicates the size of buffer area.

m_SizeUsed	Not used, should be set to 0 and ignored upon return.

m_Flag	Indicates if buffer contains a path name.

m_Buffer	Pointer to a buffer area.

The value of m_Flag should be either SFFLAG_OPER_MEMORY or SFFLAG_OPER_FILE. SFFLAG_OPER_MEMORY indicates that the bank is contained in system memory. In this case, m_Buffer contains a pointer to the memory location in which the SoundFont bank resides. SFFLAG_OPER_FILE indicates that the bank is contained in a file. In this case, m_Buffer contains a full path to the SoundFont Bank file.

Remarks

Parameters passed in as pointers must be valid memory allocated by the parent application.

This function will overwrite any presets previously attached to the m_PresetIndex.    No error or warning message is given.

Return

The return value would be SFERR_NOERR if the operation is successful, and an error code otherwise.    Possible errors are:

SFERR_DEVICE_INVALID

SFERR_DEVICE_BUSY

SFERR_PATHNAME_INVALID

SFERR_FORMAT_INVALID

SFERR_PRESET_INDEX_INVALID

SFERR_BANK_INDEX_INVALID

SFERR_RESOURCE_INSUFFICIENT

SFERR_SYSMEM_INSUFFICIENT

SFERR_SOUNDMEM_INSUFFICIENT

See Also	SF_GetLoadedPresetDescriptor

SF_ClearLoadedPreset

#### SF_GetLoadedPresetDescriptor

Syntax

LRESULT SF_GetLoadedPresetDescriptor (	SFDEVINDEX sfIdx,

PSFMIDILOCATION psfLoc,

PSFBUFFEROBJECT pBufObj)

Actions

An application calls this function to the SoundFont Manager to request information describing sounds that may be loaded into MIDI space.

Parameters

sfIdx

This is the index to the SoundFont compatible device. The index value ranges from 0 to one less than the return of SF_GetNumDevs.

psfLoc

This indicates the MIDI location that is being queried. Section .1.1.4: Manager MIDI Space Index Ranges has details on what this structure should look like when querying names of presets located in either instrumental or percussive MIDI space.

pBufObj

A pointer to a CSFBufferObject, PSFBUFFEROBJECT. When the application calls the manager, the following fields are used:

Member	Remarks

m_Size	Indicates the size of buffer available.

m_Buffer	Pointer to a buffer area.

Upon returning, the manager will update the following fields:

Member	Remarks

m_SizeUsed	Number of characters from buffer used.

m_Buffer	String of current bank descriptor.

Th Manager fills the buffer with the descriptor of the preset in question.    The string ends with two consecutive '\0's.    Size returned includes the '\0's.

Remarks

Parameters passed in as pointers must be valid memory allocated by the parent application.

If the m_Buffer is not sufficient to complete the operation, the SoundFont Manager will not update string entries into the buffer and will result in an error.    However, the m_SizeUsed field will still be updated which indicate the amount of memory required within m_Buffer for the operation to succeed.

Return

The return value would be SFERR_NOERR if the operation is successful, and an error code otherwise.    Possible errors are:

SFERR_DEVICE_INVALID

SFERR_BANK_INDEX_INVALID

SFERR_RESOURCE_INSUFFICIENT

See Also	SF_LoadBank,

SF_LoadPreset.

SF_ClearLoadedPreset

#### SF_ClearLoadedPreset

Syntax

LRESULT SF_ClearLoadedPreset ( SFDEVINDEX sfIdx,

SFMIDILOCATION psfLoc)

Actions

An application calls this function to clear the preset which has been loaded into a given bank location in MIDI Space.

Parameters

sfIdx

This is the index to the SoundFont compatible device. The index value ranges from 0 to one less than the return of SF_GetNumDevs.

psfLoc

This indicates the location in MIDI Space containing a loaded SoundFont Preset that is to be cleared. Section .1.1.4: Manager MIDI Space Index Ranges has details on what this structure should look like when clearing presets located in either instrumental or percussive MIDI space.

Remarks

Parameters passed in as pointers must be valid memory allocated by the parent application.

Return

The return value would be SFERR_NOERR if the operation is successful, and an error code otherwise.    Possible errors are:

SFERR_DEVICE_INVALID

SFERR_DEVICE_BUSY

SFERR_PRESET_INDEX_INVALID

SFERR_BANK_INDEX_INVALID

See Also	SF_LoadPreset

SF_GetLoadedPresetDescriptor

### Waveform Loading and Management Functions

These functions are valid for file formats that contain single sampled waveforms. All SoundFont Compatible Devices support the WAV format, some may support other formats.

#### SF_LoadWaveform

Syntax

LRESULT SF_LoadWaveform ( SFDEVINDEX sfIdx,

PSFMIDILOCATION psfTarget,

PSFBUFFEROBJECT pBufObj )

Actions

An application calls this function to load a waveform into a Bank/Program location in MIDI Space.

Parameters

sfIdx

This is the index to the SoundFont compatible device. The index value ranges from 0 to one less than the return of SF_GetNumDevs.

psfTarget

Indicates the target MIDI location to which the preset is to be loaded. Section .1.1.4: Manager MIDI Space Index Ranges has details on what this structure should look like when querying names of waveforms located in either instrumental or percussive MIDI space.

pBufObj

Specifies a pointer to a CSFBufferObject data type, PSFBUFFEROBJECT.    When the application calls the manager, the following fields are used:

Member	Remarks

m_Size	Indicates the size of buffer area.

m_SizeUsed	Not used, should be set to 0 and ignored upon return.

m_Flag	Indicates if buffer contains a path name.

m_Buffer	Pointer to a buffer area.

The value of m_Flag should be either SFFLAG_OPER_MEMORY or SFFLAG_OPER_FILE. SFFLAG_OPER_MEMORY indicates that the bank is contained in system memory. In this case, m_Buffer contains a pointer to the memory location in which the SoundFont bank resides. SFFLAG_OPER_FILE indicates that the bank is contained in a file. In this case, m_Buffer contains a full path to the SoundFont Bank file.

Remarks

Parameters passed in as pointers must be valid memory allocated by the parent application.

This function will overwrite any presets previously attached to the m_PresetIndex.    No error or warning message is given.

Return

The return value would be SFERR_NOERR if the operation is successful, and an error code otherwise.    Possible errors are:

SFERR_DEVICE_INVALID

SFERR_DEVICE_BUSY

SFERR_PATHNAME_INVALID

SFERR_FORMAT_INVALID

SFERR_PRESET_INDEX_INVALID

SFERR_BANK_INDEX_INVALID

SFERR_RESOURCE_INSUFFICIENT

SFERR_SYSMEM_INSUFFICIENT

SFERR_SOUNDMEM_INSUFFICIENT

See Also	SF_GetLoadedWaveformDescriptor,

SF_ClearLoadedWaveform

#### SF_ClearLoadedWaveform

Syntax

LRESULT SF_ClearLoadedWaveform ( SFDEVINDEX sfIdx,

PSFMIDILOCATION psfLoc)

Actions

An application calls this function to clear the waveform that has been loaded into a given bank and preset location in MIDI Space.

Parameters

sfIdx

This is the index to the SoundFont compatible device. The index value ranges from 0 to one less than the return of SF_GetNumDevs.

psfLoc

This indicates the location in MIDI Space containing a loaded SoundFont Preset that is to be cleared. Section .1.1.4: Manager MIDI Space Index Ranges has details on what this structure should look like when clearing waveforms located in either instrumental or percussive MIDI space.

Remarks

Parameters passed in as pointers must be valid memory allocated by the parent application.

Return

The return value would be SFERR_NOERR if the operation is successful, and an error code otherwise.    Possible errors are:

SFERR_DEVICE_INVALID

SFERR_DEVICE_BUSY

SFERR_PRESET_INDEX_INVALID

SFERR_BANK_INDEX_INVALID

See Also	SF_LoadWaveform,

SF_GetLoadedWaveformDescriptor

### Miscellaneous Functions

#### SF_IsMIDIBankUsed

Syntax

LRESULT SF_IsMIDIBankUsed (SFDEVINDEX sfIdx ,

PWORD pwBankIndex)

Actions

The client sends this message to a SoundFont Manager to identify if a particular bank index in MIDI space contains SoundFont banks, SoundFont presets, or Waveforms.

Parameters

sfIdx

This is the index to the SoundFont compatible device. The index value ranges from 0 to one less than the return of SF_GetNumDevs.

pwBankIndex

Used to hold a pointer to a WORD, which must be filled by the client with the MIDI bank index in question. See Section .1.1.4: Manager MIDI Space Index Ranges for valid range of this index.

Upon returning, the SoundFont Compatible Device Manager will change the contents of this WORD. Three types of values may be returned.

pwBankIndex	Remarks

Unchanged	The Bank in question contains loaded presets or waveforms.

Changed	The Bank in question contains loaded presets which were placed there as a result of a bank load to another bank index.

-1	The Bank in question contains no loaded presets or waveforms.

Remarks

Parameters passed in as pointers must be valid memory allocated by the parent application.

This message is available to allow an application to quickly scan all locations in the MIDI ‘bank’ dimension of MIDI space to determine if they need to fully scan the MIDI ‘preset’ dimension of that ‘bank’ index. This function reduces the number of otherwise necessary queries by as much as a factor of 128.

This function should account for sounds loaded due to the Synthesizer Emulation as well as sounds loaded using the LOAD functions of this API.

Return

The return value would be SFDEV_NO_ERR if the operation is successful, and an error code otherwise

#### SF_QueryStaticSampleMemorySize

Syntax

LRESULT SF_QueryStaticSampleMemorySize (SFDEVINDEX sfIdx ,

PDWORD pdwMaxMem,

PDWORD pdwAvailableMem)

Actions

An application calls this function to the SoundFont Manager to request the number of STATIC sample memory a particular device can hold.

Parameters

sfIdx

This is the index to the SoundFont compatible device. The index value ranges from 0 to one less than the return of SF_GetNumDevs.

pdwMaxMem

This location will be used to store the maximum amount of static sample memory found on the hardware.

pdwAvailableMem

This location will be used to store the current available amount of static sample memory found on the hardware.

Remarks

Parameters passed in as pointers must be valid memory allocated by the parent application.

This function is only valid for devices that use "static" memory to store sample data. “Static” sample memory is memory that is dedicated for a particular synthesizer and cannot be shared with the application memory space. This function SERVES NO PRACTICAL PURPOSE for a device that uses application memory space to store samples, and therefore values returned by such devices should not be considered valid. The capability structure contains information that allows the application to make this distinction.

Values are returned in BYTES.

Return

The return value would be SFERR_NOERR if the operation is successful, and an error code otherwise.    Possible errors are:

SFERR_DEVICE_INVALID

SFERR_NOT_STATIC_MEMORY

See Also	SF_GetDevCaps.

## SoundFont Master MIDI Data Router API Specification

### Theory of Operation

The SoundFont Master MIDI Data Router is a MMSystem compatible MIDI Output Device that is designed to be the single sink of all MIDI data for applications using the SoundFont Management System.

The Master MIDI Data Router is designed to be able to communicate to any and all other SoundFont Compatible Devices AND to any and all simple MMSystem compatible MIDI devices. It does this by accepting MIDI System Exclusive Messages that instruct the Master MIDI Data Router to parse and route certain MIDI data to certain devices based on the characteristics of that MIDI data. The Master MIDI Data Router may parse and re-distribute MIDI data to other devices based on which MIDI channel that data corresponds to, as well as the current bank/program index that data corresponds to.

At reset time, which is defined as all times between the time that the SoundFont Management System is opened and used by an applications, all MIDI data sent to the Master MIDI Data Router is sent to a user selected default MIDI device. At the time at which the SoundFont Management System is first opened until routing commands are sent, all MIDI data continues to be sent to the same user selected default MIDI device. There is no routing command that allows an application to select the default MIDI device, however the routing commands are comprehensive enough to permit the equivalent of this.

The Master Data Router is capable of receiving two streams of MIDI data (the result of two MODM_OPEN commands). This is supported in order to permit the application to use a separate MIDI sequencer (such as the Windows MCI Sequencer) to stream MIDI data, and at the same send dynamic routing commands for non real-time routing messages, and/or dynamic sound basic MIDI commands for interactive sound effects. Doing these results in a MIDI merge, therefore data in those streams MUST be such that either one stream is MIDI channel independent, or that both use mutually exclusive (or different) MIDI channels.

### Routing Command Overview

The following is a comprehensive list of MIDI System Exclusive commands that will be received by the SoundFont Compatible MIDI Routing Device.

#### Routing (System Exclusive Message) Command Format

All Routing Commands, which are sent in the form of MIDI System Exclusive Messages, have the following general format.

0xF0			System Exclusive message

0x00

0x20

0x21			Creative Labs Manufacturer ID

0x5F			SoundFont Management System Product ID

{Master Device ID}	1 byte ID corresponding to a single SoundFont Routing Device. This

accounts for multiple SoundFont Routing Devices that may be daisy-chained together. Default or power-on-reset is 0, 127 is reserved for a broadcast message (all SoundFont Routing devices receive), may be assigned with a front or control panel.

{Function ID}		1 byte ID identifying the routing function.

{Parameters}		Parameters for function specified above.

May include a 4 byte SoundFont Compatible Device or Multimedia System Device ID.

0xF7			End System Exclusive Message

The details for all of the variables are defined in the command descriptions.

#### Routing Command General Description

All MIDI System Exclusive messages and non-channel specific MIDI messages, other than the SoundFont Compatible Device messages, are to be relayed to all devices that are registered.

Note that this suite of commands supports MIDI devices that are not SoundFont compatible, as well as SoundFont compatible devices that are not registered with the Multimedia System.

The System Exclusive command set assumes the application has access to a list of General MIDI devices as well as SoundFont Compatible MIDI Devices.    When a routing command is intended for a particular SoundFont or MMSystem device, the ID is included within the Function ID parameter list. This ID is a 4 byte value which may be derived from either the MMSystem index or the value returned using the API functions SF_GetDevCaps or SF_GetRouterID, and which may be converted to MIDI bytes using the following macro defined in <sfman.h>:

SF_GET_ROUTING_INDEX(devID, routeID1, routeID2, routeID3, routeID4)

If command is intended for a SoundFont Compatible Device, the devID should be the dwDevID value returned with the SF_GetRouterID command. If the command is intended for a simple MMSystem Device, the userID should simply be the MMSystem ID value recast as a double word or 32 bit unsigned value..

The macro assigns the values of routeID1, routeID2, routeID3 and routeID4 to the values to be sent in the System Exclusive Message in the numeric order in which the bytes should be sent.

The routeID1 is the lower 7 bits contained within the LSB of devID. The routeID2 is the lower 7 bits contained within the third MSB of devID. The routeID3 is the lower 7 bits contained within the second MSB of devID. The routeID4 is the lower 7 bits contained within the MSB of devID. When routing to a MMSystem device, the devID is equal to the MMSystem Device ID. When routing to a SoundFont Compatible Device, the devID is equal to the value returned by SF_GetRouterID. In cases where a SoundFont Compatible Device is a MMSystem Device, use of either IDs in the command are equivalent messages.

Because of the dynamic nature of the System Exclusive commands, considering the fact that different systems may have different devices in different orders, it is expected that the System Exclusive messages be sent dynamically by the application and not embedded within static MIDI sequence data.

### Global Routing Commands

#### Reset All Routings to Default

Actions

All messages previously sent are reset. All MIDI messages sent to the default MIDI output device.

Function ID	0x7F

Parameters	None.

Remarks	None.

### SoundFont Compatible Device Routing Commands

This set of commands allows routing of MIDI data to MMSystem devices or SoundFont compatible devices on the system.

#### Route All Events to SoundFont/MIDI Device

Actions

Assigns ALL MIDI Events to go to the specified SoundFont Compatible Device. All MIDI messages in any MIDI channel after a bank select and a program select with these selected values will automatically go to the selected device, until a new routing command is entered.

Function ID

0x00

Parameters

Parameters 1 through 4: Routing Index

Index is a value corresponding to a single SoundFont Compatible Device or MMSystem Device on the system. This is a 4 byte ID that is generated with the SF_GET_ROUTING_INDEX macro.

Remarks

This command may not be used in conjunction with any other routing commands. Before using other routing commands, send a Reset All Routings (Section .2.3.1) command.

#### Route MIDI Channel to SoundFont/MIDI Device

Actions

Assign MIDI Channel to a SoundFont Compatible Device. All MIDI messages with the selected channel will automatically be directed to the selected device.

Function ID

0x02

Parameters

Parameter 1: MIDI Channel

Value of the MIDI channel whose events should be sent to given a SoundFont device.

Parameters 2 through 5: Routing Index

Index is a value corresponding to a single SoundFont Compatible Device or MMSystem Device on the system. This is a 4 byte ID that is generated with the SF_GET_ROUTING_INDEX macro.

Remarks

This command may not be used in conjunction with any other routing commands. Before using other routing commands, send a Reset All Routings (Section .2.3.1) command.

#### Route Bank and Program Data to SoundFont/MIDI Device

Actions

Assign Bank and Preset value to a Bank and Preset location on a SoundFont Compatible Device. All MIDI messages in any MIDI channel after a bank select and a program select with these selected values will automatically go to the selected device.

Function ID

0x04

Parameters

Parameter 1: Reserved

Undefined, should be set to zero.

Parameter 2: Source Bank MSB

Index is the value corresponding to CC0 Message received from the application

Parameter 3: Source Bank LSB

Index is the value corresponding to CC32 Message received from the application

Parameter 4: Source Program Index

Index is the program index received from the application

Parameters 5 through 8: Routing Index

Index is a value corresponding to a single SoundFont Compatible Device or MMSystem Device on the system. This is a 4 byte ID that is generated with the SF_GET_ROUTING_INDEX macro.

Parameter 9: Target Bank MSB

Index is the value corresponding to CC0 Message sent to the selected device upon receiving a bank/preset combination matching the Source Bank MSB/LSB and Source Program Index.

Parameter 10: Target Bank LSB

Index is the value corresponding to CC32 Message sent to the selected device upon receiving a bank/preset combination matching the Source Bank MSB/LSB and Source Program Index

Parameter 11: Target Program Index

Index is the program index sent to the selected device upon receiving a bank/preset combination matching the Source Bank MSB/LSB and Source Program Index

Remarks

This command may not be used in conjunction with any other routing commands. Before using other routing commands, send a Reset All Routings (Section .2.3.1) command.

# Glossary

Please see the glossary in Appendix A1.