Kingswood Software Development Tools                                 AS80

-------------------------------------------------------------------------

NAME

as80 - assembler for 8080, 8085 and Z80 microprocessors.

SYNOPSIS

as80 [-gchlnopqstvwxz] file

DESCRIPTION

This documentation is for as80 [1.07].

Copyright 1990-1994, Frank A. Vorstenbosch, Kingswood Software.

AS80 is an assembler for the Intel 8080/8085 and Zilog Z80

microprocessors.  It reads input from an ASCII text file, assembles

this into memory, and then writes a listing and a binary or hex file.

AS80 is case sensitive, not only does it differentiate between the

labels XYZ and xyz, but it also requires all (pseudo) instruction and

register names to be lower case.  This way, the listing is the most

readable.  Option -i can be used to make the assembler case insensitive.

Alternatively, the included TOLOWER program can be used to convert

sources to lower case.

OPTIONS

As80 recognizes the following options:

-c   Show number of cycles per instruction in listing.  This

decreases the number of columns available for listing by 5.

The number of cycles is printed between brackets [ and ].

-g   Generate source-level debug information file.  This file

can then be used in in-system debugging or a software

simulator.

-h<lines>

Specify height of page for listing.  This option determines

the number of lines per printed page.  Each page has a header

and is terminated by a form-feed character.  The special

case -h0 indicates an infinite page length.  In this case,

page breaks are only inserted between the two passes and

the symbol table (if present).

-i   Ignore case in opcodes.  In this way, the assembler does not

differentiate between 'add' and 'ADD', for example.  Labels

are still case sensitive.

-l   Generate pass 2 listing.

-l<filename>

Listing file name.  The listing file is used for pass 1 and

pass 2 listing, for the symbol table (printed between the

two passes), and some statistics.  When neither -p nor -t

is specified, and -l<filename> is given, then the assembler

automatically generates a pass 2 listing.  When -p or -t is

specified, an additional -l should be given is a pass 2

listing is required.  The filename - can be used to direct

the listing to standard output.

-l   Generate pass 2 listing.

-n   Disable optimizations.  When this option is specified no

optimizations will be done, even when the OPT pseudo-

instruction is used in the source code.

-o<filename>

Specify binary or s-records output file name.  The assembler

automatically adds ".bin" for binary output or ".s19" for

s-records output when no extension is given.

-p   Generate pass 1 listing.  This may be used to examine any

optimizations/bugs generated in pass 2.

-q   Quiet mode.  No running line counter is displayed on standard

error output.

-s   Write s-records instead of binary file.  The s-records file

contains data for (only) the used memory; the binary file

begins at the lowest used address, and continues up to the

highest used address; filling unused memory between these

two addresses with either $ff or $00.

-s2  Write intel-hex file instead of binary file.  The intel-hex

file contains data for (only) the used memory.

-t   Generate symbol table.  The symbol table is listed between

passes one and two, displaying the name, hexadecimal and

decimal value of each label, using 4-digit hexadecimal

numbers where possible, otherwise 8-digit numbers.  The

decimal value is followed by an asterisk if the label is

redefinable (defined using SET instead of EQU).

-v   Verbose mode.  More information is displayed on standard

output.

-w<width>

Specify column width.  Normally, the listing is printed using

79 columns for output to a 80-column screen or printer.  If

the -w option is given without a number following it, then

the listing will be 131 columns wide, otherwise it will be

the number of colulmns specified (between 60 and 200).

-x1  Use 8085 extensions.  The 8085 CPU has two additional

instructions and different cycle counts, but is otherwise

software compatible to the 8080.  When this option is not

specified the assembler rejects the RIM and SIM instructions.

-x   or

-x2

Use Z80 extensions.  The Z80 has many additional instructions

and addressing modes, but is otherwise software compatible to

the 8080.  When this option is not specified the assembler

rejects all Z80 new instructions and addressing modes.

-x3  Use Z80 extensions and index register byte instructions.

The IX and IY registers were originally intended to be

split in IXH/IXL and IYH/IYL register pairs.  For some

reason (bug in original mask set?) these instructions were

not included by Zilog in the programming manuals, but

they do work on all CPUs I've seen.  Your mileage may vary.

Note that these extensions do NOT work on the Z180/181 and

H64180, but they DO (and are documented) on the Z280.

-z   Fill unused memory with zeros.  Normally when a binary file

is generated, unused bytes between the lowest and highest

used addresses are filled with $ff, the unprogrammed state

of EPROMs.  If this is not wanted, the -z option can be used

to initialize this memory to $00.  With s-records, unused

memory is not output to the file, and this option forces the

creation of an S9 (start address) record instead, even if no

start address is specified in the file with the END pseudo-

instruction.

Commandline options can be catenated, but no other option can follow

one that may have a parameter.  Thus:

-tlfile

is correct (specifying symbol table and pass 2 listing), but

-h5t

is not.

It is possible to discard any of the the output files by specifying

the name 'nul'.

EXPRESSIONS

The assembler recognizes most C-language expressions.  The operators

are listed here in order of precedence, highest first:

()            braces to group subexpressions

* $           current location counter

unary + - ! ~ unary + (no-operation), negation, logical NOT,

binary NOT

* / %         multiplication, division, modulo

+ -           addition, subtraction

<< >>         shift left, shift right

< > <= >=     comparison for greater/less than

= !=          comparison for equality (== can be used for =)

&             binary AND

^             binary XOR

|             binary OR

The logical NOT (!) evaluates to zero if the parameter is nonzero,

and vice versa.  The binary NOT (~) complements all the bits in its

parameter.

Note: the asterisk is both used as the multiplication operator, and

as symbol for the current location.  The assembler determines from

the context which is which. Thus:

5**

is a valid expression, evaluating to five times the current location

counter, and:

2+*/2

is too, evaluating to the current location counter divided by two, to

which two is added.  In the same way, the % sign is both used as the

modulo operator and the prefix for binary numbers.

Numbers can be specified in any number base between 2 and 36.

Decimal numbers can be used without a prefix, hexadecimal numbers

can be prefixed by $, octal numbers by @, and binary numbers by %.

Other number bases can be used by using the following format:

<base>#<number>,

where the base is the number base to use (must be specified in

decimal), and number is the value.  Thus:

1000    - decimal number, value 10*10*10=1000

%1000   - binary number, value 2*2*2=8

@1000   - octal number, value 8*8*8=512

$1000   - hexadecimal number, value 16*16*16=4096

#1000   - hexadecimal number, value 16*16*16=4096

2#1000  - binary number, value 2*2*2=8

4#1000  - base-4 number, value 4*4*4=64

7#1000  - base-7 number, value 7*7*7=343

36#1000 - base-36 number, value 36*36*36=444528

For number bases greater than 10, additional digits are represented

by letters, starting from A.  Both lower and upper case letters can

be used.

11#aa = 120

16#ff = 255

25#oo = 624

36#zz = 1295

PSEUDO-INSTRUCTIONS

align <expression>

Align fills zero or more bytes with zeros until the new address

modulo <expression> equals zero.  If the expression is not present,

align fills zero or one bytes with zeros until the new address

is even.

Example 1:

align  256             ; continue assembly on the

; next 256-byte page

Example 2:

align                  ; make sure table begins

Table           dw     1,2,3           ; on an even address

bss

Put all following data in the bss segment.  Only data pseudo-instructions

can be used in the bss segment, and these only increment the location

counter.  It is up to the programmer to initialize the bss segment.  The

bss segment is especially meaningful in a ROM based system where

variables must be placed in RAM, and RAM is not automatically initialized.

The assembler internally maintains three separate location counters,

one for the code segment, one for the data segment and one for the

uninitialized data segment.  The user is responsible for not overlapping

the segments by setting appropriate origins.  The code, data and bss

pseudo-instructions can be used to interleave code and data in the source

listing, while separating the three in the generated program.  The

assembler starts with the code segment if none is specified.

Example:

code

org    $f000           ; Assuming 4 kbyte code ROM

data                   ; with 2 kbyte program and

org    $f800           ; 2 kbyte initialized data

bss

org    0               ; bss segment is in RAM

Buffer         ds     100

code

Begin          ld     hl,Table

ld     de,Buffer

ld     bc,3

ldir

.

.

.

data

Table          db     1,2,3

code

MyFunc         ld     ix,Table

.

.

code

Put all following assembled instructions and data in the code segment.

See BSS.

data

Put all following assembled instructions and data in the data segment.

See BSS.

db <bytelist>

Define bytes.  The bytes may be specified as expressions or strings,

and are placed in the current (code or data) segment.  This pseudo

instruction is similar to the Zilog-defined defb and defm pseudo-

instructions.

Example:

Message        db     7,"Error",13,10,0

defb <bytelist>

Define bytes.  The bytes may be specified only as expressions,

and are placed in the current (code or data) segment.  This

pseudo-instruction is similar to the db pseudo-instruction.

Example:

Message        defb   7

defm   "Error"

defb   13,10,0

defm <string>

Define message.  The bytes may be specified only as a string, and

are placed in the current (code or data) segment.  This pseudo-

instruction is similar to the db pseudo-instruction.

ds <expression>

defs <expression>

Define zero or more bytes empty space.  The specified number of

bytes are filled with zeros.  This pseudo-instruction is identical

to the Zilog-defined pseudo-instruction defs.

Example:

ds     100             ; reserve 100 bytes here

dw <wordlist>

defw <wordlist>

Define words.  The words are placed in the current (code or data)

segment.  This pseudo-instruction is identical to the Zilog-

defined defw pseudo-instruction.

Example:

ld     a,Function      ; number of function

ld     hl,JumpTable

add    a,a

add    a,l

ld     l,a

adc    a,h

sub    l

ld     h,a

jp     (hl)            ; jump to function

JumpTable      dw     Function0

dw     Function1

dw     Function2

else

The else pseudo-instruction can be used for if-then-else

constructions. It must be placed between an if and an endif

instruction.  For an example, see the if pseudo-instruction.

end <expression>

The end pseudo-instruction is optional, and need not be used.  If

it is used, its optional operand specifies the staring address of

the program.  This address is displayed when the program is

assembled, and is also placed in the s-record output file.

Example:

end    Start

endif

The endif pseudo-instruction must be used to close an if-endif

or if-else-endif construction.  For an example, see the if

pseudo-instruction.

<label> equ <expression>

The equ (equate) pseudo-instruction sets the specified label to

the value of the expression.  The label may not already exist.

Some programmers choose to use only upper-case identifiers for

labels defined in this way to differentiate them from addresses.

Example:

ESCAPE          equ    27

if <expression>

The if pseudo-instruction can be used in conjunction with the

endif and possibly the else pseudo-instructions to selectively

enable and disable pieces of code in the source.  If the expression

given evaluates to zero, then the following code up to the matching

else or endif is not included in the assembly.  If the expression

is nonzero, then the following code is included, but any code

between the matching else and endif is not.

The original Zilog assemblers called this pseudo-instruction COND.

Example:

if COUNT=2 | COUNT=4

add     a,a            ; shift left for counts

if COUNT=4             ; of 2 and 4

add     a,a

endif

else

ld      b,COUNT        ; otherwise use slow multiply

call    Multiply

endif

include <string>

The named file is included in the assembly at this point.  After

it has been read, assembly continues with the next line of the

current file. Include files may be nested.

Example:

include "defines.i"

list

Enable generation of the listing in the list-file.  If the listing

has been disabled twice, it must be enabled twice before it is

generated.  When no -p or -l option has been specified on the

command line, this pseudo-instruction has no effect.

nolist

Disable generation of the listing in the list-file.

noopt

Disable optimizations.  If the -n option has been specified on the

command line, this pseudo-instruction has no effect.

nop <expression>

No operation.  When the optional expression is not present, this

is simply the nop instruction of the processor.  When the

expression is present, the specified number of nop instructions

are inserted.

Example:

nop    10

opt

Enable optimizations.  If the -n option has been specified on the

command line, this pseudo-instruction has no effect.

When optimization is enabled, the assembler tries to use the

shortest and fastest instructions possible which have the effect

the user wants.  It may replace any extended-address instruction

by direct-address instructions (provided the direct pseudo-

instruction has been used).  It replaces long branches with jumps

or short branches, calls with branches to subroutines, and

replaces zero-offset indexed instructions by no-offset indexed

instructions.  The effects of optimizations is clearly visible if

both a pass one and a pass two listing is generated.

org <expression>

The org (origin) pseudo-instruction specifies the next address to

be used for assembly.  When no origin has been specified, the

assembler uses the value 0.  The assembler maintains three separate

location counters: one for the code segment, one for the data

segment, and one for the bss segment.  See the code and pseudo-

instruction for more information.

page <expression>

When the optional expression is not present, the assembly listing

is continued on the next page.  When the expression is present,

the listing is continued on the next page only if the specified

number of lines do not fit on the current page.

<label> set <expression>

<label> = <expression>

The set pseudo-instruction sets the specified label to the value

of the expression.  The label may or may not already exist.

Some programmers choose to use only upper-case identifiers for

labels defined in this way to differentiate them from addresses.

Example:

CURRENT         set    0

.

.

.

CURRENT         set    CURRENT+1

struct <name>

struct <name>,<expression>

The struct (structure) pseudo-instruction can be used to define

data structures in memory more easily.

The name of the structure is set to the total size of the structure;

if the expression is present, the starting offset is the value of

the expression in stead of zero.

Between the struct and end struct pseudo-instructions the following

pseudo-instructions can be used: db, dw, ds, label, align.

Within structures these pseudo-instructions take a slightly different

format than normally:

db <name>                element is one byte

dw <name>                element is two bytes

ds <name>,<expression>   element is the specified number of bytes

ds <expression>          skip the specified number of bytes

label <name>             element is zero bytes, i.e. set the name

to the current structure offset

align <expression>       skip until (offset%expression)=0

align                    skip until offset is even

Example:

struct ListNode,4

dw     LN_Next

dw     LN_Previous

db     LN_Type

align

label  LN_Simple       ; size of structure so far

align  8

ds     LN_Data,10

end struct

This is identical to:

LN_Next        equ     4       ;\

LN_Previous    equ     6       ; offset of structure elements

LN_Type        equ     8       ;/

LN_Simple      equ     10      ; size of structure so far

LN_Data        equ     16      ; offset of structure element

ListNode       equ     26      ; size of structure

title <string>

The title pseudo-instruction sets the title to be used in the

header of each listing page.  The string should be no longer than

80 characters.

Example:

title  "DIS80 : A disassembler for a 8080 CPU"

ADDRESSING MODES

The assembler allows all 8080 (and when enabled also Z80) addressing

modes.  The use of an expression between braces as an address disallows

braces at the outermost level for immediate values.  The assembler

is capable to determine that

ld     a,(10)+1

and

ld     a,1+(10)

are immediate operands.  You can also use rectangular brackets

[ and ] to include an address.

List of available modes:

immediate

(address)

(bc) (de) (hl) (sp)

Additional addressing modes for the Z80:

(ix+offset)  (ix-offset)

(iy+offset)  (iy-offset)

(c)

LIST OF ACCEPTED INSTRUCTIONS

adc add align and bit bss call ccf code cp cpd cpdr cpi cpir cpl daa

data db dd dec defb defm defs defw di disable djnz ds dw ei else enable

end endif equ ex exx fcb fcc fcw fdb halt if im in inc include ind indr

ini inir jp jr ld ldd lddr ldi ldir list neg nolist noopt nop opt or

org otdr otir out outd outi page pop push res ret reti retn rim rl rla

rlc rlca rld rmb rr rra rrc rrca rrd rst sbc scf set shl shr sim sl sla

sr sra srl stc struct sub title tsti xor

Of these instructions, the following are (more or less) synonymous,

and can be used interchangably.

YOU CAN USE      WHERE YOU WOULD PREVIOUSLY USE

nop 6      -  nop nop nop ....

push bc,de -  push bc ; push de

pop bc,de  -  pop de ; pop bc  (note reversed order)

disable    -  di

enable     -  ei

sr         -  srl

shr        -  srl

sl         -  sla

shl        -  sla

stc        -  scf

ld bc,de   -  ld b,d ; ld c,e

ld ix,bc   -  ld xh,b ; ld xl,c

add R      -  add a,R

or a,R     -  or R

in (c)     -  tsti (c)

ex hl,de   -  ex de,hl

djnz LBL   -  dec b ; jp nz,LBL

jp nv,LBL  -  jp pe,LBL  (no overflow/parity even)

jp v,LBL   -  jp po,LBL  (overflow/parity odd)

jp ns,LBL  -  jp p,LBL   (no sign/positive)

jp s,LBL   -  jp m,LBL   (sign/negative)

Operands:

xh         -  ixh

xl         -  ixl

yh         -  iyh

yl         -  iyl

[Address]  -  (Address)

[hl]       -  (hl)

(ix-6)     -  (ix+-6)

And pseudo-instructions:

db         -  defb, defm

dw         -  defw

ds         -  defs

=          -  set

struct     -  lots of EQUs

LIST OF OTHER KEYWORDS

! != $ % & ( ) * + , - / < << <= = > >= >> [ ] ^ | ~

a af af' align b bc c d db de ds dw e end h hl i ix ixh ixl iy iyh

iyl l label m nc ns nv nz p pe po r s sp struct v xh xl yh yl z

FILES

<file>.a80 - source file.

<file>.z80 - source file -- first alternative.

<file>.asm - source file -- second alternative.

<file>.lst - List file.

<file>.s19 - Motorola S-records output file.

<file>.hex - Intel hex output file.

<file>.bin - Binary output file.

BUGS

No provision for linking other pre-assembled modules is made.

RETURNS

As80 returns one of the following values:

0 - Source file assembled without errors.

1 - Incorrect parameter specified on the commandline.

2 - Unable to open input or output file.

3 - Assembly gave errors.

4 - No memory could be allocated.

DIAGNOSTICS

Help message if only parameter is a question mark, or if an

illegal option has been specified.

AUTHOR

This is copyrighted software, but may be distributed freely as long

as this document accompanies the assembler, and no copyright messages

are removed.  You are explicitly NOT allowed to sell this software

for anything more than a reasonable copying fee, say US$5.

To contact the author:

Frank A. Vorstenbosch

Kingswood Software

P.O. Box 85800              Phone: +31-(70)-355 5241

2508CM  The Hague           BBS:   +31-(70)-355 8674

Netherlands                 Email: prompt@xs4all.nl

-------------------------------------------------------------------------