Source      :  The Journal Of The CP/M User's Group

Issue       :  Vol.3  No.1

Date        :  January   1987

Section     :  Letters to the Editor

Page        :  65-66

CP/M Plus Bug and Patches to support 80 track drives.

----------------------------------------------------

This information was written by Mr. Mark S.Lee  B.Sc(Hons), AMIEE

(Manchester University) to the Journal Editor Mr.Andrew.

In the letters he said that there is a bug in the .EMS file supplied

with the CPC6128 version of CP/M+. This bug appears when the XDPB is

patched to support double sided drives. The BOOK outlines the field

of Amstrad's XDPB.

The patch provided will fix this bug so that double sided drives work

correctly. Having done this you need to write a formatting program.

This is easy with the information about the Amstrad extended jump

block outline in The BOOK.

In addition, another patch is provided so that double stepping is

supported. This allows 40 track disks to be read in 80 track drives.

This is achieved by setting the top most bit in the Amstrad sidedness

entry in the XDPB. The PCW8512 can do this without any .EMS patching.

(This fact is not in The BOOK).

The code is ORG'd at the address it will be at run time. The preceding

comments say where to patch using SID when the .EMS is loaded at 0100H.

The DPB suggested uses 9,512 byte sectors per track. The patches for

10 sectors per track (which is possible) are left as an exercise for

the interested student.

PATCH LISTING

-------------

;PATCHES TO SUPPORT TWIN 706K (DS/DD) DRIVES FOR CP/M PLUS

;THE DPB GIVEN GIVES 2K BLOCK SIZE WITH 128 DIRECTORY ENTRIES.

;CP/M PLUS ALREADY PROVIDES A LARGE ENOUGH ALV AREA TO ACCOMODATE 128

;DIRECTORY ENTRIES.

;IN ADDITION CODE IS PROVIDED TO SUPPORT DOUBLE STEPPING TO READ 40

;TRACK DISKS IN 80 TRACK DRIVES.

;THIS IS ACHEIVED BY SETTING THE TOP MOST BIT IN THE AMSTRAD SIDEDNESS

;ENTRY IN THE XDPB.

;PATCH CODE AT 0EF9 IN .EMS FILE UNDER SID TO THE FOLLOWING:

;

0D39	        ORG     0D39H

0D39 C3C403     JP      PATCH	        ;C3 C4 03 (OLD CODE WAS CD 56 0E)

;CODE BELOW GOES AT 0584H IN .EMS FILE UNDER SID;

03C4	        ORG     03C4H

03C4 CB91       PATCH:  RES     2,C     ;RESET THE HEAD BIT.

03C6 CD560E    	        CALL    0E56H   ;SEND IT TO FDC

03C9 7A		        LD      A,D

03CA DDCB117E           BIT     7,(IX+011H)  ;GET SIDEDNESS TOP BIT

;(FOR DOUBLE STEPPING)

03CE 2804               JR      Z,PTCH1

03D0 CB27               SLA     A

03D2 CB20               SLA     B

03D4 C33D0D     PTCH1:  JP      0D3DH

03D7 DD7E11     PATCH2: LD      A,(IX+11H)   ;CALLED FROM ** BELOW

03DA E67F               AND     07FH        ;THIS CODE REMOVES OUR TOP BIT

;IN SIDEDNESS FOR .EMS CODE

;SO AS NOT TO CONFUSE IT.

03DC C9                 RET

;**CODE BELOW GOES TO 0D3FH IN .EMS FILE UNDER SID

0B7F                    ORG     0B7FH

0B7F CDD703             CALL    PATCH2      ;OLD CODE WAS DD 7E 11

;

;

;THE NEW DPB'S ARE PATCHED AS FOLLOWS:-

;       CODE BELOW GOES TO 0DCBH AND 0DE5H IN .EMS FILE

;       UNDER SID

0B82 2400               DEFW    0024H       ;SPT

0B84 04                 DEFB    4           ;BSH

0B85 0F                 DEFB    0FH         ;BLM

0B86 00                 DEFB    0           ;EXM

0B87 6201               DEFW    0162H       ;DSM (0165H FOR SECOND DPB)

0B89 7F00               DEFW    07FH        ;DRM

0B8B C0                 DEFB    0C0H        ;AL0

0B8C 00                 DEFB    0           ;AL1

0B8D 2000               DEFW    020H        ;CKS

0B8F 0200               DEFW    02          ;OFF (0 FOR SECOND DPB)

;THEN PATCH AMSTRAD'S SPECIAL BITS IN THE XDPB :-

;GOES TO 0DDCH AND 0DF6H IN .EMS FILE UNDER SID

0B91 01                 DEFB    1           ;TWO SIDED DRIVE INTERLEAVED

;TRACKS

0B92 50                 DEFB    50H         ;80 TRACKS PER SIDE

========================================================================

I will not held any responsibility on any of the information above as it

is copied directly from the source. The readers are not oblidge to follows

all the information above. Thanks.

Source      :  AMSTRAD ACTION

Issue       :  NO.18

Date        :  MARCH 1987

Section     :  HOT TIPS (HACKERS ONLY)

Page        :  29

Author      :  Hackers Only, Amstrad Action, Somerton, TA11 7 PY

Tackling the XPB

----------------

At memory locations &A890 through to &A8A8 lies the extended disk

parameter block (XPB) for drive A. Drive B's starts at &A8D0 and

finishes at &A8E8. Look here for information regarding disk format

type. Altering this will let you read 'foreign' disks and create

new formats. Below is the table of the bytes found when using a disk

in Data format (the example assumes drive A is in use), with a

brief explanation on what they do or represent.

&A890   &24     records per track (each record is 128 bytes long).

&00     records per track; high byte

&03     log2BLS-1

&07     BLS/128-1

&00     if DSM < 256 then BLS/1024-1 else BLS/2048-1

&A895   &B3     DSM, size in blocks (represents amount of K on disk)

&00     DSM, high byte

&3F     the number of directory entries minus one

&00     number of directory entries; high byte

&C0     no. of dir blocks; bit significant (&80=1,&C0=2...)

&A89A   &00     number of directory blocks; high byte

&10     bit-signif checksum vector: typically dir. entries/4+1

&00     checksum vector; high byte

&00     number of reserved tracks; points to directory track

&00     number of reserved tracks; high byte

&A89F   &C1     first sector number

&09     number of sectors per track

&2A     read/write gap length

&52     format gap length

&E5     filler byte when formatting

&A8A4   &02     log2(sector size)-7

&04     records per track, which equals sector size/128

&00     current track

&00     not aligned

&00     format automatically selected unless set to &FF

BLS, which indicates the number of bytes in a block, can be 1024,

2048, 4096, 8192 or 16384.

Using the above information on the XPB I shall demostrate how it is

possible to create a new format. I said the XPB lies at &A890 for

drive A - this is true under normal conditions. However, it can lie

in virtually any position. The correct way to test for its position

is by peeking the word at &BE42.

Suppose you want a format containing 10 sectors, with 128 directory

entries (the directory block remaining at track 0) and sector numbers

starting at &71. These are simple changes; other more complicated

alterations could be made but it's best to walk before running. Here is

an outline of how this task could be achieved:

The XPB must be patched: if you want 10 sectors per track, the easiest

way is to alter the number of 128-byte records (location &A890). This

is normally 36, and 36 divided by the number of records per track (which

is  4 - location &A8A5) gives 9 - the number of sectors per track (loca

tion &A8A0). Changing the number of 128-byte records from 36 to 40 will

give us 10 sectors per track.

So far we have patched locations &A890 to read 40 and &A8A0 to read 10.

To alter the sector numbers used, location &A89F must be patched (this

contains the first sectors number) to read &71. As 128 directory entries

are required location  &A897 must read 127. The final alteration is to

location &A8A8 (to read &FF), which tells the operating system that the

XPB is being patched, preventing it from automatically selecting a

standard format.

This assembly listing demonstrates how a single track (in this case track 0)

can be formatted to the new specification:

org &9000

call find_routines

call select_the_format

call patch_XPB

call format_the_track

ret

format_the_track

ld hl,header_information

ld de,0

rst 3,format_track1

ret

patch_XPB

ld hl,(&BE42)

ld (hl),40

ld de,7

add hl,de

ld (hl),127

ld de,8

add hl,de

ld (hl),&71

inc hl

ld (hl),10

add hl,de

ld (hl),&FF

ret

select_the_format

ld a,&C1

rst 3,select_format1

ret

find_routines

ld hl,select_format

call &bcd4

ld (select_format1),hl

ld hl,format_track

call &bcd4

ld (format_track1),hl

ret

select_format

defb &83

select_format1

defb 0,0,7

format_track

defb &86

format_track1

defb 0,0,7

header_information

defb 0,0,2,&71

defb 0,0,2,&76

defb 0,0,2,&72

defb 0,0,2,&77

defb 0,0,2,&73

defb 0,0,2,&78

defb 0,0,2,&74

defb 0,0,2,&79

defb 0,0,2,&75

defb 0,0,2,&7A

When you come to use this routine a loop will have to be included enabling

all tracks to be formatted. Don't forget that when you wish to read or

write to this new format the XPB will have to be patched.

It is possible to format tracks containing a single sector of 4096 bytes

(4K) or tracks with 16 sectors each holding 256 bytes (0.25k).

Byte 20 of the XPB holds a number calculated from the formula log2(sector

size)-7.

Assign byte 20 of the XPB a value 5 and then rearrange the expression

log2(sector size)-7 = 5

log2(sector size) = 12

sector size = (2)^12 = 4096

A sector size of 4096 happens to be the maximum size of a track; therefor

altering byte 20 to 5 will give a format containing one sector per track

(each sector being 4K in size). Here is the listing:

org &8000

call routine_find

call clear_XPB

call patch_XPB

call format_the_track

ret

routine_find

ld hl,select_format

call &bcd4

ld (select_format1),hl

ld hl,format_track

call &bcd4

ld (format_track1),hl

ret

clear_XPB

ld a,&41

ld e,0

rst 3,select_format1

ret

patch_XPB

ld hl,(&BE42)

ld de,16

add hl,de

ld a,1         ;number of sectors

ld (hl),a      ;per track

ld de,4

add hl,de

ld a,5         ;=log(sector size)-7

ld (hl),de

ld a,&ff       ;do not auto-select

ld (hl),a

ret

format_the_track

ld hl,header_information

ld e,0

ld d,0

ld c,&41

rst 3,format_track1

ret

select_format

defb  &83

select_format1

defw  0

defb  7

format_track

defb &86

format_track1

defw  0

defb  7

header_information

defb 0,0,&41,5

END

This is the Amstrad DPB Table

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

_____________________________________________________________________

|Byte |   Contents                                     |  Value     |

|~~~~~|~~~~~~~~|~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~|~~~~~~|~~~~~|

| 0,1 | SPT    | Records per track                     | 36   |&24  |

| 2   | BSH    | Block Shift factor                    |  3   |&03  |

| 3   | BLM    | Block Mask                            |  7   |&07  |

| 4   | EXM    | Extent Mask                           |  0   |&00  |

| 5,6 | DSM    | Total Storage Capacity                |170   |&AA  |

| 7,8 | DRM    | Maximum directory entries             | 63   |&3F  |

| 9   | AL0    | Directory Blocks                      |192   |&C0  |

| 10  | AL1    | Directory Blocks                      |  0   |&00  |

|11,12| CKS    | Checked Records                       | 16   |&10  |

|13,14| OFF    | Reserved Tracks                       |  2   |&02  |

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

This is Amstrad XPB (Expanded Disc Parameter Block)

____________________________________________________________________

| Byte |      Contents                                 | Value      |

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

| 15   | First (physical) sector number                | 65   | &41 |

| 16   | (Physical) sectors per track                  |  9   | &09 |

| 17   | Gap length (R/W)                              | 42   | &2A |

| 18   | Gap length (Format)                           | 82   | &52 |

| 19   | Filler byte                                   | 229  | &E5 |

| 20   | log2 (sector size)-7                          |  2   | &02 |

| 21   | Records per (physical) sector                 |  4   | &04 |

| 22   | Current Track (BIOS variable)                 |  0   | &00 |

| 23   | Not aligned / Aligned (BIOS variable)         |  0   | &00 |

| 24   | Auto format select (BIOS variable)            |  0   | &00 |

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

#################