Documentation for AMSTRAD PCW Forth-83

--------------------------------------

prepared by Geoff Barnard - Sept 1986

based on previous Public Domain document files.

Any queries - contact Geoff Barnard, 39A Mill Street, Guisborough, Cleveland,

TS14 6AG   (0287 37189)

The files on this disk (these disks ?) comprise a very powerful version of the

FORTH language, to the latest Forth-83 standard.   This implementation assumes

that you are already familiar with FORTH, as there is no explicit documentation.

The system does, however, include all the source code screens, with full

annotations, along with the necessary code to re-compile the .COM file from

source to incorporate any changes you might wish to make.

Please note that you cannot do this with an AMSTRAD PCW-8256 unless you have

expanded your RAM disk, as the normal disk capacity is too small.   If you have

the fattened RAM, and/or a higher capacity drive (360k is enough), then you're

OK.

The version of F83.COM on this disk has been 'installed' for the PCW, and is

ready to run.   The following additions have been made to the version the

writer originally obtained from the PD library:

A definition "M:" has been added to select the RAM-Disk

The boot process has been altered to automatically select the necessary screen

words for the AMSTRAD.

An extra piece of code has been added to the SAVE-BUFFERS word to force a

flush of the CP/M+ buffers.

The printing definitions have the necessary codes in for Condensed print.

An editor definition has been altered to use more PCW screen, and the editor

has been 'fixed' to allow later easy patching in of a full-screen editor.

Getting Started.

----------------

Initially, you need only the file F83.COM.  This program can be run as-is.  The

following notes refer to and update the notes in the file README.80 which you

are advised to printout bafore going any further.

Disregard the first page of the document, which relates to re-compiling the

system from source.   I'll come back to that later.  Page 2 onwards are

applicable as is, except NB please note that to use the VIEW definition you will

need to have 'on line' the source files KERNEL80.BLK, EXTEND80.BLK, CPU8080.BLK

and UTILITY.BLK.   These 4 files are suppplied Squeezed, and must be unsqueezed.

Unsqueezing files - use the NSWP utility supplied (this does lots of other neat

things as well, see NEWSWEEP.DOC for info).

A>nswp

(assuming nswp was on your current drive, the program will load, will access

the current disk, read its directory, print the header on the screen and then

list the first filename from the disk.   You may now step through the directory

using either the space-bar or the return key.   When you come to the end, it

starts from the top again!  To log onto another drive, press the L key (either

case), then when asked the A, B or M key to indicate the drive.

When you come to the file(s) you want to unsqueeze, type T to 'tag' them.  You

may tag a number of squeezed files (having a file-name ????????.?Q?), but note

the following:

KERNEL80.BQK (62K) expands to KERNEL80.BLK using 188k !!

EXTEND80.BQK (10K) expands to EXTEND80.BLK using 30K

CPU8080.BQK  (10K) expands to 42K

UTILITY.BQK  (36K) expands to 110K

Please note that if you use the VIEW word, and the necessary file isn't

available, the system just gives an error message and continues, so no big

problem.   The error message tells you while file was missing, so if you want

to VIEW particular words, try them first, see which file you need, then make

sure they're available of your current drive.

Anyway, having tagged the files to unsqueeze, type Q when NSWP next prompts

you.  At the next prompt, type a U for Unsqueeze.  At the next prompt, give the

destination drive name, having made sure the space is available.  Then sit back

and wait, as the process can take some time.   The program generates its own

destination file-names, removing the Q.   When the files are unsqueezed, you

might save them in their useable form for future use.)

Follow through the text in README.80.  Note that the advice regarding getting

the Editor working is not necessary, as this version has been set up to work

from scratch on the PCW.   The Editor is a fairly crude standard FORTH editor,

although with some powerful commands.   It is not a full-screen editor, although

one of these is in preparation, and may be available later.  If you want a

printout of the editor commands (and you'll find this very useful !), enter the

following, assuming the file UTILITY.BLK is on drive B:, and the printer is

loaded with paper and is ready.  Note that the word OK is supplied by FORTH

after you press RETURN and the program is waiting for you to enter something

else.

OK

b: OK

shadow OK

12 27 show (FORTH will now print-out 6 pages of source-code and comments).

Note that in the last line above, there must be a space between the 12 and the

27, and between the 27 and show.   In FORTH spaces are VITAL!!

For general use of the Editor, see Leo Brodie's book STARTING FORTH.

It is suggested that you create a file say called TEST.BLK to play with, before

you start on the supplied source-code screens.

Re-compiling the system.

-----------------------

The file README.80 says a lot about this, as originally the system was supplied

in an unconfigured form.   You don't need to do this, and you can use FORTH

quite happily and never need to.

However, you may in time decide to change/improve/re-write the system, or want

to add your own little touches to the basic system.   In which case, you can

re-compile.   See the instructions on page 1 of README.80, except that after

entering the initial CP/M command line you have to enter "1 LOAD" before

anything will happen.

For any further information, please contact the writer, Geoff Barnard, directly

be letter or phone.   There's a full-screen editor in the works, and some other

bits may soon be available.

You might also be interested in the FORTH Interest Group UK.   Members receive

a bi-monthly magazine with very useful articles, the back-numbers are an

excellent source of FORTH code.   A recent issue published GSX routines for the

AMSTRAD 6128, which may be altered to work under F83 on the PCW.

FIG-UK, c/o Colin Walls, 54 Wild Briar, Wokingham, Berks, RG11 4UL, membership

presently #10.00 per year (incl of magazine).

A little bonus - also on this disk is a file called SORTED.DIC.  This contains

a list of most of the words in this version of FORTH, sorted in ASCII order.

If you load BASIC, and run the program PRINTOUT.BAS, you can get a printout

of this list.   Quite useful for knowing just what's there !!

JGB/4th Oct 1986

###################################################