PCW File.

DIRCHK.COM                      A DIRECTORY CHECKER

--------------------------------------------------------------------

This program is used to verify a directory. The directory is  checked

to be sure that a group is used by only one  file.  This  can  happen

when a disk reset is not performed after a new diskette is  inserted.

DIRCHK is also useful to see if groups of an accidently deleted  file

have already been allocated to another file.

The directory and group numbers  (in  decimal)  are  displayed.  Also

groups used by multiple files are marked.

The BLOCK number indicated by the DIRCHK program can  be  related  to

the Track and Sector numbers of the disc using the table below.  Each

block occupies two sectors. Information  for  both  DATA  and  SYSTEM

formats is given.

Track                     .---------------- Sector ---------------.

|    |    |    |    |    |    |    |    |

.------ DATA Format ---- 193  194  195  196  197  198  199  200  201

|   .-- SYSTEM Format --  65   66   67   68   69   70   71   72   73

|   |

00  02  ----------------   0    0    1    1    2    2    3    3    4

01  03  ----------------   4    5    5    6    6    7    7    8    8

02  04  ----------------   9    9   10   10   11   11   12   12   13

03  05  ----------------  13   14   14   15   15   16   16   17   17

04  06  ----------------  18   18   19   19   20   20   21   21   22

05  07  ----------------  22   23   23   24   24   25   25   26   26

06  08  ----------------  27   27   28   28   29   29   30   30   31

07  09  ----------------  31   32   32   33   33   34   34   35   35

08  10  ----------------  36   36   37   37   38   38   39   39   40

09  11  ----------------  40   41   41   42   42   43   43   44   44

10  12  ----------------  45   45   46   46   47   47   48   48   49

11  13  ----------------  49   50   50   51   51   52   52   53   53

12  14  ----------------  54   54   55   55   56   56   57   57   58

13  15  ----------------  58   59   59   60   60   61   61   62   62

14  16  ----------------  63   63   64   64   65   65   66   66   67

15  17  ----------------  67   68   68   69   69   70   70   71   71

16  18  ----------------  72   72   73   73   74   74   75   75   76

17  19  ----------------  76   77   77   78   78   79   79   80   80

18  20  ----------------  81   81   82   82   83   83   84   84   85

19  21  ----------------  85   86   86   87   87   88   88   89   89

20  22  ----------------  90   90   91   91   92   92   93   93   94

21  23  ----------------  94   95   95   96   96   97   97   98   98

22  24  ----------------  99   99  100  100  101  101  102  102  103

23  25  ---------------- 103  104  104  105  105  106  106  107  107

24  26  ---------------- 108  108  109  109  110  110  111  111  112

25  27  ---------------- 112  113  113  114  114  115  115  116  116

26  28  ---------------- 117  117  118  118  119  119  120  120  121

27  29  ---------------- 121  122  122  123  123  124  124  125  125

28  30  ---------------- 126  126  127  127  128  128  129  129  130

29  31  ---------------- 130  131  131  132  132  133  133  134  134

30  32  ---------------- 135  135  136  136  137  137  138  138  139

31  33  ---------------- 139  140  140  141  141  142  142  143  143

32  34  ---------------- 144  144  145  145  146  146  147  147  148

33  35  ---------------- 148  149  149  150  150  151  151  152  152

34  36  ---------------- 153  153  154  154  155  155  156  156  157

35  37  ---------------- 157  158  158  159  159  160  160  161  161

36  38  ---------------- 162  162  163  163  164  164  165  165  166

37  39  ---------------- 166  167  167  168  168  169  169  170  170

38  40  ---------------- 171  171  172  172  173  173  174  174  175

39  41  ---------------- 175  176  176  177  177  178  178  179  179

Track                    <------------------ Block ---------------->

_

#  177  177  178  178  179  179

Track