MICROPLAN                                        MML Ltd.

=========                                        11 Sun Street

London E.C.2.

Tel: 01-247 0691

COPYRIGHT 1984

-----------------------------------------------------------------

DISZ80 - Zylog Z80 mnemonics

============================

Program DISZ80 disassembles a .COM file into a .M80 file which

may be subsequently assembled using the Microsoft assembler.

DISZ80  will  disassemble the full set of 696 Z80 op  codes  into

Zylog mnemonics.

In addition, a .MAP file is produced with a table of branch

points which is used on any subsequent disassembly.

The resulting .M80 file has the following features:

1.	All branch points are provided with a label

2.	Where permitted, data blocks are split into DB, DW & DS

3.	All references before the program load address and after

the program end, are EQUATE statements

4.	Any unresolved block commences with a U type label.

.COM

|

|

----------

| DISZ80 |<------> .MAP

----------

|

|

.M80

INSTRUCTIONS

------------

The program is run by entering

A>DISZ80 filename

The program forces type .COM for the source filename and writes

dissasembled Z80 mnemonics to file type .M80, and a table of

labels to file type .MAP. Any existing file type .M80 is renamed

into file type .BAK.

The program reads the table of labels from file type .MAP if this

file exists, then the program requests any additional program

entry points. DISZ80 assumes that the first entry is at 0100H

unless the Load command is used.

.cp10

The program displays each PASS as it is processed

PASS 1 -  Starting at the LOAD address and using any

additional instructions, assembly a table

of labels and their type.

PASS 2 -  Disassembly the program code and write the

file type .M80

PASS 3 -  Write the file type .MAP

.cp10

GUIDENCE ON FIRST DISASSEMBLY OF A FILE

---------------------------------------

In the first PASS enter 0000 against the first request for

additionaly jump addresses, and DISZ80 will disassemble the file.

In most files DISZ80 is unable to resolve all of the .COM code

and in particular it cannot resolve the JP (HL) instruction and

it is most likely that a large number of ERRORS will be reported.

All unresolved code will be given a label starting with a U, and

the code will be listed as a block of data.  Armed with a list of

mnemonics you should be able to establish if this unresolved data

is actually program code.

Examine the listing and see if you can decode the unresolved

areas. In particular, search for any table of program entry

points, and for the use of data following a call statement.

The S command can be used when the data following a call statement

is terminated with a zero byte.

Then destroy the .MAP file and run DISZ80 again.

SECOND & SUBSEQUENT DISASSEMBLY

-------------------------------

On the second and subsequent passes, you may assist the disassembly

by entering the additional program entry points when requested.

You may also notify DISZ80 of other types of code by preceeding

the address with one of the folloing:

1.      L       Program load address (default is 0100H)

2.	P	Progam entry point (default)

3.	N	Data numeric entry point

4.	W	Data word address

5.	T	Data table of program jump address

6.	S	String of data terminated with a zero follows any

CALL statement to the specified entry point

'T' is normally associated with a JP (HL) statement where the

value in HL has been extracted from a lookup table containing a

table of contigeuous data words of program entry points.

.cp4

Example

ADD ENTRY POINT (0000 TO QUIT): 02DA

ADD ENTRY POINT (0000 TO QUIT): T2F8C

ADD ENTRY POINT (0000 TO QUIT): 0000

.cp10

ERRORS

------

A) FATAL ERRORS

A fatal error occurs with an ERROR message on the terminal and

the program terminates.  This is normally caused by an

eroneous instruction in the ADD ENTRY POINT address.

The solution is to recheck your entry points and re-run

B) SYNTAX ERRORS

Other errors occur when there is a conflict between address

references.  The .M80 file is marked with an ERROR message.

Minor conflicts result in a WARN message.

.cp10

ASSEMBLY SOURCE CODE

--------------------

Finally when you have resolved all blocks of the code then:

Using your favourite editor, check for the following:

1.	Any ERROR or WARN message produced by DISZ80

2.	Check if  LD rr,n  instruction is referencing

a) address (default), or

b) constant

In the latter case change the address to a constant

The .M80 file is now suitable for modification

For use in MPM, the DISZ80 produces the BASE bias for relocation

at a different address, and so long as all the constants in 2b

above have been resolved, the .M80 file can be used to produce

a reliable page relocatable file with the linker [op] option.

.cp10

USING THE MICROSOFT ASSEMBLER AND DIGITAL RESEARCH LINKER

---------------------------------------------------------

The .M80 file can now be assembled using the Microsoft assembler,

but it is advisable to first change the filename to avoid

overwriting the original code. Example of assembly using the

Microsoft assembler, and linking using the Digital Reseach LINK:

A>m80 =program.m80

A>link program

=================================================================

#####################